U.S. Nitric Acid Production

Project Data Report

The Project Data Report (PDR) Template must be completed for each verification period. This template is only intended as a guide and provides the minimum required information to be reported. This template is designed for use with the Nitric Acid Production Protocol V2.2 and V3.0. The project developer has the option to include additional information at their discretion.

Please note that this document will be made publicly available once the project has registered credits for the verification period. If there is proprietary information, please provide a redacted version for publication and a non-redacted version for internal review. Please contact the Reserve to discuss what information may be allowed to be redacted.

| Account Holder | Ascend Performance Materials Operations LLC |
| --- | --- |
| Project ID and Name | CAR1624 Nitric Acid |
| Cooperative/Aggregate ID (if applicable) |  |
| Current Reporting Period Dates | 11/11/2024 - 07/20/2025 |
| Protocol Version | Nitric Acid Production V2.2 |
| Claimed CRTs by Vintage | Vintage 2024-19184; Vintage 2025-116273 |
| Date Submitted | 1/7/2026 |

# Introduction

Provide a general description of the project, including location, baseline scenario, project activity, etc.

The purpose of this project is to voluntarily reduce nitrous oxide (N2O) emissions from the Ascend Performance Materials Nitric Acid Production Plant (NAP) in Cantonment, Florida via a secondary abatement catalyst located within the Ammonia Oxidation Reactor, immediately downstream of the ammonia oxidation catalyst. Prior to secondary catalyst installation a baseline campaign was conducted to determine the N2O emissions per unit of nitric acid production using only a primary catalyst in the ammonia oxidation process.

# Project Definition

## Project Definition

Describe the abatement technology used, and confirm that the following scenarios do not apply to the project:

- NAPs that are restarted any time after December 2, 2007, after being out of operation for a period of 24 months or longer

- New NAPs constructed after December 2, 2009, with the exception of new NAPs for which a permit application for construction was submitted to the appropriate government

authorities prior to December 2, 2009

- Secondary catalyst projects at existing NAPs where NSCR is currently operating

- Secondary catalyst projects at existing NAPs that used NSCR technology for NOX abatement at any point since December 2, 2007

The abatement technology used is a secondary N2O abatement catalyst loacated immediately downstream of the primary catalyst. The scenarios listed above do not apply to Ascend's NAP project.

## Project Developer

List the entities which have any amount of legal control over the project boundary and identify the mechanism through which the Account Holder has legal authority to implement the project. In addition, please state which entity will be designated as the project developer and will sign the Attestation of Title. If the Account Holder is not the entity listed on the facility’s operating permit, explain how they obtain the right to operate the project on the facility’s behalf. which entity will be designated as the project developer and will sign the Attestation of Title. If there are no technical consultants or other parties with material interest, state “N/A.”

The project developer is Ascend Performance Materials and is only party involved in this NAP project.

Technical Consultant(s): N/A

Other Parties with Material Interest (entity name and role): N/A

Entity Signing Attestation of Title: Ascend Performance Materials Operations LLC

Additional Information: N/A

# Project Eligibility

## Project Location

Provide the project location.

Cantonment, FL USA

## Project Start Date

For secondary projects, the project start date is the date on which production first commences after the installation of a secondary catalyst. Confirm the start date is either the start of a campaign or the commencement of nitric acid production after an outage. For tertiary projects, the project start date is the date on which production first commences after installation of a tertiary catalyst. Projects that switch abatement technologies or installs an additional system for a combined secondary and tertiary catalyst project the start date should correspond with the original start date from the initial listing.

Please provide the start date for the project and describe the documentation used to demonstrate the start date as described above. Provide the duration of the project startup testing, if applicable.

The project start date is 5/5/2022.  The project start date was determined using the campaign completion information and the records for the commencement of production. The Management of Change document for the secondary catalyst addition project was used to demonstrate  the secondary catalyst was initially installed during outage occurring 4/14/2022 through 5/5/2022.

## Project Crediting Period

Due to the nature of campaigns, a crediting period for nitric acid production projects is approximately ten years and aligns with the end of the last full campaign that begins in the tenth year of reporting. If the project developer anticipates a crediting period greater than 11 years in total, provide the date in which the project received Reserve approval prior to the initiation of the final campaign.

Provide the crediting period below using an approximate date for the crediting period end date.

5/5/2022 to approximately 5/4/2032

Date of Reserve Approval if applicable:

## Additionality

### Performance Standard Test

The Performance Standard Test for Nitric Acid Production projects is based on an assessment of common practice of use of emission control technologies in NAPs to reduce N2O emissions. Check the box below for how the project meets the Performance Standard Test.

Secondary  catalyst

Tertiary catalyst, including catalytic decomposition or NSCR

Combined secondary and tertiary catalyst

### Legal Requirement Test

Attest that the project activities are not legally required. Identify any federal, provincial, or local regulations or legally binding mandates that are relevant to the project.

Ascend Performance Materials attests that CAR1624 project activities are not legally required in any way.  The formal signed attestation for the current reporting period can be found on CAR website project portal.

## Social and Environmental Safeguards

### Social Safeguards

Confirm below that the project is in material compliance with all labor and safety laws for the verification period.

Ascend Performance Materials confirms that CAR1624 is in material compliance with all labor and safety laws.  The Envionmental and Social Safeguards assessment for the current reporting period can be found on the CAR website project portal.

### Environmental Safeguards

Confirm below that the project is in material compliance with all environmental regulations in the verification period, and how the project is designed and implemented in such a way to mitigate potential releases of pollutants that may cause degradation of the quality of soil, air, and surface and groundwater.

Ascend Performance Materials confirms that CAR1624 is in material compliance with all environmental regulations.  The Envionmental and Social Safeguards assessment for the current reporting period can be found on the CAR website project portal.

## Regulatory Compliance

Provide the federal, state, and/or local regulatory agencies with jurisdiction over the project boundary. Disclose any instances of non-compliance during the verification period. Describe how the project intends to monitor for compliance during future reporting periods.

The Ascend Performance Materials NAP project boundary must regulatory comply with USEPA and FLDEP. The formal signed attestation of regulatory compliance for the current reporting period can be found on CAR website project portal.

There were 6 incidents identified for this reporting period; however, they were not caused by the Project and would have occurred in the absence of the Project:

• OSHA Inspection Nr 1813762.015: This incident occurred at Ascend’s Foley, AL site, it is unrelated to their Nitric Acid unit and carbon credit project.

• OSHA Inspection Nr 1800388.015: This incident occurred at Ascend’s Nylon polymer unit, it is unrelated to their Nitric Acid unit and carbon credit project.

• OSHA Inspection Nr 1843410.01: This incident occurred at Ascend’s Foley, AL site, it is unrelated to their Nitric Acid unit and carbon credit project.

• The facility failed to set the oxygen level of Emission Unit 014 Boiler 4 no lower than the lowest hourly average oxygen concentration measured during the most recent carbon monoxide performance test from May 23, 2024 to July 26, 2024. This fails to demonstrate compliance with Specific Condition A.21.b. of Permit 0330040-077-AV and 40 CFR 63.7525(a)(7): This incident was addressed during the previous reporting period, it was determined to not be Project related.

• The facility failed to submit the results of biennial emission factors testing no later than 45 days after completion of testing. This fails to demonstrate compliance with Specific Condition I.8. of Permit 330040-077-AV and Rule 62-4.070(3), F.A.C: This incident was addressed during the previous reporting period, it was determined to not be Project related.

• Clean Water Act (CWA) violation identified for the period 04/01-06/30/25 and 07/01-11/14/25: This incident corresponds to a malfunction in the water plant, and it is not related to the project.

The project uses Ascend Performance Materials compliance systems and procedures to monitor for compliance.

# GHG Assessment Boundary

## GHG Assessment Boundary

The table below includes only the SSRs that are included in the project quantification. For all SSRs considered in the GHG Assessment Boundary (included and excluded GHGs and their justification) refer to Tables 4.1 and 4.2 in the Protocol.

Secondary Catalyst Projects

| SSR | Source Description | Gas | Baseline (B) or Project (P) |
| --- | --- | --- | --- |
| 1 – Nitric Acid Production | Nitric acid process unit (burner inlet to stack) | N2O from reaction byproduct | B, P |

Tertiary Catalyst Projects

| SSR | Source Description | Gas | Baseline (B) or Project (P) |
| --- | --- | --- | --- |
| 1 – Nitric Acid Production | Nitric acid process unit (burner inlet to stack) | N2O from reaction byproduct | B, P |
| 4 - Ammonia Used to Operate SCR De-NOX Unit (included if ammonia is an input to the N2O destruction facility) | GHG emissions from production of ammonia used in tertiary abatement for N2O destruction | CO2, CH4, N2O | B (if applicable), P |
| 5 - Hydrocarbon Input | Hydrocarbon used as reducing agent and/or reheating the tail gas | CO2 and/or CH4 | P |
| 7 - External Energy to Reheat Tail Gas | May be used to reheat the tail gas before entering the tertiary catalyst or NSCR | CO2, CH4, N2O | P |

Combined Secondary and Tertiary Catalyst Projects

| SSR | Source Description | Gas | Baseline (B) or Project (P) |
| --- | --- | --- | --- |
| 1 – Nitric Acid Production | Nitric acid process unit (burner inlet to stack) | N2O from reaction byproduct | B, P |
| 4 - Ammonia Used to Operate SCR De-NOX Unit (included if ammonia is an input to the N2O destruction facility) | GHG emissions from production of ammonia used in tertiary abatement for N2O destruction | CO2, CH4, N2O | B (if applicable), P |
| 5 - Hydrocarbon Input | Hydrocarbon used as reducing agent and/or reheating the tail gas | CO2 and/or CH4 | P |
| 7 - External Energy to Reheat Tail Gas | May be used to reheat the tail gas before entering the tertiary catalyst or NSCR | CO2, CH4, N2O | P |

# Quantifying GHG Emission Reductions

## Quantifying Baseline Emissions

### Summary of Baseline Quantification

Provide the following information depending on the project scenario. Provide the following information depending on the project scenario. Leave the other section blank if not applicable.

Secondary Catalyst Projects

Allowable Operating Conditions:

Oxidation temperature range: 885.03 C - 900.47 C

Oxidation pressure range: 130.7 psia - 155.7 psia

Maximum ammonia-to-air ratio input into the AOR: 11.0

Select how these allowable ranges were determined:

Option (a): The best available historical data for the operating range of temperature and

pressure, and maximum ammonia-to-air ratio from the previous five campaigns.

Option (b): The best available historical data from less than five campaigns. This option is

allowable only if limited historical data are available (e.g., an upgraded or relocated NAP that has not been operating for at least five campaigns).

Option (c) Specified range of temperature and pressure found in the operating manual20 for

the existing equipment, and maximum ammonia-to-air ratio as specified by the ammonia oxidation catalyst manufacturer or the operating manual for the NAP equipment if guidance from the ammonia oxidation catalyst manufacturer is not available. This option is only allowable if no historical data are available.

| Vintage: 2024/2025 | Vintage: 2025 |
| --- | --- |
| HNO3,MAX, scaled: 110,221.0 tHNO3 | HNO3,MAX, scaled: 120,567.1 tHNO3 |
| Baseline Emission Factor: 0.0072504 tN2O/tHNO3 | Baseline Emission Factor: 0.0072504 tN2O/tHNO3 |
| Total N2O Emissions during the Baseline Sampling Period: 748.07 tN2O | Total N2O Emissions during the Baseline Sampling Period: 812.81 tN2O |
| Baseline Emissions: 198,238.11 tCO2e | Baseline Emissions: 215,394.30 tCO2e |

Tertiary Catalyst Projects

Allowable Operating Conditions:

Oxidation temperature range:

Oxidation pressure range:

Maximum ammonia-to-air ratio input into the AOR:

Select how these allowable ranges were determined:

Option (a): The best available historical data for the operating range of temperature and

pressure, and maximum ammonia-to-air ratio from the previous five campaigns.

Option (b): The best available historical data from less than five campaigns. This option is

allowable only if limited historical data are available (e.g., an upgraded or relocated NAP that has not been operating for at least five campaigns).

Option (c) Specified range of temperature and pressure found in the operating manual20 for

the existing equipment, and maximum ammonia-to-air ratio as specified by the ammonia oxidation catalyst manufacturer or the operating manual for the NAP equipment if guidance from the ammonia oxidation catalyst manufacturer is not available. This option is only allowable if no historical data are available.

| Vintage: | Vintage: |
| --- | --- |
| HNO3,MAX, scaled: tHNO3 | HNO3,MAX, scaled: tHNO3 |
| Baseline Emission Factor: tN2O/tHNO3 | Baseline Emission Factor: tN2O/tHNO3 |
| Total N2O Emissions during the Baseline Sampling Period: tN2O | Total N2O Emissions during the Baseline Sampling Period: tN2O |
| Baseline Emissions: tCO2e | Baseline Emissions: tCO2e |

Combined Secondary and Tertiary Catalyst Projects

Allowable Operating Conditions:

Oxidation temperature range:

Oxidation pressure range:

Maximum ammonia-to-air ratio input into the AOR:

Select how these allowable ranges were determined:

Option (a): The best available historical data for the operating range of temperature and

pressure, and maximum ammonia-to-air ratio from the previous five campaigns.

Option (b): The best available historical data from less than five campaigns. This option is

allowable only if limited historical data are available (e.g., an upgraded or relocated NAP that has not been operating for at least five campaigns).

Option (c) Specified range of temperature and pressure found in the operating manual20 for

the existing equipment, and maximum ammonia-to-air ratio as specified by the ammonia oxidation catalyst manufacturer or the operating manual for the NAP equipment if guidance from the ammonia oxidation catalyst manufacturer is not available. This option is only allowable if no historical data are available.

| Vintage: | Vintage: |
| --- | --- |
| HNO3,RP, scaled: tHNO3 | HNO3,RP, scaled: tHNO3 |
| EFBL: tN2O/tHNO3 | EFBL: tN2O/tHNO3 |
| HNO3,NEW: tHNO3 | HNO3,NEW: tHNO3 |
| EFNEW: tN2O/tHNO3 | EFNEW: tN2O/tHNO3 |
| Baseline Emissions: tCO2e | Baseline Emissions: tCO2e |

## Quantifying Project Emissions

### Summary of Project Emissions

Complete the table below depending on the project scenario. Leave the other table blank if not applicable.

Secondary Catalyst Projects:

| Vintage: 2024/2025 | Vintage: 2025 |
| --- | --- |
| Total N2O emissions in the campaign: 523.46 tN2O | Total N2O emissions in the campaign: 526.26 tN2O |
| Project Emissions: 138,716 tCO2e | Project Emissions: 139,459 tCO2e |

Tertiary Catalyst Projects:

| Vintage: | Vintage: |
| --- | --- |
| Emissions from the N2O in the tail gas during the reporting period: tCO2 | Emissions from the N2O in the tail gas during the reporting period: tCO2 |
| Emissions from the ammonia input used to operate the tertiary catalyst facility during the reporting period: tCO2 | Emissions from the ammonia input used to operate the tertiary catalyst facility during the reporting period: tCO2 |
| Emissions from the use of hydrocarbons as a reducing agent or to reheat tail gas during the reporting period: tCO2 | Emissions from the use of hydrocarbons as a reducing agent or to reheat tail gas during the reporting period: tCO2 |
| Emissions from the external energy used to reheat the tail gas during the reporting period: tCO2 | Emissions from the external energy used to reheat the tail gas during the reporting period: tCO2 |
| Project Emissions: tCO2e | Project Emissions: tCO2e |

Combined Secondary and Tertiary Catalyst Projects – Approach 1:

| Vintage: | Vintage: |
| --- | --- |
| Emissions from the N2O in the tail gas during the reporting period: tCO2 | Emissions from the N2O in the tail gas during the reporting period: tCO2 |
| Emissions from the ammonia input used to operate the tertiary catalyst facility during the reporting period: tCO2 | Emissions from the ammonia input used to operate the tertiary catalyst facility during the reporting period: tCO2 |
| Emissions from the use of hydrocarbons as a reducing agent or to reheat tail gas during the reporting period: tCO2 | Emissions from the use of hydrocarbons as a reducing agent or to reheat tail gas during the reporting period: tCO2 |
| Emissions from the external energy used to reheat the tail gas during the reporting period: tCO2 | Emissions from the external energy used to reheat the tail gas during the reporting period: tCO2 |
| Project Emissions: tCO2e | Project Emissions: tCO2e |

## Final CRT Summary

Summarize the final result for determining the net GHG Reductions by vintage.

| Vintage: 2024/2025 | Vintage: 2025 |
| --- | --- |
| Baseline Emissions: 198,238.11 tCO2e | Baseline Emissions: 215,394 tCO2e |
| Project Emissions: 138,716 tCO2e | Project Emissions: 139,459 tCO2e |
| Total GHG Emission Reductions: 59,522 tCO2e | Total GHG Emission Reductions: 75,935 tCO2e |

# Variance Request

If a variance was requested during the verification period, provide a brief description of the rule/requirement in the protocol in question, the conditions of the determination, and the date of the variance request and determination.

Date of Variance Request:

Protocol Section and Rule/Requirement:

Description of how the project did not meet the above requirements:

Date Reserve Issued Determination:

Variance Conditions:

# Project Documentation

The following documents are required to be submitted for project registration. In cases where there is proprietary information, please provide unredacted and redacted versions. If additional documents were submitted, please include the document title and its associated file name.

| Document | File Name |
| --- | --- |
| Project Diagram | Nitric Acid Plant CAR Diagram.ppt |
| Attestation of Title | Attestation-Title-RP6.pdf |
| Attestation of Voluntary Implementation | Attestation-Voluntary-Implementation-RP6.pdf |
| Attestation of Regulatory Compliance | Attestation-Regulatory-Compliance-RP6.pdf |
| Monitoring Plan | Nitric Acid Production Monitoring Plan V3.0.pdf |
| Project Contributions to the U.N. Sustainable Development Goals Report Tool | CAR1624_RP6_Social-Environmental-Safeguards-Assessment.docx |
| Project Data Report (this document) | CAR1624_RP6 U.S.-Nitric-Acid-Production-Project-Data-Report V2.docx |
| Variance Request (if applicable) |  |