U.S. Landfill Protocol

Project Data Report

The Project Data Report (PDR) Template must be completed for each verification period. This template is only intended as a guide and provides the minimum required information to be reported. This template is designed for use with the U.S. Landfill Protocol V6.0 (US LP). The project developer has the option to include additional information at their discretion.

Please note that this document will be made publicly available once the project has registered credits the verification cycle. If there is proprietary information, please provide a redacted version for publication and a non-redacted version for internal review. Please contact the Reserve to discuss what information may be allowed to be redacted.

| Account Holder | City of Superior |
| --- | --- |
| Project ID and Name | CAR406 Moccasin Mike Landfill Gas Project |
| Cooperative/Aggregate ID (if applicable) | N/A |
| Current Reporting Period Dates | 2024-01-01 - 2024-12-31 |
| Protocol Version | U.S. Landfill V6.0 |
| Claimed CRTs by Vintage | 43,952 (2024) |
| Date Submitted | 2025-06-03 |

# Introduction

Provide a general description of the project, including location, baseline scenario, project activity, etc.

The project voluntarily collects and destroys landfill gas using an open flare (project activity), preventing it from being emitted into the atmosphere (baseline scenario). These emission reductions are achieved in accordance with Version 6.0 of the Landfill Project Reporting Protocol, developed by the Climate Action Reserve.

City of Superior (Moccasin Mike) Landfill is located at 15 Moccasin Mike Road, Superior, Wisconsin, United States

# Project Definition

## Project Definition

Provide information on whether the project is an expansion of an existing landfill project or is being submitted as a new project, the type of destruction device (e.g. utility flares, enclosed flares, engines, turbines, microturbines, boilers, pipelines, leachate evaporators, kilns, sludge dryers, burners, furnaces, or fuel cells), default or site-specific destruction efficiency values, and whether landfill gas is destroyed onsite or transported for offsite use. Supporting documentation must be provided to the verifier during each verification cycle, which may include project diagrams, LFG flow data, methane flow data, facility records, etc.

This project consists of the voluntary collection and destruction of landfill gas and the necessary monitoring and data recording for the purpose of verifying Climate Reserve Tonnes (CRTs).

Methane destruction has occurred through an open flare. For this destruction, a methane destruction efficiency of 96% has been applied per Protocol guidelines.

## Project Developer

List the entities which have any amount of legal control over the project boundary and identify the mechanism through which the Account Holder has legal authority to implement the project. In addition, please state which entity will be designated as the project developer and will sign the Attestation of Title. If the Account Holder is not the entity listed on the facility’s operating permit, explain how they obtain the right to operate the project on the facility’s behalf.

Project Developer: City of Superior

Landfill Owner/Operator: City of Superior

Carbon Credit Manager:  Anew Environmental, LLC

# Project Eligibility

## Project Location

Provide the project location.

The Moccasin Mike Landfill is located in Wisconsin, therefore within the United States and is thus eligible as a project under the Protocol.

## Project Start Date

Briefly describe the documentation used to demonstrate the date at which the GCCS system became operational, i.e., the date landfill gas was first destroyed in a project destruction device. Provide a description of the start-up testing period (destruction device, timeline, etc.), if applicable. Include what documentation will be provided to the verification body to demonstrate the commencement of methane destruction.

The Moccasin Mike Landfill Gas Project (MMLGP) was initiated on April 28, 2009 when the landfill gas collection system and flare began operation. This commissioning date was determined by confirmation from staff at the landfill and Practical Controls who was on-site during startup. In addition, the LFGCCS installation report by Weaver Boos Consultants confirms the flare initiated during February 2005. The project started post January 1, 2001, thus meeting the Project Start Date eligibility criteria.

## Project Crediting Period

State the project’s crediting period as defined in Section 3.2 of the Protocol.

Crediting Period #1

2009-04-28 to 2019-04-27

Crediting Period #2

2019-04-28 to 2029-04-27

## Additionality

### Performance Standard Test

Check the box below and provide a brief description of how the landfill gas destruction project meets the performance standard test as outlined in Section 3.4.1 of the protocol.

Installation of a landfill gas collection and a new qualifying destruction device at an eligible landfill where landfill gas has never been collected and destroyed prior to the start date.

Installation of a new qualifying destruction device at an eligible landfill where landfill gas is currently collected and vented, but has never been destroyed in any manner prior to the start date.

Installation of a new qualifying destruction device at an eligible landfill where landfill gas was collected and destroyed at any time prior to the project start date using:

A non-qualifying destruction device (e.g., passive flare)

A destruction device that is not otherwise eligible under the protocol

Installation of a new gas collection system on a physically-distinct cell (or cells) where neither gas collection nor destruction has previously occurred.

Description: The Moccasin Mike Landfill was collecting and venting landfill gas (but not destroying the gas in any manner) prior to the installation of a new qualifying destruction device at the project start date, and is thus considered additional as per scenario 2 of section 3.4.1 of the Protocol.

### Limits on Credit Stacking

Disclose if the project participates in carbon or other types of mitigation project types outside of the Reserve. If so, include a brief description of the project activity, the Registry in which the project is listed, and how the two projects do not result in double counting. Confirm that the Reserve and the other registry are aware of and provided approval for credit stacking. If no project stacking / credit stacking is occurring, state, “Not applicable.”

Not applicable

### Legal Requirement Test

Attest that the project activities are not legally required. Identify any federal, provincial, or local regulations or legally binding mandates that are relevant to the project. State whether the project is subject to NSPS regulations and provide information regarding recent NMOC emission rate testing. Confirm that the landfill has not exceeded the NMOC threshold or provide the date at which the landfill is legally required to implement the GCCS.

To be considered additional, emissions reductions must be achieved by a project that would not have occurred due to federal, state or local regulations. The Moccasin 
Mike Landfill is not required by law to install a landfill gas collection and control system and meets all applicable federal, state, and local regulations or ordinances, thus passing the Legal Requirements Test. The City of Superior signed and submitted the Project’s Developers Attestation of Voluntary Implementation to CAR (See CAR Registry).

The New Source Performance Standard (NSPS) for MSW Landfills requires landfills that are new or expanded in capacity after July 17, 2014, and EG sites (existing landfills that have not been expanded or were not newly constructed after July 17, 2024) to control emissions of non-methane organic compounds (NMOC). If a landfill exceeds the 34 Mg per year threshold, then the landfill has 30 months to install a landfill gas capture system. Based upon the results of the Tier II Report conducted by Short Elliot Hendrickson Inc. (SEH) in August 2019, MMLGP’s NMOC emissions did not exceed 34 Mg/yr for this site. Furthermore, the TIER II report estimated NMOC emissions for the 2024 reporting year to be 12.6 Mg.

Furthermore, the Moccasin Mike Landfill is not required to manage its landfill gas under either the Wisconsin DNR or local regulations. In addition, the landfill has met all applicable state and local environmental regulations and has obtained all applicable permits.

## Social and Environmental Safeguards

### Social Safeguards

Confirm below that the project is in material compliance with all labor and safety laws for the verification period.

The Moccasin Mike Landfill maintains compliance with all labor and safety laws for the verification period. In order for the landfill to even operate, any labor and safety laws must be upheld.

### Environmental Safeguards

Confirm below that the project was in material compliance with all environmental regulations during the reporting period, and how the project is designed and implemented in such a way to mitigate potential releases of pollutants that may cause degradation of the quality of soil, air, and surface and groundwater. Provide a brief description of any environmental benefits or harms that have resulted from the project.

The Moccasin Mike Landfill maintains environmental compliance through the guidance detailed in their operating permits required under federal, state, or local environmental regulations. The permits include inspection frequencies, sampling frequencies, and reporting requirements. These permits are included in the documentation provided for verification. The project is designed to voluntarily collect and destroy landfill gas using an open flare - mitigating releases of pollutants that may cause degradation of the quality of soil, air, and surface and groundwater. Therefore, by preventing these pollutants from being emitted into the atmosphere, the Project has a positive impact on the environment as a result. The Project also contributes to SDG goal 13.0 as under the project condition, methane is converted to CO2 through combustion of captured landfill gas which results in measurable greenhouse gas emission reductions - resulting in positive climate action.

## Regulatory Compliance

Provide the national, provincial, and/or local regulatory agencies with jurisdiction over the project boundary. Disclose any instances of non-compliance during the reporting period. Describe how the project will monitor for compliance during future reporting periods.

The Moccasin Mike Landfill maintains regulatory compliance through the guidance detailed in their operating permits required under federal, state, or local environmental regulations. The permits include inspection frequencies, sampling frequencies, and reporting requirements. These permits are included in the documentation provided for verification. There were no instances of non-compliance during the 2024 reporting period.

# GHG Assessment Boundary

## GHG Assessment Boundary

The table below includes only the SSRs that are included in the project quantification. For all SSRs considered in the GHG Assessment Boundary (included and excluded GHGs and their justification) refer to Table 4.1 in the Protocol.

| SSR | Source Description | Gas | Baseline (B) or Project (P) | Confirm each SSR is Included |
| --- | --- | --- | --- | --- |
| 4 | Emissions from Waste Breakdown in Landfill | CH4 | B,P |  |
| 5 | Emissions from Gas Collection System | CO2 | P |  |
| 6 | Emissions from Supplemental Fuel | CO2, CH4 | P |  |
| 7 | Emissions from Project LFG Boiler Destruction | CH4 | P |  |
|  | Emissions from Baseline LFG Boiler Destruction | CH4 | B |  |
| 8 | Emissions from Project LFG Electricity Generation | CH4 | P |  |
|  | Emissions from Baseline LFG Electricity Generation | CH4 | B |  |
| 9 | Emissions from Project LFG Flare Destruction | CH4 | P |  |
|  | Emissions from Baseline LFG Flare Destruction | CH4 | B |  |
| 10 | Emissions from Upgrade of LFG | CO2 | B,P |  |
| 11 | Emissions from Project LFG Pipeline or other NG end-use | CH4 | P |  |
|  | Emissions from Baseline LFG Pipeline or other NG end-use | CH4 | B |  |

# Quantifying GHG Emission Reductions

## Quantifying Baseline Emissions

### Summary of Baseline Quantification

Provide the total methane destroyed by the GCCS, factor for the oxidation of methane by soil bacteria, discount factor to account for modeling uncertainties, and adjustment factor to account for baseline LFG destruction device in the verification period, as calculated in the Protocol.

| Vintage: 2024 | Vintage: |
| --- | --- |
| Total methane destroyed: 1,745.64 tCH4 | Total methane destroyed: tCH4 |
| Oxidation factor by soil bacteria: 0.1 | Oxidation factor by soil bacteria: |
| Monitoring uncertainties discount factor: 0 | Monitoring uncertainties discount factor: |
| Adjustment factor for baseline LFG destruction device: 0 | Adjustment factor for baseline LFG destruction device: |
| Baseline Emissions: 43,990.03 tCO2e | Baseline Emissions: tCO2e |

## Quantifying Project Emissions

### Site-Specific Destruction Efficiency

Provide the name of the state or local agency accredited service provider utilized to conduct the site-specific destruction efficiency testing, as well as the date the test was conducted and the destruction efficiency used for quantification.

The destruction efficiency used for quantification has been applied per Protocol guidelines. According to the Protocol, the default destruction efficiencies are based on a preliminary set of actual source test data provided by the Bay Area Air Quality Management District.

### Summary of Project Emissions

Provide the total carbon dioxide emissions from the destruction of fossil fuel, total carbon dioxide emissions from the consumption of electricity from the grid, and the total quantity of emissions from supplemental natural gas in the verification period, as calculated in the Protocol.

| Vintage: 2024 | Vintage: |
| --- | --- |
| Emissions from fossil fuel use: 0 tCO2 | Emissions from fossil fuel use: tCO2 |
| Emissions from electricity use: 37.83 tCO2 | Emissions from electricity use: tCO2 |
| Emissions from supplemental natural gas: 0 tCO2 | Emissions from supplemental natural gas: tCO2 |
| Project Emissions: 37.83 tCO2e | Project Emissions: tCO2e |

## Final CRT Summary

Summarize the final result for determining the net GHG Reductions by vintage.

| Vintage: 2024 | Vintage: |
| --- | --- |
| Baseline Emissions: 43,990.03 tCO2e | Baseline Emissions: tCO2e |
| Project Emissions: 37.83 tCO2e | Project Emissions: tCO2e |
| Total GHG Emission Reductions/Removals: 43,952.20 tCO2e | Total GHG Emission Reductions/Removals: tCO2e |

# Project Monitoring

Check the boxes below and attest that project monitoring has occurred according to the Protocol. If requirements were not met, please list the date of variance request and fill out Section 7.

Have all monitoring and QA/QC requirements been met?  Yes  No

Have all field checks and calibrations been performed by a third-party?  Yes  No

Date of Variance Request, if applicable: N/A

# Variance Request

If a variance was requested during the reporting period, provide a brief description of the rule/requirement in the protocol in question, the conditions of the determination, and the date of the variance request and determination.

Date of Variance Request:

Protocol Section and Rule/Requirement:

Description of how the project did not meet the protocol requirement above:

Date Reserve Issued Determination:

Variance Conditions:

# Project Documentation

The following documents are required to be submitted for project registration. In cases where there is proprietary information, please provide unredacted and redacted versions. If additional documents were submitted, please include the document title and its associated file name.

| Document | File Name |
| --- | --- |
| Project Diagram | "City of Superior project diagram Jul 2024 v6.pdf" |
| Attestation of Title | "Attestation-Title-VY2024.pdf" |
| Attestation of Voluntary Implementation | "Attestation-Voluntary-Implementation-VY2024.pdf" |
| Attestation of Regulatory Compliance | "Attestation-Regulatory-Compliance-VY2024.pdf" |
| Monitoring Plan | "Moccasin Mike - 2024 Monitoring Plan 20250703.pdf" |
| Project Contributions to the U.N. Sustainable Development Goals Report Tool (if applicable) | "Moccasin Mike - SDG-Reporting-Tool-v2.0-1 20250627.xlsx" |
| Project Data Report (this document) | "Moccasin Mike - U.S.-Landfill-PDR-1 20250805.docx" |
| List of Findings | "CAR406 List of Findings.pdf" |
| Variance Request (if applicable) | N/A |