Landfill Project Submittal Form

| Instructions: Please complete all fields as thoroughly as possible. If the project in question is still in the planning or development phase, all fields must be completed using best available data and estimates based on the proposed project design. This is an interactive Word form. Upon completion, please save this form as a PDF prior to uploading it to the Reserve. This will lock your answers and protect the document from any further changes. All fields must be completed, even if the answer is also provided elsewhere; if a field is not applicable insert N/A in the space provided. Upon approval, this form will become public. |  |
| --- | --- |
| - Account Holder (as it appears in the Reserve software): | Oneida Herkimer Solid Waste Authority |
| - Project Name (as it appears in the Reserve software): | Oneida Herkimer Landfill Ava, NY |
| - Project ID #: | CAR674 |
| - Protocol Version: | Landfill Project Protocol V5.0 |
| - Project Crediting Period* (select one): *If the project is being submitted for a second crediting period, please send an email to reserve@climateactionreserve.org notifying the Reserve Administrator. In addition, please upload to the existing project in the Reserve software (there is no need to create a new project for the purposes of applying for a second crediting period). | First crediting period Second crediting period |
| - Technical Consultant(s): | Barton and Logudice, Authority Engineer |
| - Other Parties with a Material Interest: | NA |
| - Form Completed By (name, organization): | Denise Farrell, Environmental Attribute Advisors, Advisor to Authority |
| - Contact Information (phone, email): | 9176217165 |
| - Date of Form Completion: | 9/15/2019 |
| - Project Start Date (MM/DD/YYYY): | 07/15/2010 |
| - First Reporting Period of the Current Crediting Period (MM/DD/YYYY): | 7/15/2020 to 7/14/2021 First reporting period of second crediting period |

| Project Site Information |  |  |  |
| --- | --- | --- | --- |
| - Landfill name | Oneida Herkimer Landfill Ava NY |  |  |
| - Project site address (including county and country) | 7044 State Rte 294, Boonville, NY 13309 |  |  |
| - Project description (please provide one to two paragraphs) | The Oneida-Herkimer Solid Waste Authority (OHSWA) owns and operates the NYCRR Part 360 permitted Regional Landfill located on in the Town of Ava, Oneida County, New York. The facility voluntarily operates a landfill gas (LFG) collection and control system (GCCS) that collects LFG from vertical extraction wells and horizontal collection pipes. The GCCS is continuously expanded as waste placement progresses. Collected LFG is conveyed to a flare station and landfill gas to energy (LFGTE) facility for destruction. The landfill gas flare station includes two (2) 1,250-cubic feet per minute (CFM) design capacity open candlestick utility flares manufactured by John Zink Co., LLC. An enclosure surrounds the LFG blower skid to protect the skid from the elements. The flare station began operation in 2010. The WMRE landfill gas to energy (LFGTE) facility is located on a leased parcel of land adjacent to the flare skid. The WMRE LFGTE facility is separately owned, operated and permitted from the OHSWA Regional Landfill facility and landfill gas flare station. OHSWA and WMRE have a site lease and landfill gas purchase agreement in place that specifies that OHSWA retains ownership of emission reduction credits associated with the voluntary destruction of methane in landfill gas. The LFGTE facility is permitted for up to three (3) Caterpillar® G3520C lean burn, landfill gas fueled reciprocating internal combustion (IC) engine generator sets. Two engine generator sets are currently installed and operational. Flaring of landfill gas is used as an alternate control method during periods when the engines are down for periodic maintenance, as well as parallel control when excess landfill gas is collected beyond the capacity of the installed engines. |  |  |
| - Owner of landfill (name and organization) | Oneida Herkimer Solid Waste Authority |  |  |
| - Owner of gas collection and control system | Oneida Herkimer Solid Waste Authority |  |  |
| - Type of waste accepted (MSW, demolition debris, organic, etc.) | MSW |  |  |
| - Designed landfill capacity (cubic meters or tonnes) | 21.8x10^6 cubic meters |  |  |
| - Date of last modification, construction, or reconstruction (if applicable) | N/A. |  |  |
| - Year landfill opened | 2006 |  |  |
| - Year landfill closed or estimated date of closure (if known) | 2085 (estimated) |  |  |
| Baseline Destruction Information |  | Yes | No |
| - Did the landfill ever collect and destroy landfill gas using a non-qualifying destruction device (i.e., passive flares)? If yes, please describe: Prior to the startup of the active gas collection system, the facility voluntarily operated passive LFG flares in order to reduce greenhouse gas emissions and improve air quality. The flares are Solar Spark® model CF-5 flares manufactured by Landfill Service Corporation. The passive flares were located at two (2) leachate cleanouts C-1, C-2 and horizontal collector locations HC-1, HC-2 and HC-3 which were installed to control LFG until the active gas collection and control system completed construction. The flares were monitored in accordance with a pre-project monitoring plan. |  |  |  |
| - Did the landfill ever collect and destroy landfill gas using a qualifying destruction device (i.e., candlestick flare, engine, etc.)? If yes, please describe: |  |  |  |
| Project Eligibility and Monitoring |  | Yes | No |
| - Have any GHG reductions from the project ever been registered with or claimed by another registry or program prior to registering with the Reserve? If yes, you must complete and return a “Registry Project Transfer Attestation Form” |  |  |  |
| - Have any GHG reductions from the project ever been sold directly to a third party (i.e., sold without being registered with or claimed by another registry or program) prior to registering with the Reserve? If yes, please describe: |  |  |  |
| - Is the landfill subject to NSPS Regulation (greater than 2.5 million tonnes capacity)? |  |  |  |
| - If applicable, date of the most recent Tier I or Tier II NMOC test (format MM/DD/YYYY): 04/03/2019 |  | - | - |
| - NMOC emissions per year (Mg/year): 12.84 (2019) |  | - | - |
| - Date at which landfill did or is expected to exceed 34 Mg NMOC per year (if the landfill was modified, constructed, or reconstructed after July 17, 2014) or 50 Mg NMOC per year otherwise (format MM/DD/YYYY): Not expected to reach 34 Mg/yr over the next 10 years |  | - | - |

| - Is this project being implemented and maintained as the result of any law, statute, regulation, court order, or other preexisting legally binding mandate? If yes, please explain: |  |  |
| --- | --- | --- |
| - Has a detailed monitoring plan been developed for this project? If no, date a monitoring plan will be in place (format MM/DD/YYYY): |  |  |
| - Is the Account Holder authorized to sign the Attestation of Title? |  |  |
| All Projects (Optional) |  |  |
| - Additional information (if any): |  |  |