Monitoring Report

PUBLICATION DATE  14.10.2020
VERSION  v. 1.1 
RELATED SUPPORT – TEMPLATE GUIDE Monitoring Report v. 1.1

This document contains the following Sections

Key Project Information

SECTION A - Description of project

SECTION B - Implementation of project

SECTION C - Description of monitoring system applied by the project

SECTION D - Data and parameters

SECTION E - Calculation of SDG Impacts

SECTION F - Safeguards Reporting

SECTION G - Stakeholder inputs and legal disputes

KEY PROJECT INFORMATION

Key Project Information

| GS ID (s) of Project (s) | GS11674 |
| --- | --- |
| Title of the project (s) covered by monitoring report | Dongrui Animal Manure Management System GHG Mitigation Project in Heyuan City |
| Version number of the PDD/VPA-DD (s) applicable to this monitoring report | 05 |
| Version number of the monitoring report | 04 |
| Completion date of the monitoring report | 10/03/2023 |
| Date of project design certification | 08/02/2023 |
| Date of Last Annual Report | N/A |
| Monitoring period number | 01 |
| Duration of this monitoring period | The 1st monitoring period from 01/11/2021 to 31/10/2022 (first and last days included) |
| Project Representative | The official focal point and project developer: Profit Carbon Environmental Energy Technology (Shanghai) Co., Ltd. The project owner: Dongrui Food Group Co., Ltd. |
| Host Country | People’s Republic of China |
| Activity Requirements applied | Community Services Activities Renewable Energy Activities Land Use and Forestry Activities/Risks & Capacities N/A |
| Methodology (ies) applied and version number | ACM0010 GHG emission reductions from manure management systems (Version 08.0) |
| Product Requirements applied | GHG Emissions Reduction & Sequestration Renewable Energy Label N/A |

Table 1 - Sustainable Development Contributions Achieved

| Sustainable Development Goals Targeted | SDG Impact | Amount Achieved | Units/ Products |
| --- | --- | --- | --- |
| SDG 13: Climate Action | Amount of GHGs emission avoided or sequestered | 354,929 | tCO2e |
| SDG 7: Affordable and Clean Energy | Total electricity produced | 46,360.24 | MWh |
| SDG 8: Decent Work and Economic Growth | Total number of jobs | 16 full-time jobs created including 8 females and 8 males | Number |

Table 2 – Product Vintages

| SDG 7 Affordable and Clean Energy |  | Amount Achieved |
| --- | --- | --- |
| Start Dates | End Dates | Total electricity produced (MWh) |
| 01/11/2021 | 31/12/2021 | 7,747.49 |
| 01/01/2022 | 31/10/2022 | 38,612.75 |
| 01/11/2021 | 31/10/2022 | 46,360.24 |

| SDG 8 Decent Work and Economic Growth |  | Amount Achieved |
| --- | --- | --- |
| Start Dates | End Dates | Total number of jobs |
| 01/11/2021 | 31/12/2021 | 16 full-time jobs created including 8 males and 8 females |
| 01/01/2022 | 31/10/2022 | 16 full-time jobs created including 8 males and 8 females |
| 01/11/2021 | 31/10/2022 | 16 full-time jobs created including 8 males and 8 females |

| SDG 13 Climate Action |  | Amount Achieved |
| --- | --- | --- |
| Start Dates | End Dates | Amount of GHGs emission avoided or sequestered (tCO2e) |
| 01/11/2021 | 31/12/2021 | 59,406 |
| 01/01/2022 | 31/10/2022 | 295,523 |
| 01/11/2021 | 31/10/2022 | 354,929 |

- DESCRIPTION OF PROJECT

General description of project

>>

Dongrui Animal Manure Management System GHG Mitigation Project in Heyuan City installs new animal waste management systems (AWMSs) to a group of 8 swine farms in Heyuan City, which are owned by Dongrui Food Group Co., Ltd. (hereinafter called” Dongrui”). The purpose of the project activity is to treat the manure and wastewater to avoid methane emissions generated in the baseline uncovered anaerobic lagoons.

The project activity replaces the current open anaerobic lagoons with new AWMSs, and Each subsidiary swine farm will install one AWMS. All the manure and wastewater are collected and then be separated first. The separated solid are treated in aerobic composting system to produce fertilizer and part of the fertilizer will be distributed to the surrounding farmers free of charge to improve the life condition of farmers and the others will be sold to the market. The separated liquid is treated through anaerobic digestion, the biogas generated during the treatment process is captured for power generation and surplus biogas is destroyed through the flaring system (if any). The electricity generated are all used by the operation of AWMSs and the 8 swine farms. It is estimated that total 46,435,884 m3 biogas are expected to be generated annually and each generator is installed in each subsidiary swine farm, total installed capacity of the proposed project is 8.1MW and the annual electricity production is estimated to be 59,435 MWh. After anaerobic digestion. The sludge produced from anaerobic digestion will be treated through aerobic composting together with the separated solid and the effluent will be used as a liquid organic fertilizer for the irrigation of agriculture.

Benefited from the “waste-biogas-fertilizer” pattern, harmlessness and ecological utilization of the swine manure can be realized. The power generated are all used by the AWMSs and the 8 swine farms and will not be connected to another user or to the regional power grid and the baseline emissions associated with electricity generation will not be claimed.

The scenario prior to the implementation of the proposed project activity is that the manure and wastewater from the 8 swine farms were treated in uncovered anaerobic lagoons.

The project activity reduces GHG in the atmosphere through avoiding methane emissions from anaerobic treatment of swine manure and wastewater. The annual average CO2 emission reductions are estimated to be 423,531 tCO2e and total emission reductions are estimated to 2,117,655 tCO2e in the first crediting period as per PDD.

The project has been put into operation in 01/11/2021. This monitoring report is for the 1st monitoring period, which is from 01/11/2021 to 31/10/2022. Total emission reductions achieved in this monitoring period are 354,929 tCO2e.

Location of project

>>

The proposed project activity is located in Heyuan City, Guangdong Province, China. There are eight subsidiary farms involved in the project. All of the eight swine farm including market swine and breeding swine: Yujing swine farm, Zhifu swine farm, Lianping Nongmu swine farm, Longchuan Nongmu swine farm, Zijin Nongmu swine farm, Zijin Nongye swine farm, Dongyuan Nongmu swine farm, Heping Nongmu swine farm. The specific location of the eight subsidiary farms is shown below:

Table 1. The GPS location of the eight farms

| Swine Farm | Location | East longitude | North latitude |
| --- | --- | --- | --- |
| Yujing swine farm （玉井猪场） | Yujing Village, Dengta Town, Dongyuan County | 114.754424° | 23.981758° |
| Zhifu swine farm （致富猪场） | Zhifu Forest Ranch, Zhifu Village, Luohu Town, Dongyuan County | 114.844647° | 24.134251° |
| Lianping Nongmu swine farm （连平农牧猪场） | Tangbei Village, Sanjiao Town, Lianping County | 114.819683° | 24.192278° |
| Longchuan Nongmu swine farm （龙川农牧猪场） | Shierpai Village, Fengren Town, Longchuan County | 115.330901° | 24.190473° |
| Zijin Nongmu swine farm （紫金农牧猪场） | Shibei Village, Lantang Town, Zijin County | 114.933014° | 23.451163° |
| Zijin Nongye swine farm （紫金农业猪场） | Huangjinkeng Group, Fuzhu Village, Jiuhe Town, Zijin County | 114.977270° | 23.509722° |
| Dongyuan Nongmu swine farm （东源农牧猪场） | Qunfeng Village, Chuantang Town, Dongyuan County | 114.852099° | 24.142245° |
| Heping Nongmu swine farm （和平农牧猪场） | Jiufengzhang, Shicun Village, Beidun Town, Heping County | 115.177540° | 24.532783° |

Guangdong Province

Figure 1 The Geographic Location of the project

Reference of applied methodology

>>

The approved methodology applied by this project is referenced as ACM0010” GHG emission reductions from manure management systems (Version 08.0)”.

Tool applied:

Tool 02: “Combined tool to identify the baseline scenario and demonstrate additionality(version07.0)”

Tool 08: “Tool to determine the mass flow of a greenhouse gas in a gaseous stream (version03.0)”

Tool 14: “Project and leakage emissions from anaerobic digesters (Version 02.0)”

Tool 06:” Project emissions from flaring” (Version 4.0)

Tool 05:” Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation” (Version 3.0)

Tool 24:” Common practice (Version 03.1)”

All above methodology and tool can be available from:

http://cdm.unfccc.int/methodologies/PAmethodologies/approved

https://cdm.unfccc.int/Reference/tools/index.html

Crediting period of project

>>

Start date of crediting period: 01/11/2021.

End date of crediting period:31/10/2026.

Total length of crediting period: 01/11/2021 to 31/10/2026 (first and last days included) for the first crediting period (two renewal cycles and in total 15 years). The 1st monitoring period is from 01/11/2021 to 31/10/2022 (first and last days included), which belong to the first crediting period.

Expected lifetime of the project: 15 years.

- IMPLEMENTATION OF PROJECT

Description of implemented project

>>

8 sets of AWMSs in 8 existing swine farms have been implemented. All the manure and wastewater are collected and then be separated first. The solid is treated in aerobic composting system, which is used as fertilizer. And part of the fertilizer products is distributed to the surrounding farmers free of charge to improve the life condition of farmers, the others is sold to the market. The liquid is treated through anaerobic digestion, the biogas generated during the treatment process is captured for power generation. if there is surplus biogas, then the biogas can be flared through the flaring system. The sludge produced from anaerobic digestion is treated through aerobic composting together with the solid and the effluent is treated aerobically and then used for agriculture irrigation.

Anaerobic tanks adopt Upflow Anaerobic Sludge Bed Reactor (UASB) as its anaerobic digester technology, and the biogas generated from the anaerobic digesters enter the biogas generator set after biological desulfurization and dehydration purification. The generated electricity is used for daily operation of AWMSs and the swine farms, and the waste heat of the generator is recycled for increasing the temperature of the anaerobic reactor. At the same time, and the residual biogas is flared if there is any surplus biogas.

The technical flow of the project activity is shown below:

Figure 2 Process flow of biogas project in the swine farms

The project was implemented in 8 existing swine farms and the technology implemented at each swine farm was same. While the treatment capacities of each AWMS and the size of each swine farm and corresponding design of the AWMS was different. The treatment capacity and equipment of the system are designed according to the size of the farm. The construction date, commissioning date and start date of operation of AWMS in each farm are summarized in Table 2. The other mainly milestone for implementation of the project are summarized in Table 3. The volume for original uncovered anaerobic lagoons and the equipment parameters used of each swine farm are shown in the following Table 4:

Table 2 The construction, commissioning and start operation date of AWMS in each swine farm involved in this project

| Swine farm name | Construction date | Commissioning date | Start Operation date |
| --- | --- | --- | --- |
| Yujing swine farm | 05/07/2021 | 13/10/2021 | 01/11/2021 |
| Zhifu swine farm | 07/07/2021 | 15/10/2021 | 01/11/2021 |
| Lianping Nongmu swine farm | 10/07/2021 | 15/10/2021 | 01/11/2021 |
| Longchuan Nongmu swine farm | 10/07/2021 | 16/10/2021 | 01/11/2021 |
| Zijin Nongmu swine farm | 06/07/2021 | 15/10/2021 | 01/11/2021 |
| Zijin Nongye swine farm | 07/07/2021 | 18/10/2021 | 01/11/2021 |
| Dongyuan Nongmu swine farm | 07/07/2021 | 12/10/2021 | 01/11/2021 |
| Heping Nongmu swine farm | 08/07/2021 | 13/10/2021 | 01/11/2021 |

Table 3 Milestone for implementation of the project

| Time | Main actions |
| --- | --- |
| 26/04/2021 | EIA |
| 15/05/2021 | Completion of Project Evaluation Report (Including prior consideration of carbon revenue). |
| 30/05/2021 | EIA Approval by local Ecology and Environment Department |
| 25/06/2021 | Online board meeting regarding to the project implementation with Gold Standard. |
| 01/07/2021 | Start date of the project, the date on signing biogas generator purchase contract |
| 25/10/2021 | Training for the GS monitoring plan and biogas safety operation |
| 01/11/2021 | The project was put into operation |
| 19/05/2022 | Submission for preliminary review |
| 08/06/2022 | Local stakeholder consulting physical meeting |
| 17/10/2022 | Training for the GS monitoring plan and biogas safety operation |

There are no deviations or delays compared to information in approved PDD.

During this monitoring period, there was no special event occurred, such as overhaul and downtimes of anaerobic digesters. All the equipment operated well and no equipment was under maintenance during this monitoring period.

Table 4 technical parameters of main equipment and uncovered anaerobic lagoons

| Farm name | Yujing swine farm | Zhifu swine farm | Lianping Nongmu swine farm | Longchuan Nongmu swine farm | Zijin Nongmu swine farm | Zijin Nongye swine farm | Dongyuan Nongmu swine farm | Heping Nongmu swine farm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Farm Size (head) | 15300 | 72000 | 57800 | 24000 | 27200 | 59400 | 163000 | 63200 |
| Treatment capacities of AMMS(t/d) | 70 | 320 | 250 | 100 | 120 | 260 | 700 | 280 |
| Baseline anaerobic lagoon parameters |  |  |  |  |  |  |  |  |
| Size of baseline anaerobic lagoon(length*width*depth) (m3) | 40*40*3 | 100*50*4 | 100*50*3 | 50*40*3 | 50*50*3 | 80*50*4 | 100*100*4.2 | 80*70*3 |
| technical life: | 15 years |  |  |  |  |  |  |  |
| Power Generator equipment parameters |  |  |  |  |  |  |  |  |
| Type | 250GF | 1200GF | 1000GF | 400GF | 450GF | 1000GF | 2700GF | 1100GF |
| Rated power (kw) | 250 | 1200 | 1000 | 400 | 450 | 1000 | 2700 | 1100 |
| Rated output voltage(V) | 400/230 | 400/230 | 400/230 | 400/230 | 400/230 | 400/230 | 400/230 | 400/230 |
| Frequency (Hz) | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| power factor | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 |
| Rated speed (rpm) | 1500 | 1500 | 1500 | 1500 | 1500 | 1500 | 1500 | 1500 |
| Equipment Quantity | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Equipment technical life | 30 years |  |  |  |  |  |  |  |
| Anaerobic tank parameters |  |  |  |  |  |  |  |  |
| Design capacity (m3) | 1200 | 5000 | 4000 | 1500 | 2000 | 4200 | 12000 | 4500 |
| Stay time(day) | 15.5-16 | 15.5-16 | 15.5-16 | 15.5-16 | 15.5-16 | 15.5-16 | 15.5-16 | 15.5-16 |
| Fermentation temperature(℃) | 35-38 | 35-38 | 35-38 | 35-38 | 35-38 | 35-38 | 35-38 | 35-38 |
| Equipment Quantity | 1 | 2 | 2 | 1 | 1 | 2 | 4 | 2 |
| Equipment technical life | 20 years |  |  |  |  |  |  |  |
| Turnover machine parameters |  |  |  |  |  |  |  |  |
| Type | 9GTF38B | 9GTF38B | 9GTF38B | 9GTF38B | 9GTF38B | 9GTF38B | 9GTF38B | 9GTF38B |
| Single machine power (kw) | 26.5 | 26.5 | 26.5 | 26.5 | 26.5 | 26.5 | 26.5 | 26.5 |
| Drum width (m) | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 |
| Track center distance span (m) | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 |
| Speed (rmp) | 180 | 180 | 180 | 180 | 180 | 180 | 180 | 180 |
| Equipment Quantity | 1 | 2 | 2 | 1 | 1 | 2 | 6 | 2 |
| Equipment technical life | 15 years |  |  |  |  |  |  |  |
| Flared system |  |  |  |  |  |  |  |  |
| Type | Open flare | Open flare | Open flare | Open flare | Open flare | Open flare | Open flare | Open flare |
| Material | 304 stainless steels | 304 stainless steels | 304 stainless steels | 304 stainless steels | 304 stainless steels | 304 stainless steels | 304 stainless steels | 304 stainless steels |
| Height (m) | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 | 3.5 |
| Main wall thickness (mm) | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Equipment technical life | 15 years |  |  |  |  |  |  |  |

##### B.1.1 Forward Action Requests

>>

As this monitoring period is the 1st monitoring period, so this part should describe the “Declare any Forward Action Requests from Design certification”. As the project developer of this project choose combine Design Certification with their first Verification and Performance Review, so there are no Forward Action Requests from Design certification to addressed.

Post-Design Certification changes

>>

##### B.2.1. Temporary deviations from the approved Monitoring & Reporting Plan, methodology or standardized baseline

>>

NA

##### B.2.2. Corrections

>>

NA

##### B.2.3. Changes to start date of crediting period

>>

NA

##### B.2.4. Permanent changes from the Design Certified monitoring plan, applied methodology or applied standardized baseline

>>

NA

##### B.2.5. Changes to project design of approved project

>>

NA

- DESCRIPTION OF MONITORING SYSTEM APPLIED BY THE PROJECT

>>

Parameters to be monitored

According to section B.7.1 of the PDD, parameters need to be monitored is as following,

Table 5 monitoring parameter

| parameter | Measurement methods and procedures |
| --- | --- |
| For Mitigation Measure for Safeguarding Principles |  |
| Employee Training of biogas safety operation | Determined by the training Notice which was held once a year. Till this monitoring period, a total of two trainings were held. The first was conducted before the project was put into operation, all employees involved in this project was trained about and monitoring plan and biogas safety operation on 25/10/2021, the training was conducted on 17/10/2022 during this monitoring period, which can be proved by training Notice and meeting attendance record. |
| SDG 7 |  |
| Total electricity produced, EGd,y | Measured continuously by the electricity meters installed at the outlet of generator in each farm. Total 8 electricity meters were installed. The meter reading was recorded daily by the staff and the electricity generation is not included during the commissioning period. The readout records of electricity meter have been recorded at 24:00 of every day by the staff during the monitoring period. The daily electricity generation is equal to the difference between the previous day's and the next day's readings. Monthly value is the sum of the daily electricity generation. The electricity meters ware calibrated periodically by an officially accredited entity. Collected data is archived in both electronic and printed copies. Detailed information on meters including accuracy, calibration date, calibration agency and calibration standard etc. can be obtained in below Table 6 and Table 7. |
| SDG 8 |  |
| Total number of jobs | Parameters for SDG 8 were determined based on Record keeping book and cross-checked by labor contract. 16 full-time jobs for local people were created in the 1st monitoring period (including 8 females and 8 males). |
| SDG 13 |  |
| Number of animals of type LT produced annually for the year y, Np,LT | Each pig involved in this project has a unique electronic ear tag when was born, which is an electronic device dedicated to the identification and electronic management of animals, can track automatically. This electronic ear tag can be connected to the Data Collection System (DCS), which can store and read information. Therefore, the number of swine produced in the farm can be monitored through the auto tracking devices of electronic ear tag once pig slaughter monthly and obtained by the DCS. At the same time, the technicians in farms record manually the number of swine produced in the farms monthly. Monthly sale records for each farm can be crosschecked during this monitoring period. |
| Number of days animal of type LT is alive in the farm in the year y, Nda,LT | The days of swine alive in the farm can be traced through the electronic ear tag by the technical staff in each farm and obtained by the DCS. At the same time, the days of swine alive in the farm is recorded manually by the technicians in farms a. Monthly sale records for each farm used to crosschecked during this monitoring period. |
| Daily stock of animals in the farm, discounting dead and discarded animals, NAA,LT | Each pig involved in this project has a unique electronic ear tag when was born, which is an electronic device dedicated to the identification and electronic management of animals, can track automatically. This electronic ear tag can be connected to the Data Collection System (DCS), which can store and read information. Therefore, the number of swine produced in the farm can be monitored through the auto tracking devices of electronic ear tag. Traced through the electronic ear tag by the technical staff in each farm and obtained by the DCS. At the same time recorded daily by the technicians in farms from the DCS system. The technicians in farms begin to record the daily stock number of animals from the first day of monitoring period, and this record data has included the new imported animal and discounting dead and discharge animals. The annual average number of animals (NAA,LT) was calculated based on the daily stock of breeding swine in the farms without considering dead animals and discarded animals. |
| Average animal weight of a defined livestock population at the project site, Wsite | Measured by the weight measurers at the end of each month. One weight measure was equipped for each farm at least, total 8 weight measures were included for this project. Wsite was conducted by sampling in the three age groups (Nursery phase, Growing phase and Mature phase) of two type swine in each swine farm monthly. The weight data is recorded by the technicians in farms and then average animal weight of a defined livestock population at the project site was calculated based on these data. The weight measures were calibrated periodically by an officially accredited entity. Detailed information on weight measures including accuracy, calibration date, calibration agency and calibration standard etc. can be obtained in below Table 6 and Table 7. |
| Number of days treatment plant was operational in year y, ndy | Monitored and recorded by PP and the daily readout of flowmeter installed at the outlet of the anaerobic digestion in each farm can be crosscheck that the treatment plant is operational. |
| Fraction of volatile solids directed to aerobic treatment FAer | According to the methodology, FAer should be monitored annually. However, there is no monitoring equip installed in the project activity to monitor the influent into anaerobic digestion and aerobic system during this monitoring period, therefore the value of FAer is applied as 100%, which is conservative and consistency with PDD. |
| Biogas flow, Vf | Measured by the biogas flow meters (biogas flow meter 1 installed at the outlet of the anaerobic digestion, biogas flow meter 2 installed at the inlet of the biogas generator and biogas flow meter 3 installed in the inlet of the flare system) continuously, record hourly and saved automatically in the Data Collection System (DCS). There were three flow meters in each farm and total 24 flow meters were included for this project. Daily biogas flow is equal to the cumulative value of 24 hours, and monthly biogas flow can be accumulated on daily basis. The biogas flow meters used were calibrated periodically by an officially accredited entity. In addition, the biogas flow meter automatically measured the instantaneous flow rate, temperature and pressure per hour, so it is not necessary to install other equipment separately to measure the instantaneous flow rate (Vt,db), the biogas temperature (Tt) and pressure (Pt). Collected data is archived in both electronic and printed copies. Detailed information on flowmeters including accuracy, calibration date, calibration agency and calibration standard etc. can be obtained in below Table 6 and Table 7. |
| Quantity of electricity consumed by the proposed project in year y, ECPJ,j,y | Measured continuously by the electricity meters installed in project site in each farm. Total 8 electricity meters were installed in this project. The electricity meter reading was recorded daily and summarized monthly by the staff and the electricity consumption is not included during commissioning period. The readout records of electricity meter have been recorded at 24:00 of every day by the staff during the monitoring period. The daily electricity consumption is equal to the difference between the previous day's and the next day's readings. The staff in each swine farm recorded the meter reading at 24:00 of last day in each month and then reported the monthly electricity consumption to the grid company. The grid company checked and supplied the reading records of the meters to the PP monthly for confirmation. The electricity meters were calibrated periodically by an officially accredited entity. Collected data is archived in both electronic and printed copies. Detailed information on meters including accuracy, calibration date, calibration agency and calibration standard etc. can be obtained in below Table 6 and Table 7. |
| Average technical transmission and distribution losses for providing electricity to source j in year y, TDLj,y | Sourced from Tool 05 ”Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation (version 03.0)”, the value is 20% in this monitoring period. This value will be renewed once the tool is updated. |
| Volumetric flow of the gaseous stream in time interval t on a dry basis, Vt,db | Measured by flowmeters, which can record hourly and saved automatically in the DCS system. As per described in monitoring parameter Vf, the flowmeters to monitor Vt,db are the same as the flowmeters that monitor Vf since the flowmeter can monitor other parameter such as Vt,db, Tt and Pt simultaneously. So, the data can be obtained from the system and the monthly data was exported by the staff at the beginning of next month. Daily biogas flow is equal to the cumulative value of 24 hours, and monthly biogas flow can be accumulated on daily basis. Original collected data can be exported through the system and is archived in electronic. |
| Volumetric fraction of greenhouse gas i in a time interval t on a dry basis, m³ gas i/m³ dry gas, Vi,t,db | Continuously measurement and record hourly by online gas analyser installed at the outlet of the anaerobic digester and saved automatically in the DCS system. One gas analyser was equipped in each farm and there are 8 gas analysers totally. Therefore, the data can be obtained from system and the monthly data was exported by the staff at the beginning of next month. Daily and monthly CH4 concentration was calculated based on the 24-hour average and daily average CH4 concentration. Gas analyzers were calibrated periodically by an officially accredited entity. Original collected data can be exported through the system and is archived in electronic. Detailed information on gas analyzers including accuracy, calibration date, calibration agency and calibration standard etc. can be obtained in below Table 6 and Table 7. |
| Temperature of the gaseous stream in time interval t, Tt; Pressure of the gaseous stream in time interval t, Pt. | Continuously Measurement by the biogas flow meter, which can record hourly and saved automatically in the DCS system. As per described in monitoring parameter Vf, the flowmeters to monitor Tt and Pt are the same as the flowmeters that monitor Vf since the flowmeter can monitor other parameter such as Vt,db, Tt and Pt simultaneously. So, the data can be obtained from DCS system and the monthly data was exported by the staff at the beginning of next month. Daily and monthly Temperature and Pressure of the biogas flow was calculated based on the 24-hour average and daily average values. Original collected data can be exported through the system and is archived in electronic. Detailed information on Flow meters including accuracy, calibration date, calibration agency and calibration standard etc. can be obtained in below Table 6 and Table 7. |
| Density of greenhouse gas i in the gaseous stream in time interval t, ρi,t | This parameter was calculated based on the temperature and pressure of the gaseous stream in time interval t. The monitor of the temperature and pressure of the gaseous stream see the description of the above two parameters in this table. |
| Fraction of manure handled in system j in project activity MS%j | As there is no equipment installed to monitor this parameter in the actual operation. so, in the monitoring period, to be conservative, the value of this parameter in the emission reduction calculation is 100%. |
| Maximum methane producing potential of the volatile solid generated by animal type LT, B0, LT | Not monitored during this monitoring period and the value is taken from IPCC 2006, which is a published source. The parameter value will be updated on latest available public data source. |
| Type of barn and AWMS, Type | During this monitoring period, the type of barn and AWMS was not changed as the design. |
| Annual average ambient temperature at project site, T | As this project is located in Heyuan City, Guangdong Province, so the project site temperature is sourced from Heyuan Municipal Government and cross check with the public website. The average project site temperature sourced from Heyuan Municipal Government is 20.7℃ during this monitoring period covering from 01/11/2021 to 31/10/2022 while the annual average ambient temperature from public website is 21.0℃, which is obtained based on years of history, so the actual temperature at project site in this monitoring period is higher than the value from Heyuan Municipal Government. Therefore, the value of MCF is 78% with the lower temperature of 20.7℃ as per Appendix 3 of ACM0010(Version 08.0). Then multiplying MCF values with a value of 0.94, so 73.32% of MCFj is still used for calculation during in this monitoring period, which is consistent with the value applied in PDD. It is conservative. |
| Annual average nitrogen excretion per head of a defined livestock population estimated as described in Appendix 2, NEXLT,y | The parameter is calculated as the equation of option 2 in Appendix 2 of methodology ACM0010 Version 08.0 and the value of other parameter applied in the equation is taken from IPCC 2006, which is a published source. The parameter value will be updated on latest available public data source. While for parameter Wsite, it is monitored during this MR, please refer to monitoring parameter Wsite |

Monitoring framework

The project owner is responsible for the whole monitoring work. The GS Monitoring Team is established to collect and record monitoring data within the project boundary. The GS monitoring team is responsible for the normal operation of the AWMSs and the collection and record of all the monitoring data. All the data is reviewed by the project developer and VVB. Each member of the GS monitoring team is trained by the project owner at least once a year. The overall monitoring system structure of the project shows as below:

Figure 3 The Organization Structure of the Monitoring Team

Monitoring equipment and installation

Installation and configuration of monitoring equipment are shown as Figure 4.  In order to ensure measurements with a low degree of uncertainty, the data monitoring equipment have been calibrated and checked by appropriately qualified third party according to appropriate national standard.

For this monitoring period, all the monitoring equipment were calibrated according to relevant regulations.

Figure 4 The monitoring system of the project (Same monitoring system for each farm)

Corrective actions

The whole GS monitoring team fellow recognized standard data evaluation methods to guarantee that the data was reliable and accurate. There was no correction of nonconformities occurred in implementation of the project or the monitoring plan during the 1st monitoring period. For QA/QC procedure, please refer to QA/QC procedures in section D.2 of the monitoring report.

Data management

Records are the most important exercise in relation to the monitoring process. Without accurate and efficient record keeping, project emission reductions cannot be verified.

Once missing data on a given day or a certain period of time happened, the most appropriate values such as daily average or monthly average will be used for calculation, which is to ensure that emission reductions are conservative.

All data collected as part of the monitoring plan shall be archived in electronic form and paper until at least two years after the entry period.

Emergency procedure

Employees need to be familiar with the process flow and emergency situations that may occur during project operation and operate in strict accordance with the technical specifications of each process treatment equipment and monitoring equipment. During the operation of the project, all equipment and related instruments shall be checked regularly according to the manufacturer’s recommendations. If necessary, calibration and/or proper repair have to be carried out immediately.

In case of malfunction and/or damage of any system or piece of the process, it may cause damage to the operation of the system, especially serious damage to the biogas generation system and combustion system it must be dealt with as soon as possible and reported to the responsible technician immediately.

There was no emergency happened during this monitoring period.

Avoiding of double counting

All swine farms involved in this project can be identified through its GPS coordinates, which will prevent the farms counted in the project activity to be part of any other voluntary market or emission reduction mechanism (CDM, CCER, VCS etc.) as well. Besides, the project owner has signed the Declaration of No Double Counting Statement on 28/08/2021. The emission reductions were not double counted during the 1st monitoring period.

Table 6 General description of monitoring equipment

| Monitoring equipment | Parameter to be monitored | Installed location | Type | Series number | Accuracy | Date of calibration | Calibration standard | Validity | Calibration agency |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flow meter 1 | Vf Tt Pt | at the outlet of the anaerobic digestion | MKUGB-01 | Please refer to below Table 7 | 1.5 | 14/10/2021 | JJG1029-2007(Verification Regulation of Vortex-shedding Flowmeter) | 13/10/2023 | Guangdong Institute of Metrology |
| Flow meter 2 | Vf Tt Pt | at the inlet of generator | MKUGB-01 |  | 1.5 |  |  |  | Guangdong Institute of Metrology |
| Flow meter 3 | Vf Tt Pt | in inlet of flare | MKUGB-01 |  | 1.5 |  |  |  | Guangdong Institute of Metrology |
| Electricity meter 1 | EGd,y | at the outlet of generator | DTSY1878 |  | 0.5s | 11/10/2021 | JJG596-2012 (Electrical Meters for Measuring Alternating-current Electrical Energy) | 10/10/2026 | Guangdong Institute of Metrology |
| Electricity meter 2 | ECPJ,j,y | in the project site | DTSY1878 |  | 0.5s |  |  |  | Guangdong Institute of Metrology |
| Weight measurer | wsite | in each swine farm | HT9800-F1Z |  | Ⅲ level | 22/10/2021, 17/10/2022 | JJG539-2016(Digital Indicator Scale Verification Regulations) | 21/10/2022, 16/10/2023 | Guangdong Institute of Metrology |
| Gas analyser | Vi,t,db | at the outlet of the anaerobic digesters | Gasboard-3500 |  | 1% | 19/10/2021, 17/10/2022 | JJG693-2011(Verification Regulation of Alarmer Detectors of Combustible Gas) | 18/10/2022, 16/10/2023 | Guangdong Institute of Metrology |

Table 7 Series number of monitoring equipment

| Swine farm | Series number |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Flowmeter 1 | Flowmeter 2 | Flowmeter 3 | Electricity meter 1 | Electricity meter 2 | Weight measurer | Gas analyser |
| Yujing swine farm | 202103695 | 202103696 | 202103697 | 22081 | 22061 | 2103006571 | 2103101001 |
| Zhifu swine farm | 202103698 | 202103699 | 202103700 | 22082 | 22062 | 2103006572 | 2103101002 |
| Lianping Nongmu swine farm | 202103701 | 202103702 | 202103703 | 22083 | 22063 | 2103006573 | 2103101003 |
| Longchuan Nongmu swine farm | 202103704 | 202103705 | 202103706 | 22084 | 22064 | 2103006574 | 2103101004 |
| Zijin Nongmu swine farm | 202103707 | 202103708 | 202103709 | 22085 | 22065 | 2103006575 | 2103101005 |
| Zijin Nongye swine farm | 202103710 | 202103711 | 202103712 | 22086 | 22066 | 2103006576 | 2103101006 |
| Dongyuan Nongmu swine farm | 202103713 | 202103714 | 202103715 | 22087 | 22067 | 2103006577 | 2103101007 |
| Heping Nongmu swine farm | 202103716 | 202103717 | 202103718 | 22088 | 22068 | 2103006578 | 2103101008 |

- DATA AND PARAMETERS

Data and parameters fixed ex ante or at renewal of crediting period

>>

SDG13

| Data/parameter | GWPCH4 |
| --- | --- |
| Unit | tCO2e/tCH4 |
| Description | Global Warming Potential of CH4 |
| Source of data | IPCC AR5 |
| Value(s) applied | 28 |
| Choice of data or Measurement methods and procedures | All monitoring, verifications and requests for performance certification of Certified products and / or Impact Statements for the period starting 01 January 2021 shall use the AR5 GWP values (28) for calculating emission reductions and removals according to applicability of global warming potential for gold standard for the global goals projects. |
| Purpose of data | Used in project emission/baseline calculations |
| Additional comment | N/A |

| Data/parameter | GWPN2O |
| --- | --- |
| Unit | tCO2e/tN2O |
| Description | Global Warming Potential of N2O |
| Source of data | IPCC AR5 |
| Value(s) applied | 265 |
| Choice of data or Measurement methods and procedures | All monitoring, verifications and requests for performance certification of Certified products and / or Impact Statements for the period starting 01 January 2021 shall use the AR5 GWP values (265) for calculating emission reductions and removals according to applicability of global warming potential for gold standard for the global goals projects. |
| Purpose of data | Used in project emission/baseline calculations |
| Additional comment | N/A |

| Data/parameter | DCH4 |
| --- | --- |
| Unit | t/m3 |
| Description | Density of CH4 |
| Source of data | ACM0010 Version 08.0 |
| Value(s) applied | 0.00067 |
| Choice of data or Measurement methods and procedures | 0.00067 t/m3 at room temperature 20℃ and 1 atm pressure. |
| Purpose of data | Used in project emission/baseline calculations |
| Additional comment | N/A |

| Data/parameter | MCFj |
| --- | --- |
| Unit | - |
| Description | Methane conversion factor for the baseline AWMSj |
| Source of data | IPCC 2006 Table 10.17, chapter 10, volume 4 |
| Value(s) applied | 73.32% |
| Choice of data or Measurement methods and procedures | MCFj value for uncovered anaerobic lagoon (baseline AWMS) is chosen. A conservativeness factor has been applied by multiplying MCFj value (i.e., 78%) with a value of 0.94, to account for the 20 per cent uncertainty in the MCFj values as reported by IPCC 2006. For this project, the annual average temperature is 20.7°C and the value of 78 % is applied for ex-ante estimated. Therefore, MCFj value of 73.32% will be applied. The average project site temperature sourced from Heyuan Municipal Government is 20.7℃ during this monitoring period covering from 01/11/2021 to 31/10/2022 while the annual average ambient temperature from public website is 21.0℃, which is obtained based on years of history, so the actual temperature at project site in this monitoring period is higher than the value from Heyuan Municipal Government. Therefore, the value of MCF is 78% with the lower temperature of 20.7℃ as per Appendix 3 of ACM0010(Version 08.0). Then multiplying MCF values with a value of 0.94, so 73.32% of MCFj is still used for calculation during in this monitoring period, which is consistent with the value applied in PDD. It is conservative. |
| Purpose of data | project/baseline emission calculations |
| Additional comment | N/A |

| Data/parameter | MS%Bl,j |
| --- | --- |
| Unit | Fraction |
| Description | Fraction of manure handled in system j in the baseline. |
| Source of data | Project proponents |
| Value(s) applied | 100% |
| Choice of data or Measurement methods and procedures | In this project, the baseline manure management system is uncovered anaerobic lagoon only. The amount of manure handled by the anaerobic lagoon is 100%. |
| Purpose of data | Calculation of baseline emissions |
| Additional comment | N/A |

| Data/parameter | Wdefault |
| --- | --- |
| Unit | kg |
| Description | Default average animal weight of a defined population |
| Source of data | IPCC 2006 Table 10A-7 and 10A-8, chapter 10, volume 4 |
| Value(s) applied | Wdefault (Market swine) =28kg Wdefault (Breeding swine) =28kg |
| Choice of data or Measurement methods and procedures | The values in IPCC 2006 and US-EPA are compared and the lower value from IPCC 2006 is applied. |
| Purpose of data | Calculation of Baseline emissions |
| Additional comment | N/A |

| Data/parameter | VSdefault |
| --- | --- |
| Unit | kg-dm/animal/day |
| Description | Default value for the volatile solid excretion per day on a dry-matter basis for a defined livestock population |
| Source of data | IPCC 2006 Table 10A-7 and 10A-8, chapter 10, volume 4 |
| Value(s) applied | VSdefault(Market swine)=0.3 VSdefault(Breeding swine)=0.3 |
| Choice of data or Measurement methods and procedures | The values in IPCC 2006 and US-EPA are compared and the lower value from IPCC 2006 is applied. |
| Purpose of data | Used in project emission/baseline calculations |
| Additional comment | N/A |

| Data/parameter | Nrate(T) |
| --- | --- |
| Unit | kg-dm/animal/day |
| Description | Default value for the volatile solid excretion per day on a dry-matter basis for a defined livestock population |
| Source of data | IPCC 2006 Table 10.19, chapter 10, volume 4 |
| Value(s) applied | Nrate(T) (Market swine) =0.42 Nrate(T)(Breeding swine) =0.24 |
| Choice of data or Measurement methods and procedures | Default value from IPCC 2006 is applied. |
| Purpose of data | NEXIPCC default calculations |
| Additional comment | N/A |

| Data/parameter | NEXIPCC default |
| --- | --- |
| Unit | kg N/ animal/year |
| Description | Default value for the nitrogen excretion per head of a defined livestock population |
| Source of data | Calculated by the equation: NEXIPCC default =Nrate(T)* TAM/1000*365 |
| Value(s) applied | Period; Market Swine; Breeding Swine. 01/11/2021-30/11/2021; 0.35; 0.20. 01/12/2021-31/12/2021; 0.36; 0.21. 01/01/2022-31/01/2022; 0.36; 0.21. 01/02/2022-28/02/2022; 0.33; 0.19. 01/03/2022-31/03/2022; 0.36; 0.21. 01/04/2022-30/04/2022; 0.35; 0.20. 01/05/2022-31/05/2022; 0.36; 0.21. 01/06/2022-30/06/2022; 0.35; 0.20. 01/07/2022-31/07/2022; 0.36; 0.21. 01/08/2022-31/08/2022; 0.36; 0.21. 01/09/2022-30/09/2022; 0.35; 0.20. 01/10/2022-31/10/2022; 0.36; 0.21. |
| Choice of data or Measurement methods and procedures | NEXIPCC default is calculated as equation 10.30 in IPCC2006, Nrate(T) and TAM are default value from IPCC 2006. |
| Purpose of data | Baseline, Project and leakage emissions calculations |
| Additional comment | N/A |

| Data/parameter | TAM |
| --- | --- |
| Unit | kg animal-1 |
| Description | typical animal mass for livestock category |
| Source of data | IPCC 2006 Table 40A-7 and 10A-8, chapter 10, volume 4 |
| Value(s) applied | TAM (Market swine) =28 TAM (Breeding swine) =28 |
| Choice of data or Measurement methods and procedures | Default value from IPCC 2006 is applied. |
| Purpose of data | NEXIPCC default calculations |
| Additional comment | N/A |

| Data/parameter | Fgas MS,j,LT |
| --- | --- |
| Unit | Fraction |
| Description | Default values for nitrogen loss due to volatilization of NH3 and NOX from manure management |
| Source of data | IPCC 2006 Table 10.22, chapter 10, volume 4 |
| Value(s) applied | FgasMS,j,LT, (anaerobic lagoon) : 40% FgasMS,j,LT , (solid storage) : 45% |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Used in project/baseline emission calculations |
| Additional comment | N/A |

| Data/parameter | EFN2O,D,j |
| --- | --- |
| Unit | Kg N2O-N/kg N |
| Description | Direct N2O emission factor for the treatment system j of the manure management system |
| Source of data | IPCC 2006 Table 10.21, chapter 10, volume 4 |
| Value(s) applied | EFN2O,D,j=0 for anaerobic lagoon and digester, EFN2O,D,j =0.01 for aerobic lagoon |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Used in project/baseline emission calculations |
| Additional comment | N/A |

| Data/parameter | EFN2O,ID |
| --- | --- |
| Unit | kgN2O-N/kg NH3-N and NOX-N |
| Description | Indirect N2O emission factor for the treatment system j of the manure management system |
| Source of data | IPCC 2006 Table 11.3, chapter 11, volume 4 |
| Value(s) applied | 0.01 |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Used in project/baseline emission calculations |
| Additional comment | N/A |

| Data/parameter | EFCH4,default |
| --- | --- |
| Unit | t CH4 leaked / t CH4 produced |
| Description | Default emission factor for the fraction of CH4 produced that leak from the anaerobic digester (fraction) |
| Source of data | Tool 14:” Project and leakage emissions from anaerobic digesters (version02.0) |
| Value(s) applied | 0.05 |
| Choice of data or Measurement methods and procedures | UASB (Upflow Anaerobic Sludge Blanket) type digesters, floating gas holders with no external water seal |
| Purpose of data | Calculation of project emissions |
| Additional comment | N/A |

| Data/parameter | RVS,n |
| --- | --- |
| Unit | Fraction |
| Description | Fraction of volatile solid degraded in AWMS treatment method n of the N treatment steps prior to waste being treated |
| Source of data | Refer to Appendix 1 of methodology ACM0010 |
| Value(s) applied | RVS,n, aerobic treatment and anaerobic digester: 20%, 80% for leakage N2O emission released during project activity RVS,n, one cell lagoon :85% for leakage N2O emission released during baseline scenario |
| Choice of data or Measurement methods and procedures | Estimated from Table provided in Appendix 1 of ACM0010. The most conservative value for the given technology must be used. |
| Purpose of data | project emission/ leakage calculation |
| Additional comment | N/A |

| Data/parameter | RN,n |
| --- | --- |
| Unit | Fraction |
| Description | Nitrogen reduction factor |
| Source of data | Refer to Appendix 1 of methodology ACM0010 |
| Value(s) applied | RN,n, project activity(aerobic treatment and anaerobic digester): 5%, 25% RN,n, baseline ( uncovered anaerobic lagoon) : 80% |
| Choice of data or Measurement methods and procedures | Anaerobic digester has been assumed as a covered first cell of two cell lagoon. Uncovered anaerobic lagoon has been assumed as one cell lagoon. The most conservative value for the given technology has been used. |
| Purpose of data | Calculation of leakage emission |
| Additional comment | N/A |

| Data/parameter | EF1, EF4, EF5 |
| --- | --- |
| Unit | kg N2O-N/kg N for EF1, EF5 and [kg N2O-N/(kg NH3-N and NOX-N) for EF4 |
| Description | Emission factor for N2O emissions from N inputs; from N leaching and runoff; from atmospheric deposition of N on soils and water surfaces |
| Source of data | IPCC 2006 Guidelines default values are be used, since country specific or region-specific data are not available. EF1 from Table 41.1, chapter 11, volume 4. EF4 and EF5 from Table 41.3, chapter 11, volume 4 |
| Value(s) applied | EF1 = 0.010 EF4 =0.010 EF5 = 0.0075 |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Calculation of leakage emission |
| Additional comment | N/A |

| Data/parameter | Fgasm |
| --- | --- |
| Unit | Fraction |
| Description | Fraction of N lost due to volatilization |
| Source of data | Default values from Table 11.3, chapter 11, volume 4 of IPCC 2006 guidelines |
| Value(s) applied | 0.2 |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Used in project/baseline emission calculations |
| Additional comment | N/A |

| Data/parameter | Fleach |
| --- | --- |
| Unit | Fraction |
| Description | Fraction of all N added to/mineralised in managed soils in regions where leaching/runoff occurs that is lost through leaching and runoff |
| Source of data | Default values from Table 11.3, chapter 11, volume 4 of IPCC 2006 guidelines |
| Value(s) applied | 0.3 |
| Choice of data or Measurement methods and procedures | Site specific data is unavailable therefore default values are opted for. |
| Purpose of data | Calculation of leakage emission |
| Additional comment | N/A |

| Data/parameter | MCFd |
| --- | --- |
| Unit | - |
| Description | Methane conversion factor for leakage calculation |
| Source of data | Methodology ACM0010(version 08.0) |
| Value(s) applied | 1 |
| Choice of data or Measurement methods and procedures | According to paragraph 51 of the methodology ACM0010” GHG emission reductions from manure management systems (version 08.0)”, Methane conversion factor for leakage calculation assumed to be equal 1 |
| Purpose of data | Calculation of leakage emission |
| Additional comment | N/A |

| Data/parameter | EFEF,j,y |
| --- | --- |
| Unit | tCO2/MWh |
| Description | Emission factor for electricity generation |
| Source of data | Published by Ministry of Ecology and Environment of the People's Republic of China, which is the DNA of China |
| Value(s) applied | 0.50885 |
| Choice of data or Measurement methods and procedures | According to tool” Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation” |
| Purpose of data | Calculation of project emission |
| Additional comment | N/A |

| Data/parameter | Ru |
| --- | --- |
| Unit | Pa.m3/kmol.K |
| Description | Universal ideal gases constant |
| Source of data | Tool 08:” Tool to determine the mass flow of a greenhouse gas in a gaseous stream (version 03.0)” |
| Value(s) applied | 8,314 |
| Choice of data or Measurement methods and procedures | Value is fixed as Tool 08 |
| Purpose of data | Calculation of project emission |
| Additional comment | / |

| Data/parameter | MMi |
| --- | --- |
| Unit | kg/kmol |
| Description | Molecular mass of greenhouse gas I |
| Source of data | Tool 08:” Tool to determine the mass flow of a greenhouse gas in a gaseous stream (version 03.0)” |
| Value(s) applied | 16.04 kg/mol for methane |
| Choice of data or Measurement methods and procedures | Value is fixed as Tool 08 |
| Purpose of data | Calculation of project emission |
| Additional comment | / |

| Data/parameter | ηflare,m |
| --- | --- |
| Unit | % |
| Description | Flare efficiency in minute m |
| Source of data | Tool 06” Project emissions from flaring (Version 04.0)” |
| Value(s) applied | 0% |
| Choice of data or Measurement methods and procedures | 0% is the conservative value. |
| Purpose of data | Calculation of project emission |
| Additional comment | / |

Data and parameters monitored

>>

SDG 13

| Data / Parameter | Np,LT |
| --- | --- |
| Unit | Number |
| Description | Number of animals of type LT produced annually for the year y |
| Source of data | Production record of market swine |
| Value(s) of monitored parameter | Period; Np,LT. 01/11/2021-30/11/2021; 64,711. 01/12/2021-31/12/2021; 64,818. 01/01/2022-31/01/2022; 64,733. 01/02/2022-28/02/2022; 64,545. 01/03/2022-31/03/2022; 64,592. 01/04/2022-30/04/2022; 64,767. 01/05/2022-31/05/2022; 64,618. 01/06/2022-30/06/2022; 64,539. 01/07/2022-31/07/2022; 64,729. 01/08/2022-31/08/2022; 64,603. 01/09/2022-30/09/2022; 64,907. 01/10/2022-31/10/2022; 64,558. Total; 776,120. |
| Measurement methods and procedures | To be collected for each swine population in all of the pig barns. The number of swine produced in the swine farms had been recorded in the production record of Market swine manually by the responsible staff. Please refer to Section C of the MR for more details. |
| Monitoring frequency | Monitored monthly. |
| QA/QC procedures | Archive electronically during project plus 5 years. |
| Purpose of data | Use for the calculation of NLT |
| Additional comment | According to the Equation 4 in this MR, the calculated NLT is 382,744 heads (776,120 heads *180days/365days). However, according to the number of market pigs produced annually of each farm, the NLT calculated by Equation 4 is rounded to an integer as 382,744 heads. According to the conservative principle, the 382,744 heads was applied in the calculations of emission reductions in MR. Np,LT has been cross-checked through the monthly sale records of 8 swine farms during this monitoring period. And the values between the production record of Market swine and the monthly sale records are consistent. Therefore, Np,LT is reasonable and credible. |

| Data / Parameter | Nda,LT |
| --- | --- |
| Unit | Number |
| Description | Number of days animal of type LT is alive in the farm in the year y |
| Source of data | Production record of market swine |
| Value(s) of monitored parameter | Period; Nda,LT. 01/11/2021-30/11/2021; 180days. 01/12/2021-31/12/2021; 180days. 01/01/2022-31/01/2022; 180days. 01/02/2022-28/02/2022; 180days. 01/03/2022-31/03/2022; 180days. 01/04/2022-30/04/2022; 180days. 01/05/2022-31/05/2022; 180days. 01/06/2022-30/06/2022; 180days. 01/07/2022-31/07/2022; 180days. 01/08/2022-31/08/2022; 180days. 01/09/2022-30/09/2022; 180days. 01/10/2022-31/10/2022; 180days. |
| Measurement methods and procedures | The days of swine alive in the farm was recorded in the Production record of market swine manually by the responsible staff. |
| Monitoring frequency | Monitored monthly. |
| QA/QC procedures | Archive electronically during project plus 5 years. |
| Purpose of data | Use for the calculation of NLT |
| Additional comment | The number of days the animal alive in the farm sourced from the Production record of market swine and was crosschecked through the monthly sale records of 8 swine farms during this monitoring period and it can be concluded that the data is consistent. In addition, this value is in line with the number of days for pigs to be slaughtered by existing large-scale breeding groups in China. |

| Data / Parameter | NAA,LT |
| --- | --- |
| Unit | Number |
| Description | Daily stock of animals in the farm, discounting dead and discarded animals |
| Source of data | Daily stock record of breeding swine |
| Value(s) of monitored parameter | Period; NAA,LT. 01/11/2021-30/11/2021; 55,275. 01/12/2021-31/12/2021; 55,223. 01/01/2022-31/01/2022; 55,193. 01/02/2022-28/02/2022; 55,025. 01/03/2022-31/03/2022; 55,278. 01/04/2022-30/04/2022; 55,305. 01/05/2022-31/05/2022; 55,291. 01/06/2022-30/06/2022; 55,063. 01/07/2022-31/07/2022; 55,127. 01/08/2022-31/08/2022; 54,896. 01/09/2022-30/09/2022; 55,179. 01/10/2022-31/10/2022; 55,077. Average; 55,161. |
| Measurement methods and procedures | Monitored daily through the auto tracking devices of electronic ear tag and the data can be obtained from DCS. The technicians in farms record the daily stock number of animal, and this record data including the new imported animal and discounting dead and discharge animals. Archive electronically during project plus 5 years Please see the Section C of MR for details. |
| Monitoring frequency | daily |
| QA/QC procedures | The system on monitoring the number of livestock population was described in the PDD. |
| Purpose of data | Use for the calculation of baseline/project emission and leakage |
| Additional comment | The system on monitoring the daily stock of breeding swine in the farm was descried in Section C of MR. |

| Data / Parameter | Wsite |
| --- | --- |
| Unit | kg |
| Description | Average animal weight of a defined livestock population at the project site |
| Source of data | Weight record table |
| Value(s) of monitored parameter | Period; Market swine; Breeding swine. 01/11/2021-30/11/2021; 57.9; 73.3. 01/12/2021-31/12/2021; 58.1; 73.5. 01/01/2022-31/01/2022; 58.8; 73.1. 01/02/2022-28/02/2022; 58.0; 72.5. 01/03/2022-31/03/2022; 57.8; 73.1. 01/04/2022-30/04/2022; 57.8; 72.0. 01/05/2022-31/05/2022; 58.6; 73.4. 01/06/2022-30/06/2022; 57.6; 73.6. 01/07/2022-31/07/2022; 57.5; 72.1. 01/08/2022-31/08/2022; 58.0; 73.0. 01/09/2022-30/09/2022; 58.3; 73.4. 01/10/2022-31/10/2022; 57.8; 72.2. Average; 58.0; 72.9. |
| Measurement methods and procedures | Measured by the weight measurers and record by the technical staff in each farm monthly Please refer to Section D.4 of the MR for more details. |
| Monitoring frequency | monthly |
| QA/QC procedures | As this project involved 8 swine farms and two types of swine i.e., Market swine and Breeding swine, and as per applied methodology, each defined livestock population should be classified into a minimum of three age categories (The three age categories of marketing swine are classified according to the age in days, i.e. Nursery phase with 30-60days, Growing phase with 60-130days and Mature phase with 130-180days. The three age categories of breeding swine are classified according to the age in days, i.e. Nursery phase with 30-70days, Growing phase with 70-220days and Mature phase with 220-310days.), so the sampling method is Stratified random sampling. The specific sampling methods are as follows: The sampling method is followed as PDD and described as below during this monitoring period. - the overall sample size based on the design population of pigs in each farm firstly is calculated as described in PDD. - As the project involved 8 swine farms, then the sample size in each swine farm is determined based on the proportion of the number of each farm in the total number of 8 farms. - Similarly, the sample size of each age group of Market swine and Breeding swine in each farm is also calculated based on the proportion of the number of each age group of Market swine and Breeding swine to the total number of swine in the farm. - After the sample size in each age group of Market swine and Breeding swine of each swine farm determined, the sample is conducted in every swine farm. Since swine in different age are kept in the different pig houses, samples can be randomly selected from pig houses of this age group. - After the samples are selected, the weight is measured in each farm monthly by weight measurers and recorded in the weight record table. - The sample will be marked after weighting and if the sample swine is still drawn next month, then this sample swine needs to be discarded and a new sample swine is drawn. This will be done every month until the end of the monitoring period. Weight measurers were calibrated on 22/10/2021 and 17/10/2022 by an officially accredited entity in compliance with JJG539-2016 “Verification Regulation of Digital Indicating Weighting Instruments” in China. |
| Purpose of data | Used for estimating VSLT,y |
| Additional comment | Please see section D.4 for detail about sampling. |

| Data/parameter | ndy |
| --- | --- |
| Unit | number |
| Description | Number of days treatment plant was operational in year y. |
| Source of data | Project proponents |
| Value(s) of monitored parameter | The actual number of days 8 treatment plant in each farm that was operational are 365 days during this monitoring periods Period; ndy. 01/11/2021-30/11/2021; 30. 01/12/2021-31/12/2021; 31. 01/01/2022-31/01/2022; 31. 01/02/2022-28/02/2022; 28. 01/03/2022-31/03/2022; 31. 01/04/2022-30/04/2022; 30. 01/05/2022-31/05/2022; 31. 01/06/2022-30/06/2022; 30. 01/07/2022-31/07/2022; 31. 01/08/2022-31/08/2022; 31. 01/09/2022-30/09/2022; 30. 01/10/2022-31/10/2022; 31. Average; 365. |
| Measurement methods and procedures | The staff in each manure treatment plant record the number of operation days daily and then record the total monthly number of operation days in end of each month through monthly operation report. |
| Monitoring frequency | Daily |
| QA/QC procedures | The daily accumulated meter reading of flowmeters installed in the outlet of anaerobic digesters in each farm can be demonstrated that the AWMSs are actually running on a daily basis. The 8 treatment plants is operational every day during this monitoring period according to the actual daily reading of flowmeters installed in the outlet of anaerobic digesters in each farm. |
| Purpose of data | Calculation of Baseline emissions |
| Additional comment | ndy is the number of days treatment plants was operational. As the anaerobic fermentation is the main process of the treatment process for the project, so the operating days of the anaerobic tank in each farm can be considered as the operating days of the treatment plant. as long as the anaerobic tank is running, biogas will be generated, so the period of biogas generation monitored by the flow meter is the operating days of the treatment plant. Therefore, each farm was operational for 365 days and there was no breakdown/ non –operational day at any of the farms according to the daily meter reading of flowmeter installed in the outlet of anaerobic digesters in each farm. |

| Data/parameter | ECPJ,j,y |
| --- | --- |
| Unit | MWh |
| Description | Quantity of electricity consumed by the project in year y |
| Source of data | Electricity daily report |
| Value(s) of monitored parameter | period; ECPJ,j,y (MWh). 01/11/2021-30/11/2021; 154.03. 01/12/2021-31/12/2021; 158.53. 01/01/2022-31/01/2022; 159.41. 01/02/2022-28/02/2022; 143.62. 01/03/2022-31/03/2022; 158.47. 01/04/2022-30/04/2022; 153.93. 01/05/2022-31/05/2022; 158.68. 01/06/2022-30/06/2022; 154.08. 01/07/2022-31/07/2022; 159.32. 01/08/2022-31/08/2022; 159.30. 01/09/2022-30/09/2022; 154.17. 01/10/2022-31/10/2022; 159.21. Total; 1,872.75. |
| Measurement methods and procedures | Use electricity meter 2 installed in the 8 manure treatment plants. The readout records of electricity meter have been recorded at 24:00 of every day by the staff during the monitoring period. The daily electricity consumption is equal to the difference between the previous day's and the next day's readings. Monthly value is the sum of the daily electricity consumption. |
| Monitoring frequency | Continuous measurement and daily recording |
| QA/QC procedures | Total 8 meters were calibrated on 11/10/2021 by an officially accredited entity in compliance with JJG596-2012” Electrical Meters for Measuring Alternating-current Electrical Energy” in China. Please refer to Section C of the MR for more details. |
| Purpose of data | Calculation of project emissions |
| Additional comment | During this monitoring period, most of the electricity consumed by the project came from the electricity generated by the biogas generated from the anaerobic fermentation process of the project, which is belongs to self-generated and self-consumed. Only a small amount of the electricity consumption of AWMSs were sourced from the grid company when the biogas generators were not operation. The quantity of electricity consumed by the project are crosschecked with Grid Company Electricity Statement issued by the grid company, and the value is consistent. |

| Data/parameter | TDLj,y |
| --- | --- |
| Unit | % |
| Description | Average technical transmission and distribution losses for providing electricity to source j in year y |
| Source of data | Tool 05 (Version 03.0)” Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation” |
| Value(s) of monitored parameter | 20 |
| Measurement methods and procedures | Default value of 20% is applied, which is conservative according to tool” Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation” |
| Monitoring frequency | The value can be changed once the tool is updated. |
| QA/QC procedures | Default value of 20% is applied, which is the maximum value as per Methodology tool 05 Version 03.0 and conservative |
| Purpose of data | Calculation of project emission |
| Additional comment | N/A |

| Data / Parameter | FAer |
| --- | --- |
| Unit | Fraction |
| Description | Fraction of volatile solids directed to aerobic treatment |
| Source of data | PDD |
| Value(s) of monitored parameter | 100% |
| Measurement methods and procedures | As this parameter is not monitored in the actual operation. So, in the monitoring period, to be conservative, the value of this parameter in the emission reduction calculation is 100%, which is consistent with PDD. |
| Monitoring frequency | 100% |
| QA/QC procedures | Value of FAer is applied as 100%, which is conservative. |
| Purpose of data | Annually |
| Additional comment | N/A |

| Data/parameter | MS%j |
| --- | --- |
| Unit | Fraction |
| Description | Fraction of manure handled in system j in project activity |
| Source of data | PDD |
| Value(s) of monitored parameter | 100%. As this parameter is not monitored during this monitoring period. so, to be conservative, the value of this parameter in the emission reduction calculation is 100%, which is consistent with PDD. |
| Measurement methods and procedures | All the manure flew into AWMSs to be treated, therefore, value of 100% is applied. |
| Monitoring frequency | Annually |
| QA/QC procedures | All the manure flew into AWMSs to be treated; no any manure will be discharged outside. Therefore, the value of this parameter is 100%. |
| Purpose of data | Calculation of project emissions |
| Additional comment | NA |

| Data/parameter | vf |
| --- | --- |
| Unit | m3 |
| Description | Biogas flow |
| Source of data | Project proponents |
| Value(s) of monitored parameter | period; Vf1- at outlet of the anaerobic digestion (m3); Vf2- at the inlet of biogas generator (m3); Vf3- in inlet of flare system (m3). 01/11/2021-30/11/2021; 2,982,836.72; 2,923,178.78; 0.00. 01/12/2021-31/12/2021; 3,080,997.04; 3,019,380.34; 0.00. 01/01/2022-31/01/2022; 3,080,775.68; 3,019,159.55; 0.00. 01/02/2022-28/02/2022; 2,781,183.28; 2,725,559.60; 0.00. 01/03/2022-31/03/2022; 3,081,180.45; 3,019,556.07; 0.00. 01/04/2022-30/04/2022; 2,981,988.68; 2,922,348.71; 0.00. 01/05/2022-31/05/2022; 3,081,913.41; 3,020,273.38; 0.00. 01/06/2022-30/06/2022; 2,981,436.86; 2,921,806.35; 0.00. 01/07/2022-31/07/2022; 3,080,647.01; 3,019,031.01; 0.00. 01/08/2022-31/08/2022; 3,079,073.49; 3,017,491.81; 0.00. 01/09/2022-30/09/2022; 2,981,434.35; 2,921,804.43; 0.00. 01/10/2022-31/10/2022; 3,080,883.69; 3,019,267.44; 0.00. Total; 36,274,350.66; 35,548,857.47; -. |
| Measurement methods and procedures | Monitored continuously, record hourly and saved automatically in the Data Collection System (DCS). The data can be obtained from this system and the monthly data was exported by the staff at the beginning of next month and then recorded in daily production & operation report. |
| Monitoring frequency | Continuously measured by flowmeter 1, flowmeter 2 and flowmeter 3 |
| QA/QC procedures | Total 24 flow meters were calibrated on 14/10/2021 by an officially accredited entity in compliance with JJG1029-2007” Verification Regulation of Vortex-shedding Flowmeter” in China. Please refer to Section C of the MR for more details. |
| Purpose of data | To calculated CH4 generated in the anaerobic digester in the project situation QCH4,y |
| Additional comment | For this monitoring period, all the biogas produced is used for power generation, no surplus biogas for the flaring system. |

| Data/parameter | Vt,db |
| --- | --- |
| Unit | m³ dry gas/h |
| Description | Volumetric flow of the gaseous stream in time interval t on a dry basis |
| Source of data | Monitored by Flowmeters |
| Value(s) of monitored parameter | period; Vt,db (m³ dry gas/h) monitored by flow meter 1; Vt,db (m³ dry gas/h) monitored by flow meter 2; Vt,db (m³ dry gas/h) monitored by flow meter 3. 01/11/2021-30/11/2021; 2,982,836.72; 2,923,178.78; 0.00. 01/12/2021-31/12/2021; 3,080,997.04; 3,019,380.34; 0.00. 01/01/2022-31/01/2022; 3,080,775.68; 3,019,159.55; 0.00. 01/02/2022-28/02/2022; 2,781,183.28; 2,725,559.60; 0.00. 01/03/2022-31/03/2022; 3,081,180.45; 3,019,556.07; 0.00. 01/04/2022-30/04/2022; 2,981,988.68; 2,922,348.71; 0.00. 01/05/2022-31/05/2022; 3,081,913.41; 3,020,273.38; 0.00. 01/06/2022-30/06/2022; 2,981,436.86; 2,921,806.35; 0.00. 01/07/2022-31/07/2022; 3,080,647.01; 3,019,031.01; 0.00. 01/08/2022-31/08/2022; 3,079,073.49; 3,017,491.81; 0.00. 01/09/2022-30/09/2022; 2,981,434.35; 2,921,804.43; 0.00. 01/10/2022-31/10/2022; 3,080,883.69; 3,019,267.44; 0.00. Average; 36,274,350.66; 35,548,857.47; -. |
| Measurement methods and procedures | Monitored continuously, record hourly and saved automatically in the Data Collection System (DCS). The data can be obtained from this system and the monthly data was exported by the staff at the beginning of next month and then recorded in daily production & operation report. |
| Monitoring frequency | Continuous measurement |
| QA/QC procedures | Total 24 flow meters were calibrated on 14/10/2021 by an officially accredited entity in compliance with JJG1029-2007” Verification Regulation of Vortex-shedding Flowmeter” in China. Please refer to Section C of the MR for more details. |
| Purpose of data | Calculation of project emissions |
| Additional comment | N/A |

| Data/parameter | Vi,t,db |
| --- | --- |
| Unit | m³ gas i/m³ dry gas |
| Description | Volumetric fraction of greenhouse gas i in a time interval t on a dry basis |
| Source of data | Monitored by gas analyzers |
| Value(s) of monitored parameter | period; Average CH4 Concentration. 01/11/2021-30/11/2021; 60.56%. 01/12/2021-31/12/2021; 60.62%. 01/01/2022-31/01/2022; 62.22%. 01/02/2022-28/02/2022; 60.63%. 01/03/2022-31/03/2022; 61.40%. 01/04/2022-30/04/2022; 60.11%. 01/05/2022-31/05/2022; 61.17%. 01/06/2022-30/06/2022; 61.34%. 01/07/2022-31/07/2022; 60.02%. 01/08/2022-31/08/2022; 61.87%. 01/09/2022-30/09/2022; 60.83%. 01/10/2022-31/10/2022; 62.18%. Average; 61.08%. |
| Measurement methods and procedures | Measured continuously and record hourly by online gas analysers installed at the outlet of the anaerobic digesters and saved automatically in the DCS (Data Collection System). The data can be obtained from this system and the monthly data was exported by the staff at the beginning of next month and then recorded in daily production & operation report. Daily and Monthly CH4 concentration was calculated based on the 24-hour average and daily average CH4 concentration. |
| Monitoring frequency | Continuous measurement |
| QA/QC procedures | All the gas analysers were calibrated on 19/10/2021 and 17/10/2022 by an officially accredited entity in compliance with JJG693-2011” Verification Regulation of Alarmer Detectors of Combustible Gas” in China. And the zero verification with an inert gas (N2) and one reading verification with a standard gas (single calibration gas) are implemented in the process of calibration, which is in accordance with JJG693-2011. All calibration gases are from Nanjing Changyuan Industrial Gas Co. Ltd, which is a qualified reference materials manufacturer issued by the General Administration of Quality Supervision, Inspection and Quarantine of the People's Republic of China on 06/09/2017. Please refer to Section C of the MR for more details. |
| Purpose of data | Calculation of project emissions |
| Additional comment | N/A |

| Data/parameter | Tt |
| --- | --- |
| Unit | K |
| Description | Temperature of the gaseous stream in time interval t |
| Source of data | Measured by flow meter and recorded in production record |
| Value(s) of monitored parameter | period; Average Tt (K). 01/11/2021-31/12/2021; 309.65. 01/01/2021-31/01/2021; 309.45. 01/02/1021-28/02/2021; 308.65. 01/03/2021-31/03/3021; 309.75. 01/04/2021-30/04/2021; 308.55. 01/05/2021-31/05/2021; 309.95. 01/06/2021-30/06/2021; 308.35. 01/07/2021-31/07/2021; 308.55. 01/08/2021-31/08/2021; 308.75. 01/09/2021-30/09/2021; 310.95. 01/10/2021-31/10/2021; 308.75. 01/11/2021-30/11/2021; 310.25. Average; 309.30. |
| Measurement methods and procedures | Measured continuously by the biogas flow meters, since the flowmeters can monitor other parameter such as Vt,db, Tt and Pt simultaneously. And record hourly and saved automatically in the DCS system. The readout of flowmeter was indicated degrees Celsius (°C). Therefore, the temperature Tt(K) is calculated as the equation T(K)=t(°C) +273.15 The data can be obtained from this system and the monthly data was exported by the staff at the beginning of next month and then recorded in daily production & operation report. Daily and Monthly temperature was calculated based on the 24-hour average and daily average temperature. |
| Monitoring frequency | Continuous measurement |
| QA/QC procedures | Total 24 flow meters were calibrated on 14/10/2021 by an officially accredited entity in compliance with JJG1029-2007” Verification Regulation of Vortex-shedding Flowmeter” in China. Please refer to Section C of the MR for more details. |
| Purpose of data | Calculation of project emissions |
| Additional comment | N/A |

| Data/parameter | Pt |
| --- | --- |
| Unit | Pa |
| Description | Pressure of the gaseous stream in time interval t |
| Source of data | Measured by flow meter and recorded in production record |
| Value(s) of monitored parameter | period; Average Pt (Pa). 01/11/2021-30/11/2021; 101,325. 01/12/2021-31/12/2021; 101,325. 01/01/2022-31/01/2022; 101,325. 01/02/2022-28/02/2022; 101,325. 01/03/2022-31/03/2022; 101,325. 01/04/2022-30/04/2022; 101,325. 01/05/2022-31/05/2022; 101,325. 01/06/2022-30/06/2022; 101,325. 01/07/2022-31/07/2022; 101,325. 01/08/2022-31/08/2022; 101,325. 01/09/2022-30/09/2022; 101,325. 01/10/2022-31/10/2022; 101,325. Average; 101,325. |
| Measurement methods and procedures | Measured continuously by the biogas flow meters since the flowmeters can monitor other parameter such as Vt,db, Tt and Pt simultaneously. And record hourly and saved automatically in the DCS system. The data can be obtained from this system and the monthly data was exported by the staff at the beginning of next month and then recorded in daily production & operation report. Daily and Monthly pressure was calculated based on the 24-hour average and daily average pressure. |
| Monitoring frequency | Continuous measurement |
| QA/QC procedures | Total 24 flow meters were calibrated on 14/10/2021 by an officially accredited entity in compliance with JJG1029-2007” Verification Regulation of Vortex-shedding Flowmeter” in China. Please refer to Section C of the MR for more details. |
| Purpose of data | Calculation of project emissions |
| Additional comment | N/A |

| Data/parameter | ρi,t |
| --- | --- |
| Unit | kg gas i/m³ gas i |
| Description | Density of greenhouse gas i in the gaseous stream in time interval t |
| Source of data | Calculated |
| Value(s) of monitored parameter | period; ρi,t (kg gas i/m³ gas i). 01/11/2021-31/12/2021; 0.00063. 01/01/2021-31/01/2021; 0.00063. 01/02/1021-28/02/2021; 0.00063. 01/03/2021-31/03/3021; 0.00063. 01/04/2021-30/04/2021; 0.00063. 01/05/2021-31/05/2021; 0.00063. 01/06/2021-30/06/2021; 0.00063. 01/07/2021-31/07/2021; 0.00063. 01/08/2021-31/08/2021; 0.00063. 01/09/2021-30/09/2021; 0.00063. 01/10/2021-31/10/2021; 0.00063. 01/11/2021-30/11/2021; 0.00063. Average; 0.00063. |
| Measurement methods and procedures | Calculated based on temperature of the gaseous stream in time interval t and pressure of the gaseous stream in time interval t. |
| Monitoring frequency | Pressure and temperature are measured continuously by the biogas flow meters |
| QA/QC procedures | Calculated value according to the pressure and temperature. And the parameters of pressure (Pt) and temperature (Tt) is monitored by the flow meters, the flow meters were calibrated on 14/10/2021 by an officially accredited entity in compliance with JJG1029-2007” Verification Regulation of Vortex-shedding Flowmeter” in China. Please refer to monitoring parameter Pt and Tt for details. |
| Purpose of data | Calculation of project emissions |
| Additional comment | NA |

| Data/parameter | B0, LT |
| --- | --- |
| Unit | m3CH4/kg -dm |
| Description | Maximum methane producing potential of the volatile solid generated by animal type LT |
| Source of data | IPCC 2006 table 10A-7 and 10A-8, chapter 10, volume 4 |
| Value(s) applied | B0, LT (Market swine) =0.29 B0, LT (Breeding swine) =0.29 |
| Measurement methods and procedures | B0, LT can be measured as per ISO 11734:1995. As this parameter is not monitored in the actual operation. so, in the monitoring period 0.29 m3CH4/kg -dm is still applied. While the actual methane producing potential of the volatile solid generated by swine manure is 479.4ml/g VS according to the public literature, which is higher. Therefore 0.29 m3CH4/kg -dm applied in monitoring period is conservative. |
| Monitoring frequency | Annually |
| QA/QC procedures | The value is taken from published sources. The parameter value should be updated on latest available public data source. |
| Purpose of data | Used in project emission/baseline calculations |
| Additional comment | N/A |

| Data/parameter | NEXLT,y |
| --- | --- |
| Unit | kg N/animal/year |
| Description | Annual average nitrogen excretion per head of a defined livestock population estimated as described in Appendix 2 |
| Source of data | Calculated as the equation of option 2 in Appendix 2: While, NEXIPCC default calculated as equation 30 of the IPCC 2006, volume 4, chapter 10 NEXIPCC default =Nrate(T)* TAM/1000*365 |
| Value(s) applied | period; Market swine; Breeding swine. 01/11/2021-30/11/2021; 0.73; 0.53. 01/12/2021-31/12/2021; 0.76; 0.55. 01/01/2022-31/01/2022; 0.77; 0.54. 01/02/2022-28/02/2022; 0.68; 0.49. 01/03/2022-31/03/2022; 0.75; 0.54. 01/04/2022-30/04/2022; 0.73; 0.52. 01/05/2022-31/05/2022; 0.76; 0.55. 01/06/2022-30/06/2022; 0.73; 0.53. 01/07/2022-31/07/2022; 0.75; 0.54. 01/08/2022-31/08/2022; 0.76; 0.54. 01/09/2022-30/09/2022; 0.73; 0.53. 01/10/2022-31/10/2022; 0.75; 0.54. |
| Measurement methods and procedures | Nrate(T) and TAM are default value from IPCC 2006 For Wsite, please refer to the monitoring parameter Wsite |
| Monitoring frequency | Annually |
| QA/QC procedures | The parameter is calculated as the equation of option 2 in Appendix 2 and the value of other parameter applied in the equation is taken from published sources (2006 IPCC). The value should be updated on latest available public data source. For Wsite, please refer to the monitoring parameter Wsite |
| Purpose of data | Baseline, Project and leakage emissions calculations |
| Additional comment | N/A |

| Data/parameter | Type |
| --- | --- |
| Unit | / |
| Description | Type of barn and AWMS |
| Source of data | Project proponent |
| Value(s) applied | NA |
| Measurement methods and procedures | The swine barn and AWMS layout and configuration are collected. Archive electronically during project plus 5 years. |
| Monitoring frequency | Once for each monitoring period |
| QA/QC procedures | After the first verification, only changes in the type of barn and AWMS will be reported. |
| Purpose of data | Confirm whether the implementation of project as design |
| Additional comment | Swine barn and AWMS layout and configuration. |

| Data/parameter | T |
| --- | --- |
| Unit | °C |
| Description | Annual average ambient temperature at project site |
| Source of data | Official public information |
| Value(s) applied | Year; T(°C). 2021; 21.0. 2022; NA. |
| Measurement methods and procedures | The average project site temperature sourced from Heyuan Municipal Government is 20.7℃ during this monitoring period covering from 01/11/2021 to 31/10/2022 while the annual average ambient temperature from public website is 21.0℃, which is obtained based on years of history, so the actual temperature at project site in this monitoring period is higher than the value from Heyuan Municipal Government. Therefore, the value of MCF is 78% with the lower temperature of 20.7℃ as per Appendix 3 of ACM0010(Version 08.0). Then multiplying MCF values with a value of 0.94, so 73.32% of MCFj is still used for calculation during in this monitoring period, which is consistent with the value applied in PDD. It is conservative. |
| Monitoring frequency | monthly |
| QA/QC procedures | The Annual average ambient temperature at project site was sourced from Heyuan Municipal Government, which is an official institution. Therefore, the data is creditable. |
| Purpose of data | Used to select the annual MCFj from IPCC 2006 Guidelines |
| Additional comment | NA |

SDG 8

| Data / Parameter | Total number of jobs |
| --- | --- |
| Unit | Number of full-time jobs created |
| Description | Full-time Jobs created for both men and women. |
| Source of data | Record keeping book |
| Value(s) of monitored parameter | 16 full-time jobs are created including 8 males and 8 females |
| Measurement methods and procedures | Source of data was record keeping book and was cross checked by the labor contracts. |
| Monitoring frequency | annually. |
| QA/QC procedures | After the first verification, only changes in employees will be reported. The results shall be cross checked with labor contract. |
| Purpose of data | To demonstrate contribution to SDG 8. |
| Additional comment | N/A |

SDG 7

| Data / Parameter | EGd,y |
| --- | --- |
| Unit | MWh |
| Description | Total electricity produced |
| Source of data | Electricity daily report |
| Value(s) of monitored parameter | period; EGd,y(MWh). 01/11/2021-31/12/2021; 3,811.19. 01/01/2021-31/01/2021; 3,936.30. 01/02/2021-28/02/2021; 3,938.40. 01/03/2021-31/03/2021; 3,554.16. 01/04/2021-30/04/2021; 3,936.02. 01/05/2021-31/05/2021; 3,808.51. 01/06/2021-30/06/2021; 3,938.53. 01/07/2021-31/07/2021; 3,812.53. 01/08/2021-31/08/2021; 3,937.81. 01/09/2021-30/09/2021; 3,935.37. 01/10/2021-31/10/2021; 3,810.24. 01/11/2021-30/11/2021; 3,941.18. Total; 46,360.24. |
| Measurement methods and procedures | Use electricity meter 1 installed at the outlet of biogas generator. The readout records of electricity meter have been recorded at 24:00 of every day by the staff during the monitoring period. The daily electricity generation is equal to the difference between the previous day's and the next day's readings. Monthly value is the sum of the daily electricity generation. |
| Monitoring frequency | Continuous measurement and daily recording |
| QA/QC procedures | Total 8 meters were calibrated on 11/10/2021 by an officially accredited entity in compliance with JJG596-2012” Electrical Meters for Measuring Alternating-current Electrical Energy” in China. Please refer to Section C of the MR for more details. |
| Purpose of data | To demonstrate contribution to SDG 7. |
| Additional comment | N/A |

Mitigation Measure for Safeguarding Principles

| Data / Parameter | Employee Training of biogas safety operation |
| --- | --- |
| Unit | / |
| Description | Employees was trained on the safety operation of the biogas |
| Source of data | Training notice |
| Value(s) of monitored parameter | Training for the biogas safety operation was held on 25/10/2021 and 17/10/2022 |
| Measurement methods and procedures | Training once a year The training content includes basic biogas knowledge, Operating condition control, operating procedure, patrol inspection, Operation management and failure process. All 8 staffs in manure treatment plants and 8 technicians from 8 swine farms attended this training. All staff strictly follow the operation procedures in the process of operation and there are no safety accidents occurred during this monitoring period. Therefore, the training is effective. |
| Monitoring frequency | once a year |
| QA/QC procedures | Meeting attendance record was used to cross-checked. |
| Purpose of data | To demonstrate Mitigation Measure for Safeguarding Principles |
| Additional comment | N/A |

Comparison of monitored parameters with last monitoring period

NA. This is the 1st monitoring period, no last monitoring period can be compared.

Implementation of sampling plan

>>

According to the PDD, the sample size is calculated according to the design scale and fixed during the monitoring period. During this monitoring period, the sampling is implemented according to the stratified sampling method for the monitoring of Wsite described in Section C of MR. The detailed steps are as following:

- the overall sample size based on the design population of swine firstly is calculated.

- As the project involved 8 swine farms, then the sample size in each swine farm is adjusted based on the proportion of the number of each farm in the total number of 8 farms.

- Similarly, the sample size of each age group of Market swine and Breeding swine in each farm is also calculated based on the proportion of the number of each age group of Market swine and Breeding swine to the total number of swine in the farm.

- After the sample size in each age group of Market swine and Breeding swine of each swine farm determined, the sample is conducted in every swine farm. Since swine in different age are kept in the different swine houses, samples can be randomly selected from swine houses of this age group.

- After the samples are selected, the weight is measured in each farm monthly by weight measurers and recorded in the weight record table.

- The sample will be marked after weighting and if the sample swine is still drawn next month, then this sample swine needs to be discarded and a new sample swine is drawn, this will be done every month until the end of the monitoring period.

Step 1. overall sample size calculation

According to “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)”, the desired confidence level is 95% and the desired precision is 10%. According to statistical principles and sampling survey method, 423 samples should be sufficient to satisfy the desired confidence and precision. For mean value sampling, the required sampling quantity is calculated by the following steps:

n2= n1N/(N+n1)

n3=Bn2

n4= n3/r

n= n4 (110%)

Where:

Z    the Z-statistic and the value taken is 1.96 (corresponding to 90% confidence level);

σ   the standard deviation;

d   the maximum error of estimate;

X   the mean value of samples;

V   the coefficient of variation, V= σ/X≤1 and its maximum value 1 is taken;

e   the allowed relative sampling error (“precision”) and the value taken is 10%

N   the total number of swine population 481,900;

B   the survey design effect and the B value taken is 1 because for stratification sampling the B value is less than or equal to 1;

r    the survey reply rate and the value taken is 100%;

110%  10% contingency was added to produce the final sample size;

n1, n2, n3, n4, n  the adjusted values after each step with n being the final number of samples to be taken. Actually, n4 is the required number of samples based on calculation, 10% contingency was added to n4 to produce n in this sampling.

| Z | V | e | N | B | r |
| --- | --- | --- | --- | --- | --- |
| 1.96 | 1 | 10% | 481,900 | 1 | 100% |
| n1 | Z2V2/e2 | 384 |  |  |  |
| n2 | n1N/(N+n1) | 384 |  |  |  |
| n3 | Bn2 | 384 |  |  |  |
| n4 | n3/r | 384 |  |  |  |
| n | n4 (110%) | 423 |  |  |  |

Step 2 adjust sample size based on the proportion of the number of each farm in the total number of 10 farms

Stratified sampling (with samples of 423) was used for the monitoring of Wsite. However, the samples number in the 8 subsidiary farms is decimals according to the proportion of the number of each farm in the total number of 8 farms. To be conservative, both the samples number of the 8 subsidiary farms is adjusted to be integer value. As a result, 431 samples of swine population were selected within the project boundary using Excel to ensure conservation.

The distribution of the samples in each swine farm shows as follow:

| NO. | Swine farm | Sample number |
| --- | --- | --- |
| 1 | Yujing swine farm | 14 |
| 2 | Zhifu swine farm | 64 |
| 3 | Lianping Nongmu swine farm | 51 |
| 4 | Longchuan Nongmu swine farm | 22 |
| 5 | Zijin Nongmu swine farm | 25 |
| 6 | Zijin Nongye swine farm | 54 |
| 7 | Dongyuan Nongmu swine farm | 144 |
| 8 | Heping Nongmu swine farm | 57 |
| Total |  | 425 |

Step 3 Determine sample size based on the proportion of the number of each age group of Market swine and Breeding swine to the total number of swine in two kinds of farms.

According to the methodology ACM0010, each defined livestock population should be classified into a minimum of three age categories. Nursery phase, Growing phase and Mature phase in fattening farms and breeding farms was be selected through the proportion of swine population in three age categories during this monitoring period. At the same time, to be conservative, both the samples number of the three age categories is adjusted to be integer value.

In conclusion, 453 samples of swine population are fixed and the distribution of the samples for three age categories in each swine farm shows as follow:

| NO. | Swine farm | Sample number | Marketing swine |  |  | Breeding swine |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Nursery phase | Growing phase | Mature phase | Nursery phase | Growing phase | Mature phase |
| 1 | Yujing swine farm | 15 | 4 | 4 | 4 | 1 | 1 | 1 |
| 2 | Zhifu swine farm | 66 | 19 | 19 | 19 | 3 | 3 | 3 |
| 3 | Lianping Nongmu swine farm | 54 | 15 | 15 | 15 | 3 | 3 | 3 |
| 4 | Longchuan Nongmu swine farm | 26 | 6 | 7 | 7 | 2 | 2 | 2 |
| 5 | Zijin Nongmu swine farm | 28 | 8 | 8 | 8 | 1 | 1 | 2 |
| 6 | Zijin Nongye swine farm | 57 | 15 | 15 | 15 | 4 | 4 | 4 |
| 7 | Dongyuan Nongmu swine farm | 147 | 43 | 43 | 43 | 6 | 6 | 6 |
| 8 | Heping Nongmu swine farm | 60 | 17 | 17 | 17 | 3 | 3 | 3 |
| Total |  | 453 | 127 | 128 | 128 | 23 | 23 | 24 |

Step 4. sampling and measure

After the sample size in each age group of Market swine and Breeding swine of each swine farm determined, the sample and measure are conducted in every swine farm. Since swine in different age are kept in the different pig houses, samples can be randomly selected from pig houses of this age group. After the samples are selected, the weight is measured in each farm monthly by weight measurers and recorded in the weight record table. If the same sample is taken in the following month, then a new sample was taken. This was done every month until the end of the monitoring period.

For this monitoring period, then samples are selected randomly from pig houses of this age group by the technicians in the swine farms and then be measured by weight measures in the end of each month. At the same time, the measured value is recorded in the weight record table by the technicians in the swine farms.

At last, the monthly average Wsite of market swine and breeding swine was calculated by weighted the Wsite of all market swine sample and breeding swine sample.

The Monitoring Team implemented the sampling investigation strictly follow the guidance. The monitoring data will be collected throughout the entire crediting period of the project.

According to Standard: Sampling and surveys of CDM project activities and programmes of activities (Version 09.0), confidence/precision have been calculated and checked as follows:

1. The stratified estimated overall mean

(1)

The sample estimate of the overall mean operation hour is calculated with the equation below:

mStrat=i=akgiN×mi

Where:

mStrat  The stratified estimated overall mean

gi  Size of the ith district where i=a,…,k

N  Population total

mi  Mean of the ith district where i=a,…,k

2. The standard error of the stratified estimated overall mean

(2)

The standard error of the stratified estimated overall mean is calculated as follows:

s.e.mStrat=i=ak(giN)2×(1-nigi)×SDi2ni

Where:

s.e.(mStrat) Standard error of the stratified estimated overall mean

gi  Size of the ith district where i=a,…,k

N  Population total

ni  Number of sampled units the ith district where i=a,…,k

SDi2  Variance of the ith district where i=a,…,k

3. t-value

t-value is depending on:

(i) The level of confidence, and

(ii) The size of the sample.

The t-value associated with 95% confidence and the sample size of 453 is 1.9652 as derived in Microsoft Excel using the TINV function.

4. Precision

The precision associated with an estimate is:

t-value × standard error of the mean

5. Calculation results

For Wsite, according to “Standard: Sampling and surveys for CDM project activities and programmes of activities (Version 09.0)”, the desired confidence level is 95% and the desired precision is 10%. According to statistical principles and sampling survey method, 453 samples should be sufficient to satisfy the desired confidence and precision.

The calculation results are shown below. For more details of these calculations, please refer to the ER calculation spread sheet.

| time | t-value | mStrat | s.e. (mStrat) | Precision | Relative Precision |
| --- | --- | --- | --- | --- | --- |
| 01/11/2021-30/11/2021 | 1.9652 | 5067.53 | 1.1966 | 2.3517 | 0.05% |
| 01/12/2021-31/12/2021 | 1.9652 | 5081.08 | 1.2050 | 2.3681 | 0.05% |
| 01/01/2022-31/01/2022 | 1.9652 | 5159.95 | 1.2260 | 2.4093 | 0.05% |
| 01/02/2022-28/02/2022 | 1.9652 | 5080.15 | 1.1990 | 2.3562 | 0.05% |
| 01/03/2022-31/03/2022 | 1.9652 | 5064.28 | 1.2235 | 2.4045 | 0.05% |
| 01/04/2022-30/04/2022 | 1.9652 | 5034.16 | 1.1902 | 2.3391 | 0.05% |
| 01/05/2022-31/05/2022 | 1.9652 | 5144.56 | 1.2127 | 2.3832 | 0.05% |
| 01/06/2022-30/06/2022 | 1.9652 | 5073.96 | 1.2179 | 2.3934 | 0.05% |
| 01/07/2022-31/07/2022 | 1.9652 | 4991.96 | 1.2013 | 2.3609 | 0.05% |
| 01/08/2022-31/08/2022 | 1.9652 | 5087.55 | 1.2282 | 2.4137 | 0.05% |
| 01/09/2022-30/09/2022 | 1.9652 | 5098.39 | 1.2203 | 2.3981 | 0.05% |
| 01/10/2022-31/10/2022 | 1.9652 | 5067.82 | 1.2064 | 2.3707 | 0.05% |

In conclusion, the 453 samples are sufficient to satisfy the desired confidence and precision.

- CALCULATION OF SDG IMPACTS

Calculation of baseline value or estimation of baseline situation of each SDG Impact

>>

SDG 7 Impact of baseline situation

From 01/11/2021 to 31/12/2021, total electricity produced is 0 MWh without the project activity.

From 01/01/2022 to 31/10/2022, total electricity produced is 0 MWh without the project activity.

In baseline scenario, total electricity produced is 0 MWh during the 1st monitoring period from 01/11/2021 to 31/10/2022.

SDG 8 Impact of baseline situation

From 01/11/2021 to 31/12/2021, 0 full-time jobs created in the baseline situation.

From 01/01/2022 to 31/10/2022, 0 full-time jobs created in the baseline situation.

Without the project activity, 0 full-time jobs created during the 1st monitoring period from 01/11/2021 to 31/10/2022.

SDG 13 Impact of baseline situation

From 01/11/2021 to 31/12/2021, the amount of GHGs emission avoided or sequestered in baseline is 0 tCO2e.

From 01/01/2022 to 31/10/2022, the amount of GHGs emission avoided or sequestered in baseline is 0 tCO2e.

Without the project activity, 0 tCO2e GHGs emission can be avoided or sequestered during the 1st monitoring period from 01/11/2021 to 31/10/2022.

Calculation of project value or estimation of project situation of each SDG Impact

>>

SDG 7 Impact of project situation

From 01/11/2021 to 31/12/2021, total electricity produced in project situation is 7,747.49 MWh.

From 01/01/2022 to 31/10/2022, total electricity produced in project situation is 38,612.75 MWh.

For the 1st monitoring period from 01/11/2021 to 31/10/2022, total electricity produced is 46,360.24 MWh.

SDG 8 Impact of project situation

From 01/11/2021 to 31/12/2021, 16 full-time jobs created (including 8 females and 8 males).

From 01/01/2022 to 31/10/2022, 16 full-time jobs created (including 8 females and 8 males).

For the 1st monitoring period from 01/11/2021 to 31/10/2022, 16 full-time jobs created (including 8 females and 8 males).

SDG 13 Impact of project situation

As per section B.6.1 of the PDD, in project situation, the amount of GHGs emissions avoided or sequestered can be obtained by baseline emission – project emissions – leakage emissions, and the baseline emission, project emissions, leakage emissions can be determined by ACM0010” GHG emission reductions from manure management systems (Version 08.0)”, the specific calculation method and calculation result in this monitoring period are described as follows:

Baseline emissions

The baseline emission are：

BEy=BECH4,y+BEN2O,y+BEelecheat,y        (Equation 1 in ACM0010)

where:

| BEy | Baseline emissions in year y (t CO2/yr) |
| --- | --- |
| BECH4, y | Baseline CH4 emissions in year y (t CO2/yr) |
| BEN2O, y | Baseline N2O emissions in year y (t CO2/yr) |
| BEelec/heat,y | Baseline CO2 emissions from electricity and/or heat used in the baseline (t CO2/yr) |

i) Baseline CH4 emissions (BECH4, y)

BECH4,y=GWPCH4*DCH4*j,LT(MCFj*B0,LT*NLT*VSLT,y*MS%Bl,j)    (Equation 2 in ACM0010)

where:

| BECH4, y | Baseline CH4 emissions in year y (t CO2/yr) |
| --- | --- |
| GWPCH4 | Global Warming Potential (GWP) of CH4 (t CO2e/t CH4) |
| DCH4 | Density of CH4 (t/m3). 0.00067t/m3 at room temperature(20℃)and 1am pressure. |
| MCFj | Annual methane conversion factor (MCFj) for the baseline AWMSj. IPCC 2006, Table 10.17, chapter 10, volume 4. |
| B0,LT | Maximum methane producing potential of the volatile solid generated by animal type LT (m3CH4/kg-dm) |
| NLT | Annual average number of animals of type LT for the year y (number) |
| VSLT,y | Annual volatile solid excretions for livestock LT entering all AWMS on a dry matter weight basis (kg -dm/animal/yr) |
| MS%Bl,j | Fraction of manure handled in system j in the baseline. In this project, the baseline manure management system is uncovered anaerobic lagoon only. The amount of manure handled by the anaerobic lagoon is 100%. MS%Bl,j =100% |
| LT | Type of livestock |
| j | Type of treatment system |

Estimation of various variables and parameters for above equations:

(A) As per PDD, VSLT,y shall be determined in following way.

Scaling default IPCC values VSdefault to adjust for a site-specific average animal weight as shown in equation below:

VSLT,y=WsiteWdefault*VSdefault*ndy                      (Equation 4 in ACM0010)

where:

| VSLT,y | Annual volatile solid excretions for livestock LT entering all AWMS on a dry matter weight basis (kg -dm/animal/yr) |
| --- | --- |
| Wsite | Average animal weight of a defined livestock population at the project site (kg) |
| Wdefault | Default average animal weight of a defined population (kg) |
| VSdefault | Default value for the volatile solid excretion per day on a dry-matter basis for a defined livestock population (kg-dm/animal/day) |
| ndy | Number of days treatment plant was operational in year y |

(B)Annual average number of animals of type LT (NLT) shall be determined in following ways

Option 1:

NLT=Nda,LT*Np,LT365          (Equation 5 in ACM0010)

where:

| NLT | Annual average number of animals of type LT for the year y (number) |
| --- | --- |
| Nda,LT | Number of days animal of type LT is alive in the farm in the year y (number) |
| Np,LT | Number of animals of type LT produced annually for the year y (number) |

Option 2:

If the project developer can monitor in a reliable and traceable way the daily stock of animals in the farm, discounting dead animals and animals discarded from the productive process from the daily stock, then the annual average number of animals (NLT) may be calculated as follows:

NLT=1365NAA,LT365     (Equation 6 in ACM0010)

where:

| NLT | Annual average number of animals of type LT for the year y (number) |
| --- | --- |
| NAA,LT | Daily stock of animals of type LT in the farm, discounting dead and discarded animals (number) |

There are two types of swine in this project, i.e., Market swine and Breeding swine. For Market swine, Option 1 is adopted to calculate NLT. For Breeding swine, the PP can monitor the daily stock of breeding swine in a reliable way, discounting dead breeding swine and discarded them from the productive process from the daily stock. So, the Option 2 is adopted to calculate NLT for Breeding swine.

(C) Maximum methane producing potential(B0,LT)

According to applied methodology ACM0010(Version 08.0), this value varies by species and diet. Default values are used, and they are taken from tables 10A-4 through 10A-9 (IPCC 2006 Guidelines for National Greenhouse Gas Inventories volume 4, chapter10).

The proposed project is located in Guangdong Province, China, Asia. According to Table 10A-7 and 10A-8 of IPCC 2006 Guidelines for National Greenhouse Gas Inventories volume 4, chapter10, the maximum methane producing potential(B0,LT) for Market swine and Breeding swine in Asia region is 0.29 m3 CH4/kg VS.

(D) Annual methane conversion factor (MCFj) for the baseline AWMSj

(a)The MCFj values given in Table 10.17, chapter 10, volume 4, IPCC 2006 Guidelines should be used. MCFj values depend on the annual average temperature where the anaerobic manure treatment facility in the baseline existed. For this project, the annual average temperature is 20.7 ℃, so the value of 78% is applied.

(b)A conservativeness factor should be applied by multiplying MCFj values (estimated as per above bullet) with a value of 0.94, to account for the 20% uncertainty in the MCFj values as reported by IPCC 2006.

ii)Baseline N2O emissions (BEN2O,y)

BEN2O,y=GWPN2O*CFN2O-N,N*11000*（EN2O,D,y+EN2O,ID,y）        (Equation 7 in ACM0010)

where：

| BEN2O,y | Annual baseline N2O emissions in (t CO2e/yr) |
| --- | --- |
| GWPN2O | Global Warming Potential (GWP) for N2O (t CO2e/tN2O) |
| CFN2O-N,N | Conversion factor N2O-N to N2O (44/28) |
| EN2O,D,y | Direct N2O emission in year y (kg N2O-N/year) |
| EN2O,ID,y | Indirect N2O emission in year y (kg N2O-N/year) |

EN2O,D,y=j,LTEFN2O,D,j*NEXLT,y*NLT*MS%Bl,j              (Equation 8 in ACM0010)

where：

| EN2O,D,y | Direct N2O emission in year y (kg N2O-N/yr) |
| --- | --- |
| EFN2O,D,j | Direct N2O emission factor for the treatment system j of the manure management system (kg N2O-N/kg N).(Estimated with site-specific, regional or national data if such data is available, otherwise use default EF3 from Table 10.21, chapter 10, volume 4, in the IPCC 2006 Guidelines for National Greenhouse Gas Inventories). The site-specific, regional or national data are not available, so this project activity adopts default EF3. |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/yr) estimated as described in Appendix 2 |
| MS%Bl,j | Fraction of manure handled in system j (fraction) |
| NLT | Annual Average number of animals of type LT for the year y estimated as per equation (4) or (5) (number) |

EN2O,ID,y=j,LTEFN2O,ID*FgasMS,j,LT*NEXLT,y*NLT*MS%Bl,j                   (Equation 9 in ACM0010)

where:

| EN2O,ID,y | Indirect N2O emission in year y (kg N2O-N/year) |
| --- | --- |
| EFN2O,ID | Indirect N2O emission factor for N2O emissions from atmospheric deposition of nitrogen on soils and water surfaces (kgN2O-N/kg NH3-N and NOX-N). (Estimated with site-specific, regional or national data if such data is available. Otherwise, default values for EF4 from Table 11.3, chapter 11, volume 4 of IPCC 2006 Guidelines for National Greenhouse Gas Inventories can be used). The site-specific, regional or national data are not available, so this project activity adopts default EF4. |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/yr) estimated as described in appendix 2 |
| MS%Bl,j | Fraction of manure handled in system j (fraction) |
| FgasMS,j,LT | Default values for nitrogen loss due to volatilisation of NH3 and NOX from manure management (fraction) |
| NLT | Annual Average number of animals of type LT for the year y estimated as per equation (4) or (5) (number) |

Estimation of various variables and parameters for above equations:

(A) As per PDD, the procedure for estimating NEXLT,y  in following way:

In the absence of availability of project specific information on protein intake, which should be justified in the “CDM-PDD”, national or regional data should be used for the nitrogen excretion NEXLT,y, if available. In the absence of such data, default values from Table 10.19 of the IPCC 2006, volume 4, chapter 10) may be used and should be corrected for the animal weight at the project site in the following way:

NEXLT,y=WsiteWdefault*NEXIPCC default                     (Equation 2 in Appendix of ACM0010)

where:

| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/yr) |
| --- | --- |
| Wsite | Average animal weight of a defined livestock population at the project site (kg) |
| Wdefault | Default average animal weight of a defined population (kg) |
| NEX IPCC default | Default value for the nitrogen excretion per head of a defined livestock population (kg N/animal/year) |

NEXIPCC default=NrateT*TAM1000*365                (Equation 10.30 in 2006 IPCC)

where:

| Nrate(T) | the default N excretion rate, kg N/ (1000 kg animal mass)/ day, Table 10.19, chapter 10, volume 4 of IPCC 2006 Guidelines |
| --- | --- |
| TAM | Typical animal mass for livestock in kg/animal |

Table 8 data of parameter

| Time | NLT | NAA,LT | Wsite (kg) |  | VSLT,y (kg-dm/ animal/year) |  | ndy(day) | NEXIPCC (kg N/animal/year) |  | NEXLT,y (kg N/animal/year) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Market swine | Breeding swine | Market swine | Breeding swine | Market swine | Breeding swine |  | Market swine | Breeding swine | Market swine | Breeding swine |
| 01/11/2021-30/11/2021 | 382,744 | 55,275 | 57.9 | 73.3 | 18.61 | 23.56 | 30 | 0.35 | 0.20 | 0.73 | 0.53 |
| 01/12/2021-31/12/2021 | 382,744 | 55,223 | 58.1 | 73.5 | 19.30 | 24.41 | 31 | 0.36 | 0.21 | 0.76 | 0.55 |
| 01/01/2022-31/01/2022 | 382,744 | 55,193 | 58.8 | 73.1 | 19.53 | 24.28 | 31 | 0.36 | 0.21 | 0.77 | 0.54 |
| 01/02/2022-28/02/2022 | 382,744 | 55,025 | 58.0 | 72.5 | 17.40 | 21.75 | 28 | 0.33 | 0.19 | 0.68 | 0.49 |
| 01/03/2022-31/03/2022 | 382,744 | 55,278 | 57.8 | 73.1 | 19.20 | 24.28 | 31 | 0.36 | 0.21 | 0.75 | 0.54 |
| 01/04/2022-30/04/2022 | 382,744 | 55,305 | 57.8 | 72.0 | 18.58 | 23.14 | 30 | 0.35 | 0.20 | 0.73 | 0.52 |
| 01/05/2022-31/05/2022 | 382,744 | 55,291 | 58.6 | 73.4 | 19.46 | 24.38 | 31 | 0.36 | 0.21 | 0.76 | 0.55 |
| 01/06/2022-30/06/2022 | 382,744 | 55,063 | 57.6 | 73.6 | 18.51 | 23.66 | 30 | 0.35 | 0.20 | 0.73 | 0.53 |
| 01/07/2022-31/07/2022 | 382,744 | 55,127 | 57.5 | 72.1 | 19.10 | 23.95 | 31 | 0.36 | 0.21 | 0.75 | 0.54 |
| 01/08/2022-31/08/2022 | 382,744 | 54,896 | 58.0 | 73.0 | 19.26 | 24.25 | 31 | 0.36 | 0.21 | 0.76 | 0.54 |
| 01/09/2022-30/09/2022 | 382,744 | 55,179 | 58.3 | 73.4 | 18.74 | 23.59 | 30 | 0.35 | 0.20 | 0.73 | 0.53 |
| 01/10/2022-31/10/2022 | 382,744 | 55,077 | 57.8 | 72.2 | 19.20 | 23.98 | 31 | 0.36 | 0.21 | 0.75 | 0.54 |

Table 9 The calculation result of BECH4,y

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| GWPCH4 | 28 | 28 | tCO2/tCH4 |
| DCH4 | 0.00067 | 0.00067 | t/m3 |
| MCFj | 78% | 78% | % |
| Conservative Factor | 0.94 | 0.94 | / |
| MCF with cons. Factor | 73.32% | 73.32% | / |
| Bo,LT | 0.29 | 0.29 | m3 CH4 /kg-dm |
| NLT | Refer to Table 8 |  |  |
| Wsite | Refer to Table8 |  |  |
| Wdefault | 28 | 28 | kg |
| VSdefault | 0.3 | 0.3 | Kg-dm/animal/day |
| VSLT,y | Refer to Table 8 |  |  |
| MS%Bl,j | 100% | 100% | % |
| ndy | Refer to Table8 |  |  |
| BECH4,y in 01/11/2021-31/12/2021 | 68,447 |  | tCO2e |
| BECH4,y in 01/01/2022-31/10/2022 | 340,715 |  | tCO2e |
| BECH4,y | 409,162 |  | tCO2e |

Table 10 The calculation result of BEN2O,y

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| EFN2O,D,j | 0 | 0 | kg N2O-N/kg N |
| EFN2O,ID,j | 0.01 | 0.01 | kg N2O-N/kg N |
| Nrate(T) | 0.42 | 0.24 | kg N/1000kg animal mass/day |
| NEXIPCCdefault | Refer to Table 8 |  |  |
| Wsite | Refer to Table 8 |  |  |
| TAM | 28 | 28 | kg |
| Wdefault | 28 | 28 | kg |
| NEXLT,y | Refer to Table 8 |  |  |
| NLT | Refer to Table 8 |  |  |
| MS%Bl,j | 100% | 100% | / |
| Fgasm | 40% | 40% | / |
| GWPN2O | 265 | 265 | tCO2/t N2O |
| CFN20,N-N | 1.57 | 1.57 | Conversion Factor N2O-N to N20 |
| BEN2O,y in 01/11/2021-31/12/2021 | 1,046 |  | tCO2e |
| BEN2O,y in 01/01/2022-31/10/2022 | 5,211 |  | tCO2e |
| BEN2O,y | 6,257 |  | tCO2e |

Table 11 The calculation results of baseline emissions

| Period | BECH4(tCO2e) | BEN2O,y(tCO2e) | BEy(tCO2e) |
| --- | --- | --- | --- |
| 01/11/2021-31/12/2021 | 68,447 | 1,046 | 69,493 |
| 01/01/2022-31/10/2022 | 340,715 | 5,211 | 345,926 |
| 1st monitoring period | 409,162 | 6,257 | 415,419 |

Project emission

project emissions can be calculated as follows,

PEy=PEAD,y+PEAer,y+PEN2O,y+PEECFC,y            (Equation 11 in ACM0010)

where：

| PEy | Project emissions in year y |
| --- | --- |
| PEAD,y | Project emissions associated with the anaerobic digester in year y (t CO2e/yr) |
| PEAer,y | Project CH4 emissions from aerobic AWMS treatment (t CO2e/yr) |
| PEN2O,y | Project N2O emissions in year y (t CO2/yr) |
| PEEC/FC,y | Project emissions from electricity consumption and fossil fuel combustion (t CO2e/yr) |

i) Project emissions associated with the anaerobic digester in year y (PEAD,y)

Based on the methodology ACM0010” GHG emission reductions from manure management systems (Version 08.0)”. PEAD,y is determined using the methodological tool “Project and leakage emissions from anaerobic digesters”.

According to the Tool14” Project and leakage emissions from anaerobic digesters (Version 02.0)”, the project emissions associated with the anaerobic digester (PEAD,y) are determined as follows:

PEAD,y=PEEC,y+PEFC,y+PECH4,y+PEflare,y              (Equation 1 in Tool 14)

where:

| PEAD,y | Project emissions associated with the anaerobic digester in year y (t CO2e) |
| --- | --- |
| PEEC,y | Project emissions from electricity consumption associated with the anaerobic digester in year y (t CO2e) |
| PEFC,y | Project emissions from fossil fuel consumption associated with the anaerobic digester in year y (t CO2e) |
| PECH4, y | Project emissions of methane from the anaerobic digester in year y (t CO2e) |
| PEflare,y | Project emissions from flaring of biogas in year y (t CO2e) |

The biogas generated during the treatment process in this project will be captured for power generation and all the electricity generated from this project will be used by the AWMSs and 8 swine farms. The electricity generated will not be connected to another user or the regional power grid. Unless no electricity generation for this project, the electricity consumption of the project was supplied by the grid company. Since the electricity consumption of the anaerobic digestion system cannot be measured separately from the entire AWMSs, so the project emissions from electricity consumption associated with the anaerobic digester and that is not related to the anaerobic digester is calculated together.

The project emissions from electricity consumption calculated according to the “Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation”,

PEEC,y=j,LTECPJ,J,y*EFEF,j,y*(1+TDLj,y)              (Equation 1 in Tool 05)

Where

| PEEC,y | Project emissions from electricity consumption in year y (t CO2e) |
| --- | --- |
| EGPJ,J,y | Quantity of electricity consumed by the project electricity consumption source j in year y (MWh/yr) |
| EFEF,j, y | Emission factor for electricity generation for source j in year y (t CO2/MWh) |
| TDLj,y | Average technical transmission and distribution losses for providing electricity to source j in year y |

The anaerobic digestion process of this project does not involve the use of fossil fuels, so the project emissions from fossil fuel consumption associated with the anaerobic digester is 0, i.e., PEFC,y=0

The residual gas is sent to flare system (if any) and project emissions from flaring of biogas (PEflare,y) shall be estimated using the tool  “ Project emissions from flaring” (version 04.0).

As described in PDD, project emissions from flaring of biogas (PEflare,y) is determined by the below equation

PEflare,y=GWPCH4,y*Vt,db*υi,t,db*ρi,t*1-ηflare,m*10-3       (Equation 15 in Tool 06)

| Vt,db | Volumetric flow of the residual gas for flaring in time interval t on a dry basis (m³ dry gas/h) |
| --- | --- |
| υi,t,db | Volumetric fraction of greenhouse gas i in the gaseous stream for flaring in a time interval t on a dry basis (m³ gas i/m³ dry gas) |
| ρi,t | Density of greenhouse gas i in the gaseous stream in time interval t (kg gas i/m³ gas i) |

Since the gaseous stream will be distributed though the same pipeline, and the temperature and pressure of gaseous stream can be stable. Therefore the υi,t,db and ρi,t  of residual gas will be same with the biogas produced in the anaerobic digester.

In this monitoring period, all of the biogas generated in the anaerobic process was utilized by the biogas generator to generate electricity.

Determination of project emissions of methane from the anaerobic digester (PECH4,y)

PECH4,y was determined following the step 4 of the applied tool “Project and leakage emissions from anaerobic digesters(Version 02.0)”.Project emissions of methane from the anaerobic digester include emissions during maintenance of the digester, physical leaks through the roof and side walls, and release through safety valves due to excess pressure in the digester. These emissions are calculated using a default emission factor (𝐸𝐹𝐶𝐻4, 𝑑𝑒𝑓𝑎𝑢𝑙𝑡), as follows:

PECH4,y=QCH4,y*EFCH4,default*GWPCH4             (Equation 4 in Tool 14)

where:

| PECH4, y | Project emissions of methane from the anaerobic digester in year y (t CO2e) |
| --- | --- |
| QCH4,y | Quantity of methane produced in the anaerobic digester in year y (t CH4) |
| EFCH4,default | Default emission factor for the fraction of CH4 produced that leak from the anaerobic digester (fraction) |
| GWPCH4 | Global warming potential of CH4 (t CO2/tCH4) |

Estimation of various variables and parameters for above equations:

(A) Quantity of methane produced in the anaerobic digester QCH4,y

As per PDD, the biogas is produced and collected from anaerobic digestion process. The flowmeters are installed at the outlet of the biogas digesters and the measured on an hourly basis time interval. So the quantity of methane produced in the digester in year y (𝑄𝐶𝐻4,𝑦) is the accumulation of the mass flow of methane in the gaseous stream in an hourly basis time interval. i.e., QCH4,y =i=18760Fi,t

The mass flow of greenhouse gas i (Fi,t) is determined as follows:

Fi,t=Vt,db*υi,t,db*ρi,t               (Equation 5 in Tool 08)

ρi,t=Pt*MMiRu*Tt                  (Equation 6 in Tool 08)

where:

| Fi,t | Mass flow of greenhouse gas i in the gaseous stream in time interval t (kg gas/h) |
| --- | --- |
| Vt,db | Volumetric flow of the gaseous stream in time interval t on a dry basis (m³ dry gas/h) |
| υi,t,db | Volumetric fraction of greenhouse gas i in the gaseous stream in a time interval t on a dry basis (m³ gas i/m³ dry gas) |
| ρi,t | Density of greenhouse gas i in the gaseous stream in time interval t (kg gas i/m³ gas i) |
| Pt | Absolute pressure of the gaseous stream in time interval t (Pa) |
| MMi | Molecular mass of greenhouse gas i (kg/kmol) |
| Ru | Universal ideal gases constant (Pa.m3/kmol.K) |
| Tt | Temperature of the gaseous stream in time interval t (K) |

In summary, during this monitoring period, the Project emissions associated with the anaerobic digester in year y (t CO2e) is the sum of Project emissions from electricity consumption and Project emissions of methane from the anaerobic digester in year y (t CO2e). i.e., PEAD,y= PEEC,y + PECH4, y.

ii) Project CH4 emissions from aerobic AWMS treatment (PEAer, y)

IPCC guidelines specify emissions from aerobic lagoons as 0.1 per cent of total methane generating potential of the waste processed, which can be used as a default for all types of aerobic AWMS treatment.

PEAer,y=GWPCH4*DCH4*0.001*FAer*n=1N1-RVS,n*j,LTB0,LT*NLT*VSLT,y*MS%j+PEsl,y               (Equation 12 in ACM0010)

where:

| GWPCH4 | Global Warming Potential (GWP) of CH4 (t CO2e/tCH4) |
| --- | --- |
| RVS,n | Fraction of volatile solid degraded in AWMS treatment method n of the N treatment steps prior to waste being treated (fraction) |
| DCH4 | Density of CH4 (t/m3) |
| FAer | Fraction of volatile solid directed to aerobic system (fraction) |
| LT | Type of livestock |
| Bo,LT | Maximum methane producing potential of the volatile solid generated by animal type LT (m3CH4/kg dm) |
| VSLT,y | Annual volatile solid excretion livestock type LT entering all AWMS on a dry matter weight basis in (kg -dm/animal/yr) |
| NLT | Annual average number of animals of type LT for the year y (number) as estimated in equation (4) or (5) |
| PEsl,y | Project CH4 emissions from sludge disposed of in storage pit prior to disposal during the year y (t CO2e/yr) |
| MS%j | Fraction of manure handled in system j in the project activity (fraction) |

All sludge produced from the aerobic composting will be used for land application, which is calculated as leakage emission, so the PEsl,y=0.

So, PEAer,y=GWPCH4*DCH4*0.001*FAer*n=1N1-RVS,n*j,LTB0,LT*NLT*VSLT,y*MS%j      (Equation 12 in ACM0010)

where:

| GWPCH4 | Global Warming Potential (GWP) of CH4 (t CO2e/tCH4) |
| --- | --- |
| RVS,n | Fraction of volatile solid degraded in AWMS treatment method n of the N treatment steps prior to waste being treated (fraction) |
| DCH4 | Density of CH4 (t/m3) |
| FAer | Fraction of volatile solid directed to aerobic system (fraction) |
| LT | Type of livestock |
| Bo,LT | Maximum methane producing potential of the volatile solid generated by animal type LT (m3CH4/kg dm) |
| VSLT,y | Annual volatile solid excretion livestock type LT entering all AWMS on a dry matter weight basis in (kg -dm/animal/yr) |
| NLT | Annual average number of animals of type LT for the year y (number) as estimated in equation (4) or (5) |
| MS%j | Fraction of manure handled in system j in the project activity (fraction) |

iii) Project N2O emissions in year y (PEN2O,y)

PEN2O,y=GWPN2O*CFN2O-N,N*11000*(EN2O,D,y+EN2O,ID,y)          (Equation 14 in ACM0010)

where:

| PEN2O,y | Project N2O emissions in year y (t CO2/yr) |
| --- | --- |
| GWPN2O | Global Warming Potential (GWP) for N2O (t CO2e/tN2O) |
| CFN2O-N,N | Conversion factor N2O-N to N2O (44/28) |
| EN2O,D,y | Direct N2O emission in year y (kg N2O-N/year) |
| EN2O,ID,y | Indirect N2O emission in year y (kg N2O-N/year) |

EN2O,D,y=j,LTEFN2O,D,j*NEXLT,y*NLT*MS%j             (Equation 15 in ACM0010)

where：

| EN2O,D,y | Direct N2O emission in year y (kg N2O-N/yr) |
| --- | --- |
| EFN2O,D,j | Direct N2O emission factor for the treatment system j of the manure management system (kg N2O-N/kg N) |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/yr) estimated as described in appendix 2 |
| MS%j | Fraction of manure handled in system j (fraction) |
| NLT | Annual Average number of animals of type LT for the year y estimated as per equation (4) or (5) (number) |

EN2O,ID,y=j,LTEFN2O,ID*FgasMS,j,LT*NEXLT,y*NLT*MS%j     (Equation 16 in ACM0010)

where:

| EN2O,ID,y | Indirect N2O emission in year y (kg N2O-N/year) |
| --- | --- |
| EFN2O,ID | Indirect N2O emission factor for N2O emissions from atmospheric deposition of nitrogen on soils and water surfaces (kgN2O-N/kg NH3-N and NOX-N) |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/yr) estimated as described in appendix 2 |
| MS%j | Fraction of manure handled in system j (fraction) |
| FgasMS,j,LT | Default values for nitrogen loss due to volatilisation of NH3 and NOX from manure management (fraction) |
| NLT | Annual Average number of animals of type LT for the year y estimated as per equation (4) or (5) (number) |

iV) Project emissions from use of heat and/or electricity (PEelec/heat)

PEECFC,y=PEEC,y+jPEFC,j,y        (Equation 19 in ACM0010)

where:

| PEEC,y | Project emissions from electricity consumption in year y. The project emissions from electricity consumption will be calculated following the latest version of the “Tool to calculate baseline, project and/or leakage emissions from electricity consumption”. In case, the electricity consumption is not measured then the electricity consumption shall be estimated as follows ECPJ,y=iCPi,y*8760, where CPi,y is the rated capacity (in MW) of electrical equipment i used for the project activity. |
| --- | --- |
| PEFC,y | Project emissions from fossil fuel combustion in process j during the year y. The project emissions from fossil fuel combustion will be calculated following the latest version of the “Tool to calculate project or leakage CO2 emissions from fossil fuel combustion”. For this purpose, the processes j in the tool corresponds to all fossil fuel combustion in the AWMS (not including fossil fuels consumed for transportation of feed material and sludge or any other on-site transportation). |

These emissions should only be considered for consumption of electricity or heat that is not related to the anaerobic digester according to the methodology. As described in above, since the electricity consumption that is not related to the anaerobic digester cannot be separated from the total electricity consumption, therefore the emission for consumption of electricity that is not related to the anaerobic digester is calculated in PEEC,y.

The process that is not related to the anaerobic digester does not involve the use of fossil fuels。 Therefore, PEFC,y=0.

Therefore, PEelec/heat=0

Table 12 data for parameter ECPJ,j,y, QCH4,y and FCH4,RG,m

| period | ECPJ,j,y(MWh) | QCH4,y(t CH4) | FCH4,RG,m (t CH4) |
| --- | --- | --- | --- |
| 01/11/2021-30/11/2021 | 154.03 | 1,138.04 | 0.00 |
| 01/12/2021-31/12/2021 | 158.53 | 1,176.65 | 0.00 |
| 01/01/2022-31/01/2022 | 159.41 | 1,207.62 | 0.00 |
| 01/02/2022-28/02/2022 | 143.62 | 1,062.33 | 0.00 |
| 01/03/2022-31/03/2022 | 158.47 | 1,191.86 | 0.00 |
| 01/04/2022-30/04/2022 | 153.93 | 1,129.26 | 0.00 |
| 01/05/2022-31/05/2022 | 158.68 | 1,187.68 | 0.00 |
| 01/06/2022-30/06/2022 | 154.08 | 1,152.15 | 0.00 |
| 01/07/2022-31/07/2022 | 159.32 | 1,164.87 | 0.00 |
| 01/08/2022-31/08/2022 | 159.30 | 1,200.16 | 0.00 |
| 01/09/2022-30/09/2022 | 154.17 | 1,142.57 | 0.00 |
| 01/10/2022-31/10/2022 | 159.21 | 1,206.89 | 0.00 |

Table 13 the calculation result of PEEC,y

| Parameter | Value | Unit |
| --- | --- | --- |
| ECPJ,j,y | Refer to table 12 | MWh |
| EFEF,j,y | 0.50885 | tCO2/MWh |
| TDLj,y | 20% | / |
| PEEC,y in 01/11/2021-31/12/2021 | 191 | tCO2e |
| PEEC,y in 01/01/2022-31/10/2022 | 953 | tCO2e |
| PEEC,y in 1st monitoring period | 1,144 | tCO2e |

Table 14 the calculation result of PECH4,y

| Parameter | Value | Unit |
| --- | --- | --- |
| GWPCH4 | 28 | tCO2/tCH4 |
| EFCH4,default | 0.05 | / |
| QCH4,y | Refer to table 12 | t CH4 |
| PECH4,y in 01/11/2021-31/12/2021 | 3,241 | tCO2e |
| PECH4,y in 01/01/2022-31/10/2022 | 16,304 | tCO2e |
| PECH4,y in 1st monitoring period | 19,545 | tCO2e |

Table 15 the calculation result of PEflare,y

| Parameter | Value | Unit |
| --- | --- | --- |
| GWPCH4 | 28 | tCO2/tCH4 |
| ηflare,m | 0 | / |
| FCH4,RG,m | 0 | t CH4 |
| PEflare,y in 01/11/2021-31/12/2021 | 0 | tCO2e |
| PEflare,y in 01/01/2022-31/10/2022 | 0 | tCO2e |
| PEflare,y in 1st monitoring period | 0 | tCO2e |

Table 16 calculation of PEAD,y

| Period | PEEC,y (tCO2e) | PECH4,y (tCO2e) | PEflare,y (tCO2e) | PEAD,y (tCO2e) |
| --- | --- | --- | --- | --- |
| 01/11/2021-31/12/2021 | 191 | 3,241 | 0 | 3,432 |
| 01/01/2022-31/10/2022 | 953 | 16,304 | 0 | 17,257 |
| 1st monitoring period | 1,144 | 19,545 | 0 | 20,689 |

Table 17 The calculation result of PEAer,y

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| GWPCH4 | 28 | 28 | tCO2/tCH4 |
| DCH4 | 0.00067 | 0.00067 | t/m3 |
|  | 0.001 | 0.001 | kg N/1000kg animal mass/day |
| FAer | 100% | 100% | / |
| 1-Rvs,1 | 20% | 20% | / |
| B0,LT | 0.29 | 0.29 | m3CH4/kg-dm |
| NLT | Refer to Table 8 |  |  |
| VSLT,y | Refer to Table 8 |  |  |
| MS%j | 100% | 100% |  |
| PEAer,y in 01/11/2021-31/12/2021 | 19 |  | tCO2e |
| PEAer,y in 01/01/2022-31/10/2022 | 93 |  | tCO2e |
| PEAer,y in 1st monitoring period | 112 |  | tCO2e |

Table 18 The calculation result of PEN2O,y

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| EFN2O,D,j | 0 | 0 | kg N2O-N/kg N |
| NEXLT,y | Refer to Table 8 |  |  |
| NLT | Refer to Table 8 |  |  |
| MS%j | 100% | 100% | / |
| EN2O,D,y in 01/11/2021-31/12/2021 | 0 |  | kg N2O-N/year |
| EN2O,D,y in 01/01/2022-31/10/2022 | 0 |  | kg N2O-N/year |
| EN2O,D,y in the 1st monitoring period | 0.00 |  | kg N2O-N/year |
| EFN2O,D,j | 0.006 | 0.006 | kg N2O-N/kg N |
| NEXLT,y | Refer to Table 8 |  |  |
| NLT | Refer to Table 8 |  |  |
| MS%j | 100% | 100% | / |
| EN2O,D,y in 01/11/2021-31/12/2021 | 3,769 |  | kg N2O-N/year |
| EN2O,D,y in 01/01/2022-31/10/2022 | 18,771 |  | kg N2O-N/year |
| EN2O,D,y in the 1st monitoring period | 22,540 |  | kg N2O-N/year |
| EFN2O,ID | 0.01 | 0.01 | kg N2O-N/kg NH3 and NOX-N |
| FgasMS,j,LT | 0.4 | 0.4 | / |
| NEXLT,y | Refer to Table 8 |  |  |
| NLT | Refer to Table 8 |  |  |
| MS%j | 100% | 100% | % |
| GWPN2O | 265 | 265 | tCO2/tN2O |
| EN2O,ID,y in 01/11/2021-31/12/2021 | 2,513 |  | kg N2O-N/year |
| EN2O,ID,y in 01/01/2022-31/10/2022 | 12,514 |  | kg N2O-N/year |
| EN2O,ID,y in the 1st monitoring period | 15,027 |  | kg N2O-N/year |
| EFN2O,iD | 0.01 | 0.01 | kg N2O/kg N |
| Fgasm | 45% | 45% | / |
| NEXLT,y | Refer to Table 8 |  |  |
| NLT | Refer to Table 8 |  |  |
| MS%j | 100% | 100% | % |
| GWPN2O | 265 | 265 | tCO2/tN2O |
| EN2O,ID,y in 01/11/2021-31/12/2021 | 2,827 |  | kg N2O-N/year |
| EN2O,ID,y in 01/01/2022-31/10/2022 | 14,078 |  | kg N2O-N/year |
| EN2O,ID,y in the 1st monitoring period | 16,905 |  | kg N2O-N/year |
| PEN2O,y in 01/11/2021-31/12/2021 | 3,793 |  | tCO2e |
| PEN2O,y n 01/01/2022-31/10/2022 | 18,891 |  | tCO2e |
| PEN2O,y in the 1st monitoring period | 22,684 |  | tCO2e |

Table 19 Calculation results of project emissions

| Period | PEAD,y (tCO2e) | PEAer,y (tCO2e) | PEN2O,y (tCO2e) | PEy (tCO2e) |
| --- | --- | --- | --- | --- |
| 01/11/2021-31/12/2021 | 3,432 | 19 | 3,793 | 7,244 |
| 01/01/2022-31/10/2022 | 17,257 | 93 | 18,891 | 36,241 |
| 1st monitoring period | 20,689 | 112 | 22,684 | 43,485 |

Calculation of leakage

>>

As per section B.6.1 of the PDD, Leakage can be calculated as follows,

LEy=LEPJ,N2O,y-LEBL,N2O,y+LEPJ,CH4,y-LEBL,CH4,y            (Equation 20 in ACM0010)

Where:

| LEPJ, N2O, y | Leakage N2O emissions released during project activity from land application of the treated manure in year y (t CO2e/yr) |
| --- | --- |
| LEBL, N2O, y | Leakage N2O emissions released during baseline scenario from land application of the treated manure in year y (t CO2e/yr) |
| LEPJ, CH4, y | Leakage CH4 emissions released during project activity from land application of the treated manure in year y (t CO2e/yr) |
| LEBL, CH4, y | Leakage CH4 emissions released during baseline scenario from land application of the treated manure in year y (t CO2e/yr) |
| LEAD, y | Leakage emissions associated with the anaerobic digester in year y (t CO2e) |

i) Estimation of leakage N2O emissions released during baseline scenario from land application of the treated manure in year y, LEBL, N2O, y

LEBL,N2O,y=GWPN2O*CFN2O-N,N*11000*(LEN2O,land,y+LEN2O,runoff,y+LEN2O,vol,y)       (Equation 21 in ACM0010)

LEN2O,land,y=EF1*n=1N1-RN,n*LTNEXLT,y*NLT         (Equation 22 in ACM0010)

LEN2O,runoff, y=EF5*Fleach*n=1N1-RN,n*LTNEXLT,y*NLT        (Equation 23 in ACM0010)

LEN2O,vol, y=EF4*n=1N1-RN,n*Fgasm*LTNEXLT,y*NLT      (Equation 24 in ACM0010)

where:

| GWPN2O | Global Warming Potential (GWP) for N2O (t CO2e/tN2O) |
| --- | --- |
| CFN2O-N,N | Conversion factor N2O-N to N2O (44/28) |
| LEN2O,land,y | Leakage N2O emissions from application of manure waste in year y (kg N2O-N/year) |
| LEN2O,runoff,y | Leakage N2O emissions due to leaching and run-off in year y (kg N2O-N/year) |
| LEN2O,vol,y | Leakage N2O emissions due to volatilisation in year y (kg N2O-N/year) |
| Fgasm | Fraction of N lost due to volatilization (fraction) |
| NLT | Annual average number of animals of type LT estimated as per equation (4) or (5) (number) |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/year) estimated as described in appendix 2 |
| EF1 | Emission factor for N2O emissions from N inputs (kg N2O-N/kg N input) |
| EF5 | Emission factor for N2O emissions from N leaching and runoff in (kg N2O-N/kg N leached and runoff) |
| EF4 | Emission factor for N2O emissions from atmospheric deposition of N on soils and water surfaces, [kg N- N2O/ (kg NH3-N + NOX-N volatilized)] |
| Fleach | Fraction of all N added to/mineralised in managed soils in regions where leaching/runoff occurs that is lost through leaching and runoff (fraction) |
| RN,n | Nitrogen reduction factor (fraction) |

ii) Estimation of leakage N2O emissions released during project activity from land application of the treated manure in year y, LEPJ, N2O

LEPJ,N2O=GWPN2O*CFN2O-N,N*11000*(LEN2O,land,y+LEN2O,runoff,y+LEN2O,vol,y)  (Equation 25 in ACM0010)

LEN2O,land,y=EF1*n=1N1-RN,n*LTNEXLT,y*NLT    (Equation 26 in ACM0010)

LEN2O,runoff, y=EF5*Fleach*n=1N1-RN,n*LTNEXLT,y*NLT    (Equation 27 in ACM0010)

LEN2O,vol, y=EF4*n=1N1-RN,n*Fgasm*LTNEXLT,y*NLT    (Equation 28 in ACM0010)

where:

| GWPN2O | Global Warming Potential (GWP) for N2O (t CO2e/tN2O) |
| --- | --- |
| CFN2O-N,N | Conversion factor N2O-N to N2O (44/28) |
| LEN2O,land,y | Leakage N2O emissions from application of manure waste in year y (kg N2O-N/year) |
| LEN2O,runoff,y | Leakage N2O emissions due to leaching and run-off in year y (kg N2O-N/year) |
| LEN2O,vol,y | Leakage N2O emissions due to volatilisation in year y (kg N2O-N/year) |
| Fgasm | Fraction of N lost due to volatilization (fraction) |
| NLT | Annual average number of animals of type LT estimated as per equation (4) or (5) (number) |
| NEXLT,y | Annual average nitrogen excretion per head of a defined livestock population (kg N/animal/year) estimated as described in appendix 2 |
| EF1 | Emission factor for N2O emissions from N inputs (kg N2O-N/kg N input) |
| EF5 | Emission factor for N2O emissions from N leaching and runoff in (kg N2O-N/kg N leached and runoff) |
| EF4 | Emission factor for N2O emissions from atmospheric deposition of N on soils and water surfaces, [kg N- N2O/ (kg NH3-N + NOX-N volatilized)] |
| Fleach | Fraction of all N added to/mineralised in managed soils in regions where leaching/runoff occurs that is lost through leaching and runoff (fraction) |
| RN,n | Nitrogen reduction factor (fraction) |

It is not possible to measure the quantity of manure applied to land in kg manure/yr (QDM) and the nitrogen concentration in kg N/kg manure (NDM) in the manure to estimate the total quantity of nitrogen applied to land. In this case, n=1N1-RN,n*LTNEXLT,y*NLT does not need to be substituted byQDM*NDM.

iii) Estimation of leakage CH4 emissions from land application of the treated manure

The calculation of methane emissions from land application of manure in the baseline and project cases are estimated as below:

LEBL,CH4,y=GWPCH4*DCH4*MCFd*n=1N1-RVS,n*j,LT(B0,LT*NLT*VSLT,y*MS%j)   (Equation 29 in ACM0010)

LEPJ,CH4,y=GWPCH4*DCH4*MCFd*n=1N1-RVS,n*j,LTB0,LT*NLT*VSLT,y*MS%j    (Equation 30 in ACM0010)

where:

| LEBL,CH4,y | Leakage CH4 emissions released during baseline scenario from land application of the treated manure in year y (t CO2e/yr) |
| --- | --- |
| LEPJ,CH4,y | Leakage CH4 emissions released during project activity from land application of the treated manure in year y (t CO2e/yr) |
| RVS,n | Fraction of volatile solid degraded in AWMS treatment method n of the N treatment steps prior to sludge being treated |
| GWPCH4 | Global Warming Potential (GWP) of CH4 (t CO2e/tCH4) |
| DCH4 | Density of CH4 (t/m3) |
| B0,LT | Maximum methane producing potential of the volatile solid generated by animal type LT (m3CH4/kg dm) |
| NLT | Annual average number of animals of type LT estimated as per equation (4) or (5), expressed (number) |
| VSLT,y | Annual volatile solid excretions for livestock LT entering all AWMS on a dry matter weight basis (kg -dm/animal/yr) |
| MS%j | Fraction of manure handled in system j in the project activity (fraction) |
| MCFd | Methane conversion factor (MCF) assumed to be equal to 1 |

Table 20 Calculation results of LEBL,CH4,y、LEPJ,CH4,y、LEBL,N2O,y and LEPJ,N2O,y

| Parameter | Value |  | Unit |
| --- | --- | --- | --- |
| species | Market Swine | Breeding Swine |  |
| NLT | Refer to Table 8 |  |  |
| NEXLT,y | Refer to Table 8 |  |  |
| RN,n | 80% | 80% | / |
| EF1 | 0.01 | 0.01 | kg N2O-N/kg N |
| EF5 | 0.0075 | 0.0075 | kg N2O-N/kg N |
| EF4 | 0.01 | 0.01 | KgN-N2O-N/kg NH3-N+NOX-N |
| Fleach | 0.3 | 0.3 | / |
| Fgasm | 0.2 | 0.2 | / |
| GWPN2O | 265 | 265 | tCO2/t N2O |
| LEN2O,land,y in 01/11/2021-31/12/2021 | 1,256 |  | kg N2O-N/year |
| LEN2O,land,y in 01/01/2022-31/10/2022 | 6,257 |  | kg N2O-N/year |
| LEN2O,land,y in 1st monitoring period | 7,513 |  | kg N2O-N/year |
| LEN2O,runoff,y in 01/11/2021-31/12/2021 | 283 |  | 43.00 |
| LEN2O,runoff,y in 01/01/2022-31/10/2022 | 1,408 |  | kg N2O-N/year |
| LEN2O,runoff,y in 1st monitoring period | 1,690 |  | kg N2O-N/year |
| LEN2O,vol,y in 01/11/2021-31/12/2021 | 251 |  | kg N2O-N/year |
| LEN2O,vol,y in 01/01/2022-31/10/2022 | 1,251 |  | kg N2O-N/year |
| LEN2O,vol,y in 1st monitoring period | 1,503 |  | kg N2O-N/year |
| LEBL,N2O,y in 01/11/2021-31/12/2021 | 746 |  | tCO2e |
| LEBL,N2O,y in 01/01/2022-31/10/2022 | 3,713 |  | tCO2e |
| LEBL,N2O,y in 1st monitoring period | 4,459 |  | tCO2e |
| NLT | Refer to Table 8 |  |  |
| NEXLT | Refer to Table 8 |  |  |
| RN | 25% | 25% | - |
| RN | 5% | 5% | - |
| EF1 | 0.01 | 0.01 | kg N2O-N/kg N |
| EF5 | 0.0075 | 0.0075 | kg N2O-N/kg N |
| EF4 | 0.01 | 0.01 | KgN-N2O-N/kg NH3-N+NOX-N |
| Fleach | 0.3 | 0.3 | / |
| Fgasm | 0.2 | 0.2 | / |
| LEN2O,land,y in 01/11/2021-31/12/2021 | 4,475 |  | kg N2O-N/year |
| LEN2O,land,y in 01/01/2022-31/10/2022 | 22,291 |  | kg N2O-N/year |
| LEN2O,land,y in 1st monitoring period | 26,766 |  | kg N2O-N/year |
| LEN2O,runoff,y in 01/11/2021-31/12/2021 | 1,007 |  | kg N2O-N/year |
| LEN2O,runoff,y in 01/01/2022-31/10/2022 | 5,015 |  | kg N2O-N/year |
| LEN2O,runoff,y in 1st monitoring period | 6,022 |  | kg N2O-N/year |
| LEN2O,vol,y in 01/11/2021-31/12/2021 | 895 |  | kg N2O-N/year |
| LEN2O,vol,y in 01/01/2022-31/10/2022 | 4,458 |  | kg N2O-N/year |
| LEN2O,vol,y in 1st monitoring period | 5,353 |  | kg N2O-N/year |
| LEPJ,N2O,y in 01/11/2021-31/12/2021 | 2,656 |  | tCO2e |
| LEPJ,N2O,y in 01/01/2022-31/10/2022 | 13,228 |  | tCO2e |
| LEPJ,N2O,y in 1st monitoring period | 15,884 |  | tCO2e |
| GWPCH4 | 28 | 28 | tCO2e/t CH4 |
| DCH4 | 0.00067 | 0.00067 | t/m3 |
| MCFd | 1 | 1 | / |
| VSLT,y | Refer to Table 8 |  |  |
| NLT | Refer to Table 8 |  |  |
| B0,LT | 0.29 | 0.29 | m3 CH4/kg-dm |
| RVS | 85% | 85% | / |
| MS%j | 100% | 100% | / |
| LEBL,CH4,y in 01/11/2021-31/12/2021 | 14,004 |  | tCO2e |
| LEBL,CH4,y in 01/01/2022-31/10/2022 | 69,705 |  | tCO2e |
| LEBL,CH4,y in 1st monitoring period | 83,709 |  | tCO2e |
| GWPCH4 | 28 | 28 | tCO2e/t CH4 |
| DCH4 | 0.00067 | 0.00067 | t/m3 |
| MCFd | 1 | 1 | / |
| VSLT,y | Refer to Table 8 |  |  |
| NLT | Refer to Table 8 |  |  |
| B0,LT | 0.29 | 0.29 | m3 CH4/kg-dm |
| RVS | 80% | 80% | / |
| RVS | 20% | 20% | / |
| MS%j | 100% | 100% | / |
| LEPJ,CH4,y in 01/11/2021-31/12/2021 | 14,937 |  | tCO2e |
| LEPJ,CH4,y y in 01/01/2022-31/10/2022 | 74,352 |  | tCO2e |
| LEPJ,CH4,y1st monitoring period | 89,289 |  | tCO2e |

Table 21 Calculation results of leakage

| Period | LEPJ,CH4,y (tCO2e) | LEBL,CH4,y (tCO2e) | LEBL,N2O,y (tCO2e) | LEPJ,N2O,y (tCO2e) | LEy (tCO2e) |
| --- | --- | --- | --- | --- | --- |
| 01/11/2021-31/12/2021 | 14,937 | 14,004 | 746 | 2,656 | 2,843 |
| 01/01/2022-31/10/2022 | 74,352 | 69,705 | 3,713 | 13,228 | 14,162 |
| 1st monitoring period | 89,289 | 83,709 | 4,459 | 15,884 | 17,005 |

In summary, the baseline emission, project emission, leakage emission and emission reductions in this monitoring period are listed below:

| Period | Baseline emission (tCO2e) | Project emission (tCO2e) | Leakage emission (tCO2e) | Emission reductions (tCO2e) |
| --- | --- | --- | --- | --- |
| 01/11/2021-31/12/2021 | 69,493 | 7,244 | 2,843 | 59,406 |
| 01/01/2022-31/10/2022 | 345,926 | 36,241 | 14,162 | 295,523 |
| 1st monitoring period | 415,419 | 43,485 | 17,005 | 354,929 |

Therefore, the project value or estimation of project situation of SDG13 in this monitoring period is:

Amount of GHGs emission avoided or sequestered from 01/11/2021 to 31/12/2021 is 59,406 tCO2e.

Amount of GHGs emission avoided or sequestered from 01/01/2022 to 31/10/2022 is 295,523 tCO2e.

For the 1st monitoring period from 01/11/2021 to 31/10/2022, Amount of GHGs emission avoided or sequestered is 354,929 tCO2e.

Calculation of net benefits or direct calculation for each SDG Impact

| SDG | SDG Impact | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- | --- |
| 13 | Amount of GHGs emission avoided or sequestered | 0tCO2e | 354,929 tCO2e | 354,929 tCO2e |
| 7 | Total electricity produced | 0 MWh | 46,360.24 MWh | 46,360.24 MWh |
| 8 | Total number of jobs | 0 full-time jobs created | 16 full-time jobs created (8 females and 8 males) | 16 full-time jobs created (8 females and 8 males) |

Furthermore, if the actual methane captured from anaerobic digesters in project activity is lower than (BECH4,y - PEAD,y), then (BECH4,y - PEAD,y) is replaced by actual methane captured QCH4,y.

Total biogas captured through the anaerobic treatment during monitoring period is 36,274,350.66 m3, which equals to 390,882 tCO2e emission. And then 35,548,857.47 m3 of biogas is utilized for power generation after desulfurization purification system. Baseline methane emission (BECH4,y) is 409,162 tCO2e. Project emissions associated with anaerobic digester is (PEAD,y) is 20,689 tCO2e. Actual methane captured from anaerobic digesters is higher than the difference of BECH4,y and  PEAD,y. Therefore, the equation (BECH4,y - PEAD,y) can be used to calculate emission reduction.

Comparison of actual SDG Impacts with estimates in approved PDD

| SDG | Values estimated in ex ante calculation of approved PDD for this monitoring period | Actual values achieved during this monitoring period |
| --- | --- | --- |
| 13 | 423,531 tCO2e | 354,929 tCO2e |
| 7 | 59,435.00 MWh | 46,360.24 MWh |
| 8 | 16 full-time jobs created (8 females and 8 males) | 16 full-time jobs created (8 females and 8 males) |

    - Explanation of calculation of value estimated ex ante calculation of approved PDD for this monitoring period

>>

According to PDD, the estimated outcomes of SDG 7 for the 1st monitoring period are 59,435 MWh electricity produced annually which is calculated according to the amount of manure produced from the number of animals. While the actual outcomes of SDG 7 are 46,360.24 MWh electricity produced in the 1st monitoring period, which is measured by the electricity meters.

For SDG 8, total number of jobs created by the project during the 1st monitoring period were determined by the record keeping book and cross-checked with the labor contract. 16 full-time jobs for local people were created (including 8 females and 8 males), which is consistent with the estimated outcomes in the PDD.

The estimated value of the outcomes of SDG 13 for the 1st monitoring period is 423,531 tCO2e, while the actual outcomes of SDG13 is 354,929 tCO2e. The calculation of the estimated and actual value of SDG 13 is based on methodology ACM0010” GHG emission reductions from manure management systems” (version 08.0).

The mainly difference between the estimated value and actual value for SDG7 and SDG 13 is due to the changes of the number of animals in the swine farm (NLT), the average animal weight (Wsite) and the amount of biogas generation and utilized during the anaerobic treatment.

Remarks on increase in achieved SDG Impacts from estimated value in approved PDD

>>

The actual outcomes of SDG 7 are lower than the estimated value in the PDD. The main reason for the difference is that the actual electricity produced is related to the actual biogas generation through the anaerobic treatment and actual amount of manure produced.

The estimated value of the outcomes of SDG 13 for the 1st monitoring period is 423,531 tCO2e, and the actual value is 354,929 tCO2e, which is lower than the estimated value in PDD. The difference between the estimated value and actual values is mainly due to the number of animals in the swine farm and the average animal weight is lower than the estimated values in the PDD. In addition, for this monitoring period, no surplus biogas is flared, which can result less project emission.

For SDG 8, 16 full-time jobs for local people were created (including 8 females and 8 males), which is consistent with the estimated outcomes in the PDD.

- SAFEGUARDS REPORTING

>>

According to the Section D.1 and Appendix 1 in PDD, only one assessment question answered ‘Potentially’ i.e., in case of biogas is not handled properly during the operation period of the project, methane explosion may be caused. Therefore, mitigation measure for safeguarding principles is required and the parameter of Employee Training of biogas safety operation during the operation period shall be monitored, which is described in section D.2 of this MR.

During this monitoring period, no biogas explosion or leakage was happened. The monitoring of this parameters was fully implemented in accordance with the monitoring plan in PDD and the monitoring activities was successful, so there is no need to update the mitigation measures.

- STAKEHOLDER INPUTS AND LEGAL DISPUTES

List all Inputs and Grievances which have been received via the Continuous Input and Grievance Mechanism together with their respective responses/mitigations.

>>

|  | Method Chosen (include all known details e.g., location of the book, phone, number, identity of mediator) | Justification of Choice (best practice) |
| --- | --- | --- |
| Continuous Input / Grievance Expression Process Book (mandatory) | A grievance book was put in Administration Office of each farm. The contact information is listed on the first page of the book. On each page that follows there is one table, incl. Date, Comment, Action requested from PP, Response from PP, Person designated with responsibility by PP. | All stakeholders have access to provide feedback on this project on this book kept by Administration Office of each farm. Stakeholders are allowed to make anonymous comments should they wish. The project owner will assign one specific person to check the comments in the book daily, record responses, and ensure the safety of the book as well. |
| GS Contact (mandatory) | help@goldstandard.org | Gold standard |
| Telephone access (optional) | Stakeholders can also provide comments via phone. The telephone number (+86 0762-8729999, Qiu Weifen) of the project owner is provided to contact. Calls received will be logged and recorded in the same way as the comment book aforementioned. | Stakeholders are not required to give their personal information. The project owner will assign one specific person to answer the phone call daily, record responses. |
| Internet/email access (optional) | Email address (wangxiaoxing123456@163.com, Qiu Weifen) of the project owner is provided as well for stakeholders to provide comments on the internet. Emails comments received will be logged and recorded in the same way in the comment book. | The project owner will assign one specific person to check the email daily, record responses. |
| Nominated Independent Mediator (optional) | heyuannyncj@163.com | Department of Agriculture and Rural Affairs of Heyuan City |
| Other | A grievance book was put in Administration Office of each farm. The contact information is listed on the first page of the book. On each page that follows there is one table, incl. Date, Comment, Action requested from PP, Response from PP, Person designated with responsibility by PP. | All stakeholders have access to provide feedback on this project on this book kept by Administration Office of each farm. Stakeholders are allowed to make anonymous comments should they wish. The project owner will assign one specific person to check the comments in the book daily, record responses, and ensure the safety of the book as well. |

No inputs/grievances received during the 1st monitoring period through all the methods.

Report on any stakeholder mitigations that were agreed to be monitored.

>>

No inputs/grievances received during the 1st monitoring period; therefore, stakeholder mitigations do not need to be reported.

Provide details of any legal contest that has arisen with the project during the monitoring period

>>

No legal contest or dispute has been arisen with the project during the 1st monitoring period.

Revision History

| Version | Date | Remarks |
| --- | --- | --- |
| 1.1 | 14 October 2020 | Hyperlinked section summary to enable quick access to key sections Improved clarity on Key Project Information Section for POA monitoring Forward action request section Improved Clarity on SDG contribution/SDG Impact term used throughout Clarity on safeguard reporting Clarity on design changes Leakage section added for VER/CER projects Addition of Comparison of monitored parameters with last monitoring period Provision of an accompanying Guide to help the user understand detailed rules and requirements |
| 1.0 | 10 July 2017 | Initial adoption |

As per purchase agreement of anaerobic tanks, the technical lifetime of anaerobic tanks is more than 15 years.

http://www.heyuan.gov.cn/zjhy/hygk/dlhj/content/post_442695.html

https://baike.baidu.com/item/%E6%B2%B3%E6%BA%90/36386?fr=aladdin#3_3

http://www.heyuan.gov.cn/zjhy/hygk/dlhj/content/post_442695.html

https://baike.baidu.com/item/%E6%B2%B3%E6%BA%90/36386?fr=aladdin#3_3

https://cdm.unfccc.int/DNA/bak/index.html

https://www.mee.gov.cn/ywgz/ydqhbh/wsqtkz/202012/t20201229_815386.shtml

Partial electricity used by the project are from Southern China Power Grid (SCPG). And as per 2019 China regional power grid carbon dioxide baseline emission factor published by Ministry of Ecology and Environment of the People’s Republic of China, the EFgrid,OM,y of SCPG is 0.8042 tCO2/MWh and the EFgrid,BM,y of ECPG is 0.2135 tCO2/MWh. Therefore, based on equation (16) of Tool 07 (Version 7.0), EFgrid,CM,y of SCPG can be calculated as 0.8042 tCO2/MWh *0.5+0.2135 tCO2/MWh *0.5=0.50885 tCO2/MWh.

https://zhuanlan.zhihu.com/p/38676811

http://finance.people.com.cn/n1/2017/1121/c1004-29658996.html

The project is located in Guangdong Province, China, Asia. According to Table 10A-7 and 10A-8 of IPCC 2006 Guidelines for National Greenhouse Gas Inventories volume 4, chapter10, the maximum methane producing potential(B0,LT) for Market swine and Breeding swine in Asia region is 0.29 m3 CH4/kg VS.

http://www.doc88.com/p-9902182440715.html

http://www.heyuan.gov.cn/zjhy/hygk/dlhj/content/post_442695.html

https://baike.baidu.com/item/%E6%B2%B3%E6%BA%90/36386?fr=aladdin#3_3

According to PDD, this value is the total of population of animals in the farms, including market swine and breeding swine.

Biogas captured during this monitoring period is 36,274,350.66 m3. The quality of methane is 13,960.08 tCH4, which can be calculated by the amount of biogas, the density of biogas and the content of methane in this monitoring period. Then the quality of methane multiplied by the global warming potential of CH4 i.e. 28tCO2/tCH4.is the result of 390,882 tCO2e.Please refer to the ER calculation sheet for more details.