|  | GS 1321 Project KPT Report |  |  |
| --- | --- | --- | --- |
|  | Stove Capital Guatemala Improved Stoves and Water Purification Project |  |  |
| Doc Rev 4 | Issuance 2: Monitoring Period: 4/1/2014 to 3/31/2015 |

Table of Contents

Executive summary	2

MONITORING PLAN DESCRIPTION	3

Sampling Approach & Requirement	3

Target Population & Sampling Frame	4

Sampling Method & Size	4

SURVEY development & implementation	5

Survey Development	5

Enumerator Training	5

Data collection, Analysis and QAQC	5

Field Measurement Schedule & Data Collection	5

Procedures for Minimizing Non-Sampling Errors & Internal QAQC	6

Data Analysis	6

Data Checks for Inconsistencies & Completeness	6

Outlier Removal	6

Qualitative Survey Results	7

Cooking Practices & Seasonality	7

Leakage - Heating	9

Baseline and Project Fuel Consumption and Fuel Consumption Savings	9

Appendix 1 – Survey Questionnaire	11

# Executive summary

As part of the GS 1321 Project Activity (“The Project”), Socorro Maya manufactures and sells improved wood stoves, known as EkoEstufas, to households in Guatemala.

Baseline and Project paired Kitchen Performance Tests (KPTs) were conducted between December 2014 and February 2015 by Green Development. (3rd party professional monitoring team). These KPTs were conducted to measure the average fuel savings of households that adopt the Socorro Maya improved cook stove during the GS 1321 Issuance 1 Monitoring Period.  During the GS review a Forward Action Request (FAR 2) was raised for the PP to carry out additional KPTs at the time of verification in order to crosscheck fuel savings values in the department of San Marcos.  FAR 2 reads:

“The San Marcos (SM) department seems to have high fuel savings i.e., 80% as compared to the AV department i.e., 69%, which is close to the savings reported by the technology provider elsewhere. However, there is no apparent reason for this variation. The emission reduction volume in question is quite low, therefore the PP is requested to carry out the additional KPTs in SM department to ensure the fuel savings level prior to next issuance.”

In response to the FAR the PP, with guidance from the Gold Standard, conducted 41 paired KPT tests (matching baseline and project tests in the same household, 1 set of tests while the household still used the baseline stove, and 1 set of tests after the EkoEstufa was introduced) with CO2 Management Ltd. (3rd party professional monitoring team) in January and February of 2015 to verify the results. Additionally, the PP elected to conduct 55 additional baseline KPT tests on clay, plancha-style, stoves commonly found in San Marcos as a number of these technologies were witnessed in baseline households during monitoring. Sampling and survey procedures were conducted in accordance with Gold Standard Technologies and Practices to Displace Decentralized Thermal Energy Consumption - 11/04/2011 guidelines for the monitoring required for project verifications.

Baseline Stove Types

Baseline stoves include 3-stone and all other unimproved stove types using an open fire with some sort of device to support the pot, whether that be 3 stones, a metal grate or cinder blocks.

Fig 1: Open fire on tabletop (Polloton)        Fig 2: Metal Grate over open fire                Fig 3: Cement blocks, open fire between

Plancha Stove Types

Plancha-style stoves (sometime referred to as “pollos” in the local context) include all stoves that utilize a flat, metal cooking surface known as a “plancha”. Some of these stoves are made from clay or ceramic and physically attached to the home. Some are made from metal, others from cinder block and wood and many feature chimneys. All have in common some attempt to enclose the fire under the metal plancha surface in order to improve efficiency.

Fig 4: In-built clay plancha		     Fig 5: Metal plancha			       Fig 6: Brick plancha

# Testing Results

This report presents the results of the 41 paired household KPT surveys and 55 baseline household KPT surveys that were conducted in-person by CO2 Management enumerators.  The results for the main quantitative parameters determined in the 41 paired KPTs are used in Emissions Reductions (ER) calculations for the Issuance 2 Monitoring Period and are listed in Table 1.

Table 1.  Summary of fuel savings per person meal per day Determined Through Issuance 2 US & MS Analysis

| Parameter | Definition | Value |
| --- | --- | --- |
| Fuel Savings per person meal per Day (Baseline Stoves) | Specific fuel savings for an individual technology of project p against an individual technology of baseline b in year y, in tons/meal/day, as derived from the statistical analysis of the data collected from the field tests (tons/meal/day) | 0.0015 |

The results for the main quantitative parameters determined in the 55 baseline KPTs are compared to baseline KPT figures from the same region in Table 1b.

Table 1b.  Summary of baseline fuel Use per person meal per day Determined Through Issuance 2 US & MS Analysis

| Baseline average fuel per person meal per day (kg) | Avg. (kg) | St. Dev. | Max. | Min. | Confidence 90% | Precision | Meets 90/30? | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline KPT Surveys: Plancha-style Stoves | 4.75 | 3.18 | 21.85 | 1.85 | 0.71 | 15% | Yes | 55 |
| Baseline KPT Surveys: Open Fires/Unimproved | 4.30 | 1.68 | 8.16 | 2.23 | 0.43 | 10% | Yes | 41 |

As evidenced by the summary tables above and KPT survey outcomes, surveys show that plancha-style stoves consume as much or more fuel than a three stone or other unimproved stoves. The PP believes this is the result of poor design and the use of very dense construction materials that tend to absorb heat. Given the relatively insignificant number of plancha-style stoves in baseline scenario homes to date, and for the sake of conservatism, the PP shall treat plancha-style stoves as unimproved stoves consistent with the baseline definition and apply the values associated with the three stone fires for the calculation of ERs for all baseline households included in Issuance 2. Thus the remainder of this report will focus on the paired KPT results.

# MONITORING PLAN DESCRIPTION

## Sampling Approach & Requirement

At the request of Gold Standard, the PP performed paired KPTs for the Issuance 2 monitoring period, in which the enumerators surveyed the same household for Baseline and Project scenarios. Since this was a paired study, the PP and CO2 Management randomly selected households which agreed to pre-order stoves and participate in both the baseline and project portion of the survey.  Sampling procedures were conducted by the PP and CO2 Management in accordance with the KPT sampling requirement as stated in the Gold Standard methodology Technologies and Practices to Displace Decentralized Thermal Energy Consumption - 11/04/2011.

Sample size was estimated using data from San Marcos communities from Issuance 1 (See Table 2 below) to estimate the number of households that would meet the required confidence interval. Looking at this data, we were able to reasonably estimate the number of households that would be required to reach 90/30 precision.  The means, number of samples in the community, and Standard Deviation from 3 communities surveyed in San Marcos were entered into the CDM equation for multi-stage sampling. Since the KPTs were to be done in the same region as the monitoring conducted in San Marcos for Issuance 1, we could be reasonably confident that communities in this same region would share similar characteristics and thus require similar sampling numbers to achieve the desired outcome.

Samples were selected randomly from pre-sales lists from the communities to which Socorro Maya had scheduled to deliver stoves.  Thus these communities are representative of typical sales in San Marcos, as they are part of Socorro Maya's normal business practices.  Additionally all sales in San Marcos are focused in a small geographic region so there is little variation across geographies.

Table 2. Sample Size Requirements using cdm equation for multi-stage sampling on issuance 1 data

| Village | Number HH per village | Mean usage per HH/village | Total usage all HH | SD between HH within villages | SD^2*# HH | (yi-y)^2 |
| --- | --- | --- | --- | --- | --- | --- |
| Eden | 19 | 2.14 | 40.72 | 1.18 | 26.59 | 20.26 |
| Las Guayabas | 23 | 2.55 | 58.76 | 0.83 | 15.83 | 183.55 |
| Loma Linda | 13 | 2.78 | 36.17 | 0.81 | 8.60 | 81.85 |

| Parameter | Description | Value |
| --- | --- | --- |
| Confidence | 90% Confidence | 1.65 |
| Precision | 90/30 | 0.3 |
| M | total number of villages | 49 |
| N | Average Number of units per a group | 20.7 |
| u | # units pre-specified per a group | 9 |
| Overall mean | mean (usage) | 2.47 |
| Cluster mean |  | 45.22 |
| SDB | Standard Deviation | 11.95 |
| SDw |  | 0.96 |
| c * | Required number of villages | 3.00 |

## Target Population & Sampling Frame

The target population for the KPT was households in the department of San Marcos in Guatemala that currently use an open fire or other unimproved stove for cooking and are likely to purchase a Socorro Maya improved cook stove.  The sampling frame was identified through a list of pre-sales provided by Socorro Maya. Due to the nature of a paired sample, samples could not be drawn from a sales database, therefore the pre-sales list was used.  Pre-sale households were those that had already expressed a strong interest to Socorro Maya in purchasing an improved stove prior to survey timing.  This method of selecting samples ensures that the selected households are representative of the sales database for San Marcos for the Issuance 2 monitoring period.

## Sampling Method & Size

Per the above calculations from Issuance 1 data, 3 communities in San Marcos were selected randomly from those communities in which sales were scheduled to take place. Since this was a paired KPT sampling, it was conducted using a systematic sampling approach as the communities and households selected were ones which self-identified as intended purchasers of efficient stoves, but whom had not yet converted from their traditional stove to an Ekoestufa.  They had committed to purchase, but had not yet executed the sale. Thus, they represented the most accurate monitoring households since they were intending to purchase a stove as per usual business practices employed in the project. Household selection within each community was conducted by Socorro Maya based on their willingness to participate in both the baseline and project home visits required during the KPT. As per the methodology, a paired sample was conducted with 41 households sampling the same users in the baseline and project scenario.

# SURVEY development & implementation

## Survey Development

The quantitative and qualitative KPT surveys used in Issuance 2 monitoring are provided in Appendix 1. To maintain consistency with baseline and Issuance 1 monitoring, all KPT participants were provided with an Ecofiltro Filter prior to testing to ensure that fuel used for water boiling was not included in the KPT fuel savings value.  The KPT was subsumed and measured total wood consumption, accounting for usage of multiple stoves.  Qualitative surveys were developed based on the PP’s experience conducting KPTs for GS and CDM carbon asset verification in other countries.  Socorro Maya, Ecofiltro reviewed the surveys at the time of their creation to ensure that they were locally appropriate.  Timothy Longwell and Gracia Flores of Zamorano University also reviewed the surveys and provided input to ensure that the surveys were in line with the aforementioned protocols.  Pilot surveys were conducted prior to official Issuance 1 Monitoring Period surveying to ensure all questions and measurement procedures were understood by both enumerators and respondents.

## Enumerator Training

CO2 Management was trained in survey procedures, fuel measurement, moisture meter use and data entry by Josh Lazarus, Carbon Director for the Paradigm Project, who is experienced in the proper methods and procedures of conducting a KPT.  KPT training consisted of a theoretical training, where KPT procedures were explained and trainers reviewed the survey with enumerators.  A practical component was also conducted, ensuring the enumerators had field experience and familiarity with the equipment before conducting surveys.  The training also reviewed possible pitfalls the enumerators may encounter and provided best practices for survey implementation and data entry. Pilot tests were conducted, which gave enumerators the opportunity to familiarize themselves with the testing procedures and equipment.

# Data collection, Analysis and QAQC

## Field Measurement Schedule & Data Collection

The KPTs were conducted in-person by CO2 Management enumerators during January of 2015.

The CO2 Management enumerators who attended the training were assigned field sites in San Marcos and local community members were employed to assist with the surveys and provide translation when needed.  Data entry was done on-site using ODK digital data collection software on Android phones. All data was synched to the cloud real-time, or if service was unavailable, at the end of each day. Incoming data was verified daily by both the CO2 Management study supervisor and The PP. All data was then exported to Microsoft excel and the final dataset was then reviewed by CO2 Management leadership and the PP to mitigate any data entry errors.

## Procedures for Minimizing Non-Sampling Errors & Internal QAQC

The following internal QAQC checks were completed by the CO2 Management Data Manager:

- Questions were multiple choice to facilitate data entry and minimize enumerator error

- A quality check on all data was completed to ensure accuracy of entry

- Basic analysis was conducted to check for any abnormality in the data

- Surveys were reviewed after the first day of monitoring to ensure that data was being collected properly and there were no abnormal results

# Data Analysis

The PP was responsible for KPT data cleaning, consistency checks, outlier removal, and analysis.

## Data Checks for Inconsistencies & Completeness

The PP reviewed the KPT data to ensure that household responses were consistent, and anomalies were reviewed with the CO2 Management supervisor to ensure there were no data-entry errors. In the baseline KPT, 42 households were visited, but 1 elected not to participate in the project, and thus a project KPT could not be conducted on that home. As a result the PP elected to remove this household from the study, resulting in 41 households used for the paired study.

## Outlier Removal

During the detailed data review, the PP removed surveys as outliers if one of the following occurred:

- Survey was incomplete/missing data for a significant number of questions

- Information was inconsistent or did not make sense (households reported consuming negative values of wood, meaning that households did not follow proper KPT procedures or there was an error in fuel measurement).

- Households refused to participate in the survey or were not able to provide wood measurement for one or more of the days which enumerators visited their household

- Average daily fuel per person meals savings was outside of two standard deviations above or below the mean

No surveys had to be removed as outliers, resulting in 41 KPT surveys being used for analysis. Table 3 summarizes KPT households by community:

Table 3.  paired KPT HOUSEHOLDS BY COMMUNITY

| Community | N | % |
| --- | --- | --- |
| linda_vista | 19 | 46% |
| nueva_esperanza | 8 | 20% |
| cancela | 14 | 34% |
| Total | 41 | 100% |

# Qualitative Survey Results

A qualitative survey was conducted as part of the Paired KPTs, and all households selected for survey participated. The purpose of this portion of the survey was to collect information on demographics and kitchen practices of the households participating in the baseline KPT in order to demonstrate that the survey population is representative of the project population. As this was a supplementary KPT activity conducted in a specific region of the project for the purposes of confirming fuel savings, qualitative data collection was purposefully limited to those questions directly impacting fuel savings and cooking practices.

## Cooking Practices & Seasonality

As shown in Table 4, 100% of households reported using firewood exclusively as their baseline fuel type before purchasing their improved stove.

Table 4.  baseline Fuel type

| Baseline Fuel Type | N | % |
| --- | --- | --- |
| Firewood | 41 | 100% |
| Charcoal | 0 | 0% |
| Gas | 0 | 0% |
| Biogas | 0 | 0% |
| Cardboard | 0 | 0% |
| Kerosene | 0 | 0% |
| Agricultural Waste | 0 | 0% |
| Organic Waste | 0 | 0% |
| Firewood Cardboard | 0 | 0% |
| Total | 41 | 100% |

All households reported using their stove for domestic cooking purposes only.  There were no differences found between the number of meals cooked in the rainy versus dry season.  Meals cooked per day were calculated using a 0.25 weighting for two snack meals. As shown in Table , the average number of meals households reported cooking per day is virtually identical for each season.  Therefore, there are no apparent changes in cooking practices between seasons.

Table 5.  average number of meals cooked per day, rainy & dry season

| AVG; N; STDEV; Max; Min. Meals cooked per day in dry season; 20.7; 1; 7.6; 42; 6. Meals cooked per day in wet season; 20.6; 1; 7.5; 42; 6. *There are two snack meals that are weighted by 0.25. Additionally one household did not respond to this question. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

## Leakage - Heating

Tables 6 and 7 below summarize use of stoves for the purpose of heating the home. While the majority (90%) of households reported that they do use their stove specifically to heat their home, the overwhelming majority of those using the stove for heating (97%) stated that they simply allow the fire to die out after cooking, and thus it is concluded that heating does not significantly effect fuelwood consumption.

Table 6. Do you use any stove to specifically heat your home?

|  | N | % |
| --- | --- | --- |
| Yes | 37 | 90% |
| No | 4 | 10% |
| Total | 41 | 100% |

Table 7. If yes, how do you heat your home in relation to your cooking practices?

|  | N | % |
| --- | --- | --- |
| Let the cooking fuel die out after cooking | 36 | 97% |
| Add Fuel | 0 | 0% |
| Start the stove specifically for warmth and not any other purposes | 0 | 0% |
| No Response | 1 | 3% |
| Total | 37 | 100% |

## Baseline and Project Fuel Consumption and Fuel Consumption Savings

For the baseline and project KPTs, households were asked how many people ate at each meal, the age and gender of those eating, type of food that was prepared, and stove(s) used. Enumerators measured the quantity of fuel consumed by taking daily measurements in pounds (lbs) of the fuel supply.  The KPT is a subsumed KPT and measures wood consumed from all stoves present and in use in the home during the test period. Tables 8 through 10 below present the average fuel consumption per day, average person meals per day, and average fuel per person meal per day for baseline, project, and savings.  Fuel consumption savings were calculated by comparing baseline fuel consumption to fuel consumption while using the EkoEstufa.  Meals cooked per day included 0.25 weights for two snack meals.  Fuel savings are exclusive of fuel used for boiling, as participants were provided with water filters prior to the baseline portion of the KPT.

Table 4.  Average daily household fuel consumption, lbs/hh/day

|  | Avg. (lbs) | St. Dev. | Max. | Min. | Confidence 90% | Precision | Meets 90/30? | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline average fuel used per day | 45.99 | 13.35 | 84.58 | 28.56 | 3.43 | 7% | N/A | 41 |
| Project average fuel used per day | 12.45 | 2.76 | 17.47 | 6.01 | 0.71 | 6% | N/A | 41 |
| Average daily fuel savings | 33.55 | 12.73 | 68.16 | 17.74 | 3.27 | 10% | Yes | 41 |

Table 5.  Average Household Person meals per day meals/hh/day

|  | Avg. (lbs) | St. Dev. | Max. | Min. | Confidence 90% | Precision | Meets 90/30? | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline average person meals per day | 13.10 | 5.95 | 38.00 | 4.00 | 1.53 | 12% | No | 41 |
| Project average person meals per day | 15.87 | 6.55 | 38.00 | 4.00 | 1.68 | 11% | No | 41 |

Table 6.  Average daily per person-meal fuel consumption and savings, lbs/person-meal/day

|  | Avg. (lbs) | St. Dev. | Max. | Min. | Confidence 90% | Precision | Meets 90/30? | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline average fuel per person meal per day | 4.30 | 1.68 | 8.16 | 2.23 | 0.43 | 10% | N/A | 41 |
| Project average fuel per person meal per day | 0.94 | 0.50 | 3.17 | 0.43 | 0.13 | 14% | N/A | 41 |
| Average daily fuel per person meals savings | 3.35 | 1.67 | 7.42 | 0.81 | 0.43 | 13% | Yes | 41 |

Consistent with prior findings, the San Marcos region has higher fuel use and savings than other regions in which the project is active.

# Appendix 1 – Survey Questionnaire

DATOS GENERALES DE LA ENCUESTA

- Número de identidad de casa: _________

| Día |  | Mes |  | Año |  |
| --- | --- | --- | --- | --- | --- |

- Hora de Inicio y final:

Hora Inicial:______________________ Hora Final: ___________________

- Fecha:

| 1.Lunes |  | 5. Viernes |  |
| --- | --- | --- | --- |
| 2.Martes |  | 6. Sábado |  |
| 3.Miércoles |  | 7. Domingo |  |
| 4.Jueves |  |  |  |

- Localización:

| Departamento |  | Municipio |  | Comunidad |
| --- | --- | --- | --- | --- |

- Coordenadas GPS:

Latitud Norte:

| Grados |  | Minutos |  | Segundos |
| --- | --- | --- | --- | --- |
| ° |  | ‘ |  | “ |

Longitud Este:

| Grados |  | Minutos |  | Segundos |
| --- | --- | --- | --- | --- |
| ° |  | ‘ |  | “ |

SE DEBE TENER EN MENTE ANTES DE INICIAR LA ENCUESTA, QUE EL ENCUESTADO DEBE SER LA AMA DE CASA O EL RESPONSABLE A CARGO DEL HOGAR, EL CUAL TENGA EL CONOCIMIENTO PLENO DE TODAS LAS PREGUNTAS POSTERIORES.

- ¿Es usted mayor de 18 años? Si la respuesta es NO, agradecerle por su tiempo y terminar la encuesta, contactar con otra persona que sea mayor de 18 años.

| - Sí |  |  | - No |  |
| --- | --- | --- | --- | --- |

- ¿Está usted de acuerdo con participar en la encuesta? Si la respuesta es NO, agradecerle por su tiempo y terminar la encuesta, contactar con otra persona.

| - Sí |  |  | - No |  |
| --- | --- | --- | --- | --- |

INFORMACIÓN DE IDENTIFICACIÓN

- ¿Cuál es su número de DPI? Si no tiene número de DPI pregunte por su número de cédula.

|  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

DPI:

Cédula: _________________________________

- ¿Cuál es su nombre?_______________________________________________________

- Género. No realice la pregunta, dedúzcalo por cuenta propia.

| - Hombre |  |  | - Mujer |  |
| --- | --- | --- | --- | --- |

- ¿Es usted la persona que cocina en su casa?

| - Sí |  | Pase a pregunta 8 |  | - No |  | Pase a pregunta 5, 6, y7 |
| --- | --- | --- | --- | --- | --- | --- |

5. ¿Cuál es el nombre de la persona que cocina en su casa?________________________________

6. Género. No realice la pregunta, dedúzcalo por el nombre.

| - Hombre |  |  | - Mujer |  |
| --- | --- | --- | --- | --- |

7. ¿Por qué no pude hablar la cocinera de la casa?

| - No está en casa |  |
| --- | --- |
| - No está disponible para realizar la entrevista |  |
| - No quiere participar en la entrevista |  |
| - Otro, especificar: |  |

8. ¿Podría proporcionarme un número de teléfono?

|  |  |  |  | - |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

9. ¿De quién es el número de teléfono?

| 1.Mío |  | 2.Mi Esposo/a |  |
| --- | --- | --- | --- |
| 3. Otro miembro del hogar |  | 4.Mi vecino |  |
| 5.Líder de la comunidad |  | 6.Líder del Grupo |  |

10. ¿Cuál es el tipo de localidad? No realice la pregunta a la persona que esta entrevistando, respóndala usted.

| 1.Rural |  |
| --- | --- |
| 2. Peri-urbana (cerca de la ciudad o pueblo) |  |
| 3.Urbano(ciudad o pueblo) |  |

INFORMACIÓN DEL HOGAR

Me gustaría hacerle algunas preguntas acerca de su familia y de la casa. Algunas de estas preguntas son un poco personales, pero ayudaran a que usted y su familia entienda más sobre los beneficios del programa Socorro Maya. Por favor conteste todas las preguntas honestamente y con la mayor exactitud posible.

- ¿Cuántas personas viven en su casa, incluyéndolo/incluyéndola a usted?

| 1.Una Personas |  | 6. Seis Personas |  |
| --- | --- | --- | --- |
| 2.Dos Personas |  | 7. Siete Personas |  |
| 3.Tres Personas |  | 8. Ocho Personas |  |
| 4.Cuatro Personas |  | 9.Nueve Personas |  |
| 5.Cinco Personas |  | 10.Especifique número |  |

- ¿Cuántas de las personas que viven en su casa son menores de 5 años?

| 1.Una Personas |  | 6. Seis Personas |  |
| --- | --- | --- | --- |
| 2.Dos Personas |  | 7. Siete Personas |  |
| 3.Tres Personas |  | 8. Ocho Personas |  |
| 4.Cuatro Personas |  | 9.Nueve Personas |  |
| 5.Cinco Personas |  | 10.Especifique número |  |

- ¿Cuántas de las personas que viven en su casa son entre 5 y 15 años o más?

| 1.Una Personas |  | 6. Seis Personas |  |
| --- | --- | --- | --- |
| 2.Dos Personas |  | 7. Siete Personas |  |
| 3.Tres Personas |  | 8. Ocho Personas |  |
| 4.Cuatro Personas |  | 9.Nueve Personas |  |
| 5.Cinco Personas |  | 10.Especifique número |  |

- ¿Cuántas de las personas que viven en su casa son mujeres de 15 años o más?

| 1.Una Personas |  | 6. Seis Personas |  |
| --- | --- | --- | --- |
| 2.Dos Personas |  | 7. Siete Personas |  |
| 3.Tres Personas |  | 8. Ocho Personas |  |
| 4.Cuatro Personas |  | 9.Nueve Personas |  |
| 5.Cinco Personas |  | 10.Especifique número |  |

- ¿Cuántas de las personas que viven en su casa son hombres entre 15 y 59 años?

| 1.Una Personas |  | 6. Seis Personas |  |
| --- | --- | --- | --- |
| 2.Dos Personas |  | 7. Siete Personas |  |
| 3.Tres Personas |  | 8. Ocho Personas |  |
| 4.Cuatro Personas |  | 9.Nueve Personas |  |
| 5.Cinco Personas |  | 10.Especifique número |  |

- ¿Cuántas de las personas que viven en su casa son hombres mayores de 59 años?

| 1.Una Personas |  | 6. Seis Personas |  |
| --- | --- | --- | --- |
| 2.Dos Personas |  | 7. Siete Personas |  |
| 3.Tres Personas |  | 8. Ocho Personas |  |
| 4.Cuatro Personas |  | 9.Nueve Personas |  |
| 5.Cinco Personas |  | 10.Especifique número |  |

- ¿Se inscribieron para el presente año escolar en un plantel educativo todos niños del hogar de edades entre 7 y 13?

| 1. Sí |  |  | - No |  |  | - No están entre 7 y 13 años |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

- ¿Sabe la jefa/esposa del hogar leer y escribir?

| 1. Sí |  |  | 2. No |  |  | - No hay jefa/esposa del hogar |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

- ¿Trabaja algún miembro del hogar principalmente como jornalero o empleado doméstico?

| - Sí |  |  | - No |  |
| --- | --- | --- | --- | --- |

- ¿Cuál es la principal fuente de ingresos en su hogar?

| - Agricultura / Ganadería |  | Pasar a pregunta 10 |
| --- | --- | --- |
| - Pequeña empresa (propia) |  | Pasar a pregunta 11 |
| - Empleo (asalariado: profesor, policía, trabajador del gobierno…) |  | Pasar a pregunta 11 |
| - Empleo temporal (informal) |  | Pasar a pregunta 11 |
| - Remesas. |  | Pasar a pregunta 11 |

- Si algún miembro del hogar se dedica en su actividad principal a la agricultura, la ganadería, la caza o la pesca, ¿Tiene vacas, toros, terneros, cerdos, caballos, burros o mulas?

| 1. Sí |  |  | 2. No |  |  | - Trabaja en agricultura |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

- ¿Cuáles el material predominante de los pisos de la vivienda? Evitar preguntar, en caso se tenga duda realizar la pregunta.

| - Tierra, madera y otro |  |
| --- | --- |
| - Ladrillos de barro o torta de cemento |  |
| - Ladrillo de cemento |  |
| - Parque, ladrillo cerámico o granito |  |
| - Otro, especificar: |  |

- ¿Cuál es la principal fuente de iluminación en el hogar?

| - Quema de leña |  |
| --- | --- |
| - Lámpara de gas |  |
| - Velas |  |
| - Linterna |  |
| - Electricidad |  |
| - Lámpara que funciona con energía solar |  |
| - Generador |  |

- ¿Cómo carga su teléfono celular?

| - Electricidad en mi hogar |  |
| --- | --- |
| - Electricidad en el hogar de un vecino |  |
| - En una tienda |  |
| - Una cargador de energía solar |  |
| - No tengo |  |
| - Otro, especificar: |  |

- ¿Tiene el hogar un refrigerador?

| - Sí |  |  | - No |  |
| --- | --- | --- | --- | --- |

- ¿Tiene el hogar una estufa a gas o eléctrica?

| - Sí |  |  | - No |  |
| --- | --- | --- | --- | --- |

- ¿Tiene el hogar una piedra de moler?

| - Sí |  |  | - No |  |
| --- | --- | --- | --- | --- |

- ¿Tiene el hogar una plancha eléctrica para planchar ropa?

| - Sí |  |  | - No |  |
| --- | --- | --- | --- | --- |

- Tomando en cuenta toda las cosas en su conjunto, Ud. diría que es:

| - Muy feliz |  |
| --- | --- |
| - Bastante feliz |  |
| - No muy feliz |  |
| - Para nada feliz |  |

19. En general ¿Cuál satisfecho o insatisfecho esta Ud. con su: (Elegir una respuesta para cada categoría)?

| - Vida en general | 1.Muy satisfecho | 2.Bastante satisfecho | 3.No muy satisfecho | 4.Para nada satisfecho | 99. No sabe/no responde |
| --- | --- | --- | --- | --- | --- |
| - Alimentación | 1.Muy satisfecho | 2.Bastante satisfecho | 3.No muy satisfecho | 4.Para nada satisfecho | 99. No sabe/no responde |
| - Vivienda | 1.Muy satisfecho | 2.Bastante satisfecho | 3.No muy satisfecho | 4.Para nada satisfecho | 99. No sabe/no responde |
| - Ingreso | 1.Muy satisfecho | 2.Bastante satisfecho | 3.No muy satisfecho | 4.Para nada satisfecho | 99. No sabe/no responde |
| - Salud | 1.Muy satisfecho | 2.Bastante satisfecho | 3.No muy satisfecho | 4.Para nada satisfecho | 99. No sabe/no responde |
| - Trabajo | 1.Muy satisfecho | 2.Bastante satisfecho | 3.No muy satisfecho | 4.Para nada satisfecho | 99. No sabe/no responde |
| - Nivel de Seguridad Local | 1.Muy satisfecho | 2.Bastante satisfecho | 3.No muy satisfecho | 4.Para nada satisfecho | 99. No sabe/no responde |
| - Amigos | 1.Muy satisfecho | 2.Bastante satisfecho | 3.No muy satisfecho | 4.Para nada satisfecho | 99. No sabe/no responde |
| - Familia | 1.Muy satisfecho | 2.Bastante satisfecho | 3.No muy satisfecho | 4.Para nada satisfecho | 99. No sabe/no responde |
| - Educación | 1.Muy satisfecho | 2.Bastante satisfecho | 3.No muy satisfecho | 4.Para nada satisfecho | 99. No sabe/no responde |
| - Libertad de elegir y control sobre su vida | 1.Muy satisfecho | 2.Bastante satisfecho | 3.No muy satisfecho | 4.Para nada satisfecho | 99. No sabe/no responde |
| - Dignidad | 1.Muy satisfecho | 2.Bastante satisfecho | 3.No muy satisfecho | 4.Para nada satisfecho | 99. No sabe/no responde |
| - Barrio/ comunidad/ Pueblo | 1.Muy satisfecho | 2.Bastante satisfecho | 3.No muy satisfecho | 4.Para nada satisfecho | 99. No sabe/no responde |
| - Capacidad de ayudar a los demás | 1.Muy satisfecho | 2.Bastante satisfecho | 3.No muy satisfecho | 4.Para nada satisfecho | 99. No sabe/no responde |
| - Creencias espirituales, religiosas o filosóficas | 1.Muy satisfecho | 2.Bastante satisfecho | 3.No muy satisfecho | 4.Para nada satisfecho | 99. No sabe/no responde |

INFORMACIÓN DE LA COCINA

- ¿En cuáles meses es la época seca o caliente? (Elegir todos los que apliquen)

| - Enero |  |
| --- | --- |
| - Febrero |  |
| - Marzo |  |
| - Abril |  |
| - Mayo |  |
| - Junio |  |
| - Julio |  |
| - Augusto |  |
| - Septiembre |  |
| - Octubre |  |
| - Noviembre |  |
| - Diciembre |  |

- ¿En cuáles meses es la época de lluvia? (Elegir todos los que apliquen)

| - Enero |  |
| --- | --- |
| - Febrero |  |
| - Marzo |  |
| - Abril |  |
| - Mayo |  |
| - Junio |  |
| - Julio |  |
| - Augusto |  |
| - Septiembre |  |
| - Octubre |  |
| - Noviembre |  |
| - Diciembre |  |

- ¿Para qué tipo de personas cocina usted? (Elegir todos los que apliquen)

| - Doméstica |  | Pase a pregunta 4 |
| --- | --- | --- |
| - Comercial |  | Pase a pregunta 9 |

- ¿Para cuantas personas cocina Usted en la época seca? (Cocina Domestica)

| 1.Una Personas |  | 6. Seis Personas |  |
| --- | --- | --- | --- |
| 2.Dos Personas |  | 7. Siete Personas |  |
| 3.Tres Personas |  | 8. Ocho Personas |  |
| 4.Cuatro Personas |  | 9.Nueve Personas |  |
| 5.Cinco Personas |  | 10.Especifique número |  |

- ¿Para cuantas personas cocina Usted en la época de lluvia? (Cocina Doméstica)

| - Una Personas |  | 6. Seis Personas |  |
| --- | --- | --- | --- |
| - Dos Personas |  | 7. Siete Personas |  |
| - Tres Personas |  | 8. Ocho Personas |  |
| - Cuatro Personas |  | - Nueve Personas |  |
| - Cinco Personas |  | - Especifique número |  |

- ¿En época de zafra o cosecha de café varia el número de personas para las que cocina? (Cocina Domestica)

| - Sí |  | Pase a Pregunta 7 y 8 |
| --- | --- | --- |
| - No |  | Pase a Pregunta |

- ¿Para cuantas personas cocina en la época de zafra o cosecha de café? (Cocina Domestica)

| 1.Una Personas |  | 6.Seis Personas |  |
| --- | --- | --- | --- |
| 2.Dos Personas |  | 7.Siete Personas |  |
| 3.Tres Personas |  | 8.Ocho Personas |  |
| 4.Cuatro Personas |  | 9.Nueve Personas |  |
| 5.Cinco Personas |  | 10.Especifique número |  |

- ¿En cuales meses es período de zafra o cosecha de café? (Elegir todos los que apliquen)

| 1.Enero |  |
| --- | --- |
| 2.Febrero |  |
| 3.Marzo |  |
| 4.Abril |  |
| 5.Mayo |  |
| 6.Junio |  |
| 7.Julio |  |
| 8.Augusto |  |
| 9.Septiembre |  |
| 10.Octubre |  |
| 11.Noviembre |  |
| 12.Diciembre |  |

- ¿La comida qué cocina para vender, para cuantas personas cocina en época seca? (Cocina Comercial)

| 1.Una Personas |  | 6.Seis Personas |  |
| --- | --- | --- | --- |
| 2.Dos Personas |  | 7.Siete Personas |  |
| 3.Tres Personas |  | 8.Ocho Personas |  |
| 4.Cuatro Personas |  | 9.Nueve Personas |  |
| 5.Cinco Personas |  | 10.Especifique número |  |

- ¿La comida qué cocina para vender, para cuantas personas cocina en época de lluvia?  (Cocina Comercial)

| 1.Una Personas |  | 6.Seis Personas |  |
| --- | --- | --- | --- |
| 2.Dos Personas |  | 7.Siete Personas |  |
| 3.Tres Personas |  | 8.Ocho Personas |  |
| 4.Cuatro Personas |  | 9.Nueve Personas |  |
| 5.Cinco Personas |  | 10.Especifique número |  |

- ¿Qué tiempos de comida cocina típicamente en época seca? (Elegir todos los que apliquen)

| 1.Desayuno |  | 4.Refacción en la mañana |  |
| --- | --- | --- | --- |
| 2.Almuerzo |  | 5.Refacción en la tarde |  |
| 3.Cena |  | 6.Otro |  |

- ¿Qué tiempos de comida cocina típicamente en época de lluvia? (Elegir todos los que apliquen)

| 1.Desayuno |  | 4.Refacción en la mañana |  |
| --- | --- | --- | --- |
| 2.Almuerzo |  | 5.Refacción en la tarde |  |
| 3.Cena |  | 6.Otro |  |

- ¿Cuál de las siguientes Fuentes de energía usas para cocinar en época seca? (Elegir todos los que apliquen)

| 1.Leña |  | 5.Cartón |  |
| --- | --- | --- | --- |
| 2.Carbón |  | 6.Desechos agrícolas |  |
| 3.Gas Propano |  | 7.Desechos orgánicos |  |
| 4.Biogas |  | 8.Otros |  |

- ¿Cuál de las siguientes Fuentes de energía usas para cocinar en época de lluvia? (Elegir todos los que apliquen)

| 1.Leña |  | 5.Cartón |  |
| --- | --- | --- | --- |
| 2.Carbón |  | 6.Desechos agrícolas |  |
| 3.Gas Propano |  | 7.Desechos orgánicos |  |
| 4.Biogas |  | 8.Otros |  |

- ¿Cómo obtienen este combustible en la casa?

| 1.Comprar |  |
| --- | --- |
| 2.Colectar |  |

- ¿Cuánto combustible compra/colecta cada viaje/compra?

| 1.Media Tarea |  | 4.Dos Tareas |  |
| --- | --- | --- | --- |
| 2.Una Tarea |  | 5.Tres Tareas |  |
| 3.Una Tarea y media |  | 6.Especifique número |  |

- ¿Con que frecuencia compra o colecta este combustible?

| 1.Una vez a la semana |  | 4.Una vez al mes |  |  |
| --- | --- | --- | --- | --- |
| 2.Dos Veces a la semana |  | 5.Tres veces a mes |  |  |
| 3.Una vez cada 15 días |  | 6.Especifique número |  |  |

- ¿Cuánto se gasta en cada viaje/compra?

| 1.Sí colecta, cuántas horas camina? |  |
| --- | --- |
| 2.Sí compra, cuánto gasta? (Q.) |  |

- ¿Qué tipo de estufa utiliza en la época seca? (Elegir todos los que aplican)

| - Fuego abierto (tenamaste, tetunte) |  |  |
| --- | --- | --- |
| - Fuego abierto en alto (poyo) |  |  |
| - Estufa mejorada (Lorena, Plancha) |  |  |
| - Estufa de gas |  |  |
| - Estufa de queroseno |  |  |
| - Churrasquera |  |  |
| - Biogás |  |  |
| - Electricidad |  |  |
| - Otro, especificar: |  |  |

- ¿Qué tipo de estufa utiliza en la estación lluviosa? (Elegir todos los que aplican)

| - Fuego abierto (tenamaste, tetunte) |  |  |
| --- | --- | --- |
| - Fuego abierto en alto (poyo) |  |  |
| - Estufa mejorada (Lorena, Plancha) |  |  |
| - Estufa de gas |  |  |
| - Estufa de queroseno |  |  |
| - Churrasquera |  |  |
| - Biogás |  |  |
| - Electricidad |  |  |
| - Otro, especificar: |  |  |

Nota: Saque una foto de cada estufa en el hogar

- ¿Qué tipo de estufa utiliza la mayoría del días (tiempo) para todos los propósitos del hogar? (Elegir únicamente una opción)

| - Fuego abierto (tenamaste, tetunte) |  |  |
| --- | --- | --- |
| - Fuego abierto en alto (poyo) |  |  |
| - Estufa mejorada (Lorena, Plancha) |  |  |
| - Estufa de gas |  |  |
| - Estufa de queroseno |  |  |
| - Churrasquera |  |  |
| - Biogás |  |  |
| - Electricidad |  |  |
| - Otro, especificar: |  |  |

- ¿Utiliza su estufa para calentar su hogar (no para cocinar)?

| 1.Sí |  | Pasar a pregunta 23 | 2.No |  | Pasar a pregunta 28 |
| --- | --- | --- | --- | --- | --- |

- ¿Qué tipo de estufa utiliza para calentar su hogar? (Elegir todos los que apliquen)

| - Fuego abierto (tenamaste, tetunte) |  |  |
| --- | --- | --- |
| - Fuego abierto en alto (poyo) |  |  |
| - Estufa mejorada (Lorena, Plancha) |  |  |
| - Estufa de gas |  |  |
| - Estufa de queroseno |  |  |
| - Churrasquera |  |  |
| - Biogás |  |  |
| - Electricidad |  |  |
| - Otro, especificar: |  |  |

- ¿Cuáles meses utiliza su estufa para calentar su hogar? (Elegir todos los que apliquen)

| - Enero |  |
| --- | --- |
| - Febrero |  |
| - Marzo |  |
| - Abril |  |
| - Mayo |  |
| - Junio |  |
| - Julio |  |
| - Augusto |  |
| - Septiembre |  |
| - Octubre |  |
| - Noviembre |  |
| - Diciembre |  |

- ¿Para los meses seccionados en una semana típica cuantas horas usa su estufa para calentar su hogar?

Horas

- ¿Cómo calienta su hogar en relación a sus otras prácticas de cocinar?

| - Deja el fuego después de cocinar |  |  |
| --- | --- | --- |
| - Empieza la estufa para calentar el hogar específicamente |  |  |
| - Otro, especificar: |  |  |

- ¿Qué cantidad de combustible utiliza cada vez que se utiliza su estufa para calentar el hogar?

| - Más que uso cuando cocina una comida |  |
| --- | --- |
| - La misma que uso cuando cocina una comida |  |
| - La mitad de la cantidad que uso cuando concina una comida |  |
| - Mucho menos que lo que uso cuando cocina una comida |  |
| - No combustible adicional (dejo el fuego después de cocinar) |  |

- ¿Se utiliza su estufa para calentar agua para bañarse (no para cocinar)?

| 1.Sí |  | Pasar a pregunta 29 | 2.No |  | Concluye Encuestas |
| --- | --- | --- | --- | --- | --- |

- ¿Qué tipo de estufa utiliza para calentar agua para bañarse? (Elegir todos los que apliquen)

| - Fuego abierto (tenamaste, tetunte) |  |  |
| --- | --- | --- |
| - Fuego abierto en alto (poyo) |  |  |
| - Estufa mejorada (Lorena, Plancha) |  |  |
| - Estufa de gas |  |  |
| - Estufa de queroseno |  |  |
| - Churrasquera |  |  |
| - Biogás |  |  |
| - Electricidad |  |  |
| - Otro, especificar: |  |  |

- ¿Cuáles meses utiliza su estufa para calentar agua para bañarse?

| 1.Enero |  |
| --- | --- |
| 2.Febrero |  |
| 3.Marzo |  |
| 4.Abril |  |
| 5.Mayo |  |
| 6.Junio |  |
| 7.Julio |  |
| 8.Augusto |  |
| 9.Septiembre |  |
| 10.Octubre |  |
| 11.Noviembre |  |
| 12.Diciembre |  |

- ¿Para los meses seccionados en una semana típica cuantas horas usa su estufa para calentar agua para bañarse?

Horas

- ¿Cómo calienta agua para bañarse en relación a sus otras prácticas de cocinar?

| - Deja el fuego después de cocinar |  |
| --- | --- |
| - Empieza la estufa para calentar el hogar específicamente |  |
| - Otro, especificar: |  |

- ¿Qué cantidad de combustible utiliza cada vez que se utiliza su estufa para calentar agua para bañarse?

| - Más que uso cuando cocina una comida |  |
| --- | --- |
| - La misma que uso cuando cocina una comida |  |
| - La mitad de la cantidad que uso cuando concina una comida |  |
| - Mucho menos que lo que uso cuando cocina una comida |  |
| - No combustible adicional (dejo el fuego después de cocinar) |  |

Despedirse de la persona encuestada y agradecer por su tiempo y apoyo.

Nombre del encuestador: _________________________________________________

Código del Encuestador: ______________________________________________________

Nombre del colaborador local:_________________________________________________

Código del colaborador local:__________________________________________________

Doy fe que la información contenida es verídica según las respuestas proporcionadas porla persona encuestada:

Firma Encuestador:_______________________ DPI No. ________________________________

Firma Colaborador Local: __________________ DPI No. ________________________________

Observaciones: ____________________________________________________________

________________________________________________________________________________

Reference Xcel Document: KPT Paired Data (San Marcos) GS1321 Iss 2 [DATE]

Reference Xcel Document: KPT Add. Baseline Data GS1321 Iss 2 [DATE]

Reference Xcel Document: KPT Paired Data (San Marcos) GS1321 Iss 2 [DATE]