Monitoring Report

###### PUBLICATION DATE  14.10.2020
VERSION  v. 1.1 
RELATED SUPPORT - TEMPLATE GUIDE Monitoring Report v. 1.1

This document contains the following Sections

Key Project Information

0 - Description of project

0 - Implementation of project

0 - Description of monitoring system applied by the project

0 - Data and parameters

0 - Calculation of SDG Impacts

0 - Safeguards Reporting

0 - Stakeholder inputs and legal disputes

### KEY PROJECT INFORMATION

##### Key Project Information

| GS ID (s) of Project (s) | GS 3958 |
| --- | --- |
| Title of the project (s) covered by monitoring report | Vaayu India Wind Power Project in Gujarat |
| Version number of the PDD/VPA-DD (s) applicable to this monitoring report | - |
| Version number of the monitoring report | 4.1 |
| Completion date of the monitoring report | 04/04/2022 |
| Date of project design certification | 31/07/2015 (GS Registration Date) |
| Date of Last Annual Report | NA |
| Monitoring period number | Third |
| Duration of this monitoring period | 01/04/2018 to 31/12/2019 including both start and end date of monitoring period |
| Project Representative | Vaayu (India) Power Corporation Private Limited (India) |
| Host Country | India |
| Activity Requirements applied | Community Services Activities Renewable Energy Activities Land Use and Forestry Activities/Risks & Capacities N/A |
| Methodology (ies) applied and version number | “Consolidated baseline methodology for grid connected electricity generation from renewable sources” ACM0002 (Version 11, EB 52) |
| Product Requirements applied | GHG Emissions Reduction & Sequestration Renewable Energy Label N/A |

##### Table 1 - Sustainable Development Contributions Achieved

| Sustainable Development Goals Targeted | SDG Impact | Amount Achieved | Units/ Products |
| --- | --- | --- | --- |
| SDG 7 | Affordable and Clean Energy | 132,789.152 MWh | MWh |
| SDG 8 | Decent Work and Economic Growth | 2 training 31 employees | numbers |
| SDG 13 | Climate Action | 119,302 tCO2 | tCO2e |

##### Table 2 – Product Vintages

|  |  | Amount Achieved (tCO2e) |  |  |
| --- | --- | --- | --- | --- |
| Start Dates | End Dates | Baseline Emission | Project Emission | Leakage Emission |
| 01/04/2018 | 31/12/2018 | 26,788 | 0 | 0 |
| 01/01/2019 | 31/12/2019 | 92,514 | 0 | 0 |

For the delay in site visit, the emission reductions can only be claimed in the period of three years prior to onsite verification date 22/07/2021, i.e., from 22/07/2018. PP has foregone generation for the period of 22/07/2018 to 31/07/2018 to simplify calculation of emission reductions. The credits are claimed from 01/08/2018 to 31/12/2019.

#### SECTION A. DESCRIPTION OF PROJECT

##### A.1. General description of project

>>

The project activity includes development, design, engineering, procurement, finance, construction, operation and maintenance of Vaayu 51.2 MW wind power project (“Project”) in the Indian state of Gujarat to provide reliable, renewable power to the Gujarat state electricity grid which is part of the NEWNE regional electricity grid. The Project leads to reduce greenhouse gas emissions because it displaces electricity from grid connected fossil fuel based electricity generation plants. The Project involves 64 wind energy converters (WECs) of 800 kW E-53 with internal electrical lines connecting the Project with local evacuation facility.

The first WEC under the project activity was commissioned on 25 June 2010 and the last WEC under the project activity was commissioned on 4 July 2011. The expected operational lifetime of the project is for 20 years. The length of the Crediting period of the project activity as per registered PDD is 10 years (Fixed). The seventh monitoring period considered is from 01 April 2018 to 31 Dec 2019. The details of issuance of CERs for the previous monitoring periods are as follows:

| Monitoring Period No. | Monitoring Period | CER Issued |
| --- | --- | --- |
| First Issuance | 01 Jun 2011 - 29 Feb 2012 | 61,182 |
| Second issuance | 01 Mar 2012 - 31 Dec 2012 | 87,690 |
| Third issuance | 01 Jan 2013 - 31 Aug 2013 | 68,389 |
| Fourth Issuance | 01 Sep 2013 - 31 Dec 2014 | 109,896 |
| Fifth Issuance | 01 Jan 2015 - 31 Mar 2017 | 192,213 |
| Sixth Issuance | 01 Apr 2017 - 31 Mar 2018 | 75,186 |

The total emission reductions achieved during the current monitoring period is 119,302 tCO2.

##### A.2. Location of project

>>

(a) Host Party (ies);

India

(b) Region/ State/ Province, etc.;

Gujarat State in India

(c) City/ Town/ Community, etc.;

The Project is connected to Sadodar substation at Jamnagar District in Gujarat. The Project is spread across villages Chattar, Narmana, Seth Wadala, Jam Ambardi, Mevasa, Dhun Dhoraji, Sadodar, Bodi, Padavala and Machharda in Jamnagar and Rajkot Districts of Gujarat state in India

(d) Physical/ Geographical location

The project area extends between latitude 21º 55’ and 22º 08’ North and longitude 70º 05’ and 70º 19’ East. The location numbers & latitude & longitude details of the Wind Energy Generators have been mentioned in the Appendix 1.

| Sr. No. | Location No. | WTG-ID No. | Village | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- |
| 1 | 3020 | EIL/800/10-11/1826 | Machharda | N22 O 06’ 19.0” | E70 O 18’ 45.7” |
| 2 | 3021 | EIL/800/10-11/1827 | Machharda | N22 O 06’ 23.5” | E70 O 18’ 43.7” |
| 3 | 3022 | EIL/800/10-11/1828 | Machharda | N22 O 06’ 29.7” | E70 O 18’ 44.6” |
| 4 | 3072 | EIL/800/09-10/1738 | Padavala | N21 O 57’ 19.6” | E70 O 15’ 05.0” |
| 5 | 3073 | EIL/800/09-10/1739 | Padavala | N21 O 57’ 14.9” | E70 O 15’ 11.7” |
| 6 | 3075 | EIL/800/09-10/1740 | Padavala | N21 O 56’43.1” | E70 O 15’ 20.6” |
| 7 | 3076 | EIL/800/09-10/1741 | Padavala | N21 O 55’ 59.2” | E70 O 15’ 33.7” |
| 8 | 3088 | EIL/800/09-10/1742 | Padavala | N21 O 56’ 19.3” | E70 O 14’ 38.0” |
| 9 | 62 | EIL/800/09-10/1766 | Chattar | N22 O 07’ 40.2” | E70 O 15’ 10.7” |
| 10 | 63 | EIL/800/09-10/1767 | Chattar | N22 O 07’ 46.6” | E70 O 15’00.6” |
| 11 | 64 | EIL/800/09-10/1768 | Chattar | N22 O 07’ 53.3” | E70 O 14’ 57.1” |
| 12 | 539 | EIL/800/09-10/1789 | Seth Wadala | N22 O 04’ 46.7” | E70 O 05’ 34.3” |
| 13 | 540 | EIL/800/09-10/1790 | Seth Wadala | N22 O 04’ 33.3” | E70 O 05’ 43.1” |
| 14 | 541 | EIL/800/09-10/1791 | Seth Wadala | N22 O 04’ 27.4” | E70 O 05’ 47.6” |
| 15 | 543 | EIL/800/09-10/1792 | Seth Wadala | N22 O 04’ 17.3” | E70 O 05’ 53.7” |
| 16 | 544 | EIL/800/09-10/1793 | Seth Wadala | N22 O 04’ 13.5” | E70 O 06’ 00.7” |
| 17 | 545 | EIL/800/09-10/1794 | Seth Wadala | N22 O 03’ 31.5” | E70 O 05’ 32.6” |
| 18 | 546 | EIL/800/09-10/1795 | Jam Ambardi | N22 O 03’ 40.2” | E70 O 05’ 31.0” |
| 19 | 547 | EIL/800/09-10/1796 | Jam Ambardi | N22 O 03’ 45.3” | E70 O 05’ 31.9” |
| 20 | 548 | EIL/800/09-10/1797 | Jam Ambardi | N22 O 03’ 50.7” | E70 O 05’ 34.2” |
| 21 | 903 | EIL/800/09-10/1747 | Mevasa/Haripar | N22 O 01’ 23.0” | E70 O 15’ 35.2” |
| 22 | 904 | EIL/800/09-10/1748 | Mevasa/Haripar | N22 O 01’ 30.2” | E70 O 15’ 41.0” |
| 23 | 905 | EIL/800/09-10/1749 | Mevasa/Haripar | N22 O 01’ 36.6” | E70 O 15’ 27.2” |
| 24 | 906 | EIL/800/09-10/1750 | Mevasa/Haripar | N22 O 01’ 30.7” | E70 O 14’ 55.0” |
| 25 | 907 | EIL/800/09-10/1751 | Mevasa/ Haripar | N22 O 01’ 37.9” | E70 O 14’ 56.8” |
| 26 | 908 | EIL/800/09-10/1752 | Mevasa/ Haripar | N22 O 01’ 44.8” | E70 O 14’ 54.1” |
| 27 | 909 | EIL/800/09-10/1753 | Mevasa/ Haripar | N22 O 01’ 51.2” | E70 O 14’ 51.2” |
| 28 | 910 | EIL/800/09-10/1754 | Mevasa/ Haripar | N22 O 01’ 57.7” | E70 O 14’ 55.7” |
| 29 | 912 | EIL/800/09-10/1746 | Dhun Dhoraji | N22 O 02’ 09.1” | E70 O 15’ 04.4” |
| 30 | 926 | EIL/800/09-10/1769 | Chattar | N22 O 06’ 57.6” | E70 O 16’ 33.0” |
| 31 | 927 | EIL/800/09-10/1770 | Chattar | N22 O 06’ 59.3” | E70 O 16’ 23.3” |
| 32 | 928 | EIL/800/09-10/1771 | Chattar | N22 O 07’ 10.0” | E70 O 16’ 16.5” |
| 33 | 929 | EIL/800/09-10/1772 | Chattar | N22 O 07’ 15.9” | E70 O 16’ 11.3” |
| 34 | 931 | EIL/800/10-11/1870 | Chattar | N22 O 07’ 12.7” | E70 O15’ 23.5” |
| 35 | 932 | EIL/800/09-10/1773 | Chattar | N22 O 07’ 05.5” | E70 O 15’ 27.2” |
| 36 | 933 | EIL/800/09-10/1774 | Chattar | N22 O 06’ 59.3” | E70 O 15’ 31.5” |
| 37 | 934 | EIL/800/09-10/1775 | Chattar | N22 O 06’ 53.9” | E70 O 15’ 27.9” |
| 38 | 935 | EIL/800/09-10/1776 | Chattar | N22 O 06’ 46.0” | E70 O 15’ 22.7” |
| 39 | 936 | EIL/800/09-10/1777 | Chattar | N22 O 06’ 40.3” | E70 O 15’ 25.7” |
| 40 | 937 | EIL/800/09-10/1778 | Chattar | N22 O 06’ 32.0” | E70 O 15’ 23.4” |
| 41 | 938 | EIL/800/09-10/1779 | Chattar | N22 O 06’ 25.7” | E70 O 15’ 22.1” |
| 42 | 939 | EIL/800/09-10/1760 | Jamvadi | N22 O 08’ 19.5” | E70 O 19’ 02.3” |
| 43 | 941 | EIL/800/09-10/1761 | Jamvadi | N22 O 08’ 07.2” | E70 O 18’ 57.8” |
| 44 | 942 | EIL/800/09-10/1762 | Jamvadi | N22 O 08’ 08.6” | E70 O 19’ 30.2” |
| 45 | 943 | EIL/800/09-10/1763 | Jamvadi | N22 O 08’ 00.9” | E70 O 19’ 25.4” |
| 46 | 944 | EIL/800/09-10/1764 | Jamvadi | N22 O 07’ 53.9” | E70 O 19’ 26.0” |
| 47 | 945 | EIL/800/09-10/1765 | Jamvadi | N22 O 07’49.5” | E70 O 19’ 31.4” |
| 48 | 947 | EIL/800/09-10/1755 | Moti Vavdi | N22 O 06’ 04.0” | E70 O 18’ 16.9” |
| 49 | 948 | EIL/800/09-10/1756 | Moti Vavdi | N22 O 05’ 57.0” | E70 O 18’ 17.8” |
| 50 | 950 | EIL/800/09-10/1757 | Moti Vavdi | N22 O 05’ 45.7” | E70 O 18’ 21.5” |
| 51 | 951 | EIL/800/09-10/1758 | Moti Vavdi | N22 O 05’ 38.3” | E70 O 18’ 18.4” |
| 52 | 952 | EIL/800/09-10/1759 | Moti Vavdi | N22 O 05’ 31.6” | E70 O 18’ 16.9” |
| 53 | 958 | EIL/800/09-10/1743 | Dhun Dhoraji | N22 O 02’ 32.4” | E70 O 16’ 42.8” |
| 54 | 959 | EIL/800/09-10/1744 | Dhun Dhoraji | N22 O 02’ 26.2” | E70 O 16’ 44.6” |
| 55 | 960 | EIL/800/09-10/1745 | Dhun Dhoraji | N22 O 02’ 19.0” | E70 O 16’ 44.4” |
| 56 | 992 | EIL/800/09-10/1782 | Sadodar | N22 O 03’ 13.6” | E70 O 10’ 37.3” |
| 57 | 993 | EIL/800/09-10/1783 | Sadodar | N22 O 03’ 09.5” | E70 O 10’ 40.0” |
| 58 | 994 | EIL/800/09-10/1784 | Sadodar | N22 O 02’ 59.6” | E70 O 10’ 36.4” |
| 59 | 995 | EIL/800/09-10/1785 | Sadodar | N22 O 02’ 54.2” | E70 O 10’ 33.5” |
| 60 | 996 | EIL/800/09-10/1786 | Sadodar | N22 O 02’ 47.4” | E70 O 10’ 22.2” |
| 61 | 997 | EIL/800/09-10/1787 | Sadodar | N22 O 02’ 41.3” | E70 O 10’ 32.4” |
| 62 | 1028 | EIL/800/09-10/1788 | Seth Wadala | N22 O 03’ 06.0” | E70 O 08’ 36.9” |
| 63 | 1045 | EIL/800/09-10/1780 | Bodi | N22 O 08’ 43.4” | E70 O 15’ 11.4” |
| 64 | 1046 | EIL/800/09-10/1781 | Bodi | N22 O 08’ 48.8” | E70 O 15’ 08.5” |

##### A.3. Reference of applied methodology

>>

Title: “Consolidated baseline methodology for grid-connected electricity generation from renewable sources”

Reference: Approved consolidated baseline methodology ACM0002 (Version 11, EB 52)

ACM0002 draws upon the following tools which have been used in the PDD:

• 	Tool to calculate the emission factor for an electricity system – Version 02

• 	Tool for the demonstration and assessment of additionality – Version 5.2

Further information with regards to the methodology/ tools can be obtained at

https://cdm.unfccc.int/methodologies/DB/VJI9AX539D9MLOPXN2AY9UR1N4IYGD

##### A.4. Crediting period of project

>>

Type of crediting period         : 	Fixed

Start date of crediting period	: 06/01/2011

End date of crediting period	: 06/01/2021

#### SECTION B. IMPLEMENTATION OF PROJECT

##### B.1. Description of implemented project

>>

The starting date of operation of the project activity is 21/05/2011. The commissioning date for all the WECs included in the project activity is given in the table below:

| Sr. No. | Location No. | WTG-ID No. | Commissioning Date |
| --- | --- | --- | --- |
| 1 | 3020 | EIL/800/10-11/1826 | 12/07/2010 |
| 2 | 3021 | EIL/800/10-11/1827 | 12/07/2010 |
| 3 | 3022 | EIL/800/10-11/1828 | 12/07/2010 |
| 4 | 3072 | EIL/800/09-10/1738 | 25/06/2010 |
| 5 | 3073 | EIL/800/09-10/1739 | 25/06/2010 |
| 6 | 3075 | EIL/800/09-10/1740 | 25/06/2010 |
| 7 | 3076 | EIL/800/09-10/1741 | 25/06/2010 |
| 8 | 3088 | EIL/800/09-10/1742 | 25/06/2010 |
| 9 | 62 | EIL/800/09-10/1766 | 27/06/2011 |
| 10 | 63 | EIL/800/09-10/1767 | 04/07/2011 |
| 11 | 64 | EIL/800/09-10/1768 | 04/07/2011 |
| 12 | 539 | EIL/800/09-10/1789 | 14/02/2011 |
| 13 | 540 | EIL/800/09-10/1790 | 14/02/2011 |
| 14 | 541 | EIL/800/09-10/1791 | 14/02/2011 |
| 15 | 543 | EIL/800/09-10/1792 | 18/02/2011 |
| 16 | 544 | EIL/800/09-10/1793 | 14/02/2011 |
| 17 | 545 | EIL/800/09-10/1794 | 18/02/2011 |
| 18 | 546 | EIL/800/09-10/1795 | 18/03/2011 |
| 19 | 547 | EIL/800/09-10/1796 | 18/02/2011 |
| 20 | 548 | EIL/800/09-10/1797 | 18/02/2011 |
| 21 | 903 | EIL/800/09-10/1747 | 04/05/2011 |
| 22 | 904 | EIL/800/09-10/1748 | 04/05/2011 |
| 23 | 905 | EIL/800/09-10/1749 | 04/05/2011 |
| 24 | 906 | EIL/800/09-10/1750 | 05/03/2011 |
| 25 | 907 | EIL/800/09-10/1751 | 05/03/2011 |
| 26 | 908 | EIL/800/09-10/1752 | 05/03/2011 |
| 27 | 909 | EIL/800/09-10/1753 | 05/03/2011 |
| 28 | 910 | EIL/800/09-10/1754 | 05/03/2011 |
| 29 | 912 | EIL/800/09-10/1746 | 14/02/2011 |
| 30 | 926 | EIL/800/09-10/1769 | 10/06/2011 |
| 31 | 927 | EIL/800/09-10/1770 | 10/06/2011 |
| 32 | 928 | EIL/800/09-10/1771 | 10/06/2011 |
| 33 | 929 | EIL/800/09-10/1772 | 10/06/2011 |
| 34 | 931 | EIL/800/10-11/1870 | 10/06/2011 |
| 35 | 932 | EIL/800/09-10/1773 | 10/06/2011 |
| 36 | 933 | EIL/800/09-10/1774 | 10/06/2011 |
| 37 | 934 | EIL/800/09-10/1775 | 10/06/2011 |
| 38 | 935 | EIL/800/09-10/1776 | 10/06/2011 |
| 39 | 936 | EIL/800/09-10/1777 | 27/06/2011 |
| 40 | 937 | EIL/800/09-10/1778 | 27/06/2011 |
| 41 | 938 | EIL/800/09-10/1779 | 27/06/2011 |
| 42 | 939 | EIL/800/09-10/1760 | 24/05/2011 |
| 43 | 941 | EIL/800/09-10/1761 | 24/05/2011 |
| 44 | 942 | EIL/800/09-10/1762 | 24/05/2011 |
| 45 | 943 | EIL/800/09-10/1763 | 24/05/2011 |
| 46 | 944 | EIL/800/09-10/1764 | 24/05/2011 |
| 47 | 945 | EIL/800/09-10/1765 | 24/05/2011 |
| 48 | 947 | EIL/800/09-10/1755 | 06/05/2011 |
| 49 | 948 | EIL/800/09-10/1756 | 06/05/2011 |
| 50 | 950 | EIL/800/09-10/1757 | 06/05/2011 |
| 51 | 951 | EIL/800/09-10/1758 | 06/05/2011 |
| 52 | 952 | EIL/800/09-10/1759 | 06/05/2011 |
| 53 | 958 | EIL/800/09-10/1743 | 04/05/2011 |
| 54 | 959 | EIL/800/09-10/1744 | 04/05/2011 |
| 55 | 960 | EIL/800/09-10/1745 | 04/05/2011 |
| 56 | 992 | EIL/800/09-10/1782 | 18/03/2011 |
| 57 | 993 | EIL/800/09-10/1783 | 18/03/2011 |
| 58 | 994 | EIL/800/09-10/1784 | 18/03/2011 |
| 59 | 995 | EIL/800/09-10/1785 | 18/03/2011 |
| 60 | 996 | EIL/800/09-10/1786 | 18/03/2011 |
| 61 | 997 | EIL/800/09-10/1787 | 18/03/2011 |
| 62 | 1028 | EIL/800/09-10/1788 | 04/05/2011 |
| 63 | 1045 | EIL/800/09-10/1780 | 04/07/2011 |
| 64 | 1046 | EIL/800/09-10/1781 | 04/07/2011 |

The project activity involves 64 numbers wind energy converters (WECs) of 800 kW, E-53 with internal  electrical  lines  connecting  the  project  activity  with  local  evacuation  facility.  The WECs generate 3-phase power at 400V, which is stepped up to 33 kV. The E-53 WECs can operate in the frequency range of 46–54 Hz.  As per the specification of WEC the output voltage of WEC is 400V as specified by manufacture. The average life time of the WEC is around 20 years as per the equipment supplier specifications. The technology employed is environmentally safe and sound since project activity doesn’t uses fossil fuel for electricity generation though project activity uses wind as source of energy and there is no project emission or leakage into the environment. Wind World (India) Limited wind turbines are equipped with state-of-the-art microelectronic control technology produced in-house at our Daman plant. The other salient features of the state-of-art-technology are mentioned below:

E-53 Specifications

| Parameter | Value |
| --- | --- |
| Turbine model | E- 53 |
| Rated power | 800 kW |
| Rotor diameter | 53 m |
| Hub height | 75 m |
| Turbine Type | Gearless horizontal axis wind turbine with variable rotor speed |
| Power regulation | Independent electromechanical pitch system for each blade |
| Cut in wind speed | 2.5 m/s |
| Rated wind speed | 12 m/s |
| Cut out Wind speed | 28-34 m/s |
| Extreme Wind Speed | 59.5 m/s |
| Rated rotational speed | 32 rpm |
| Operating range rot. speed | 12-29 rpm |
| Orientation | Upwind |
| No of Blades | 3 |
| Blade Material | Fibre Glass Epoxy reinforced with integral lightning protection |
| Gear box type | Gear less |
| Generator type | Synchronous generator |
| Braking | Aerodynamic |
| Output Voltage | 400 V |
| Yaw System | Active yawing with 4 electric yaw drives with brake motor and friction bearing |
| Tower | 74 m concrete |

Wind World (India) Limited is responsible for operation and maintenance activities for this project. Wind World (India) Limited operation and maintenance activities are ISO 9001:2008 certified and all the events are recorded in the log book available at the project site. Referring to the data available it can be inferred that there have not been any major special events for any of the WECs that are included in the project activity. As a part of regular maintenance the WECs are stopped for mechanical, electrical, grease and visual maintenance.  Detailed maintenance procedures of the WECs are mentioned below:

Description of maintenance intervals:

There is a pre-defined maintenance schedule for annual maintenance for all the WECs at project site. There are four types of maintenance activity have been executed for all the WECs. During maintenance, WEC needs to stop for defined time period which are as follows:

| 1) | Visual maintenance | : Average 3 to 4 hr stoppage of WEC |
| --- | --- | --- |
| 2) | Grease maintenance | : Average 3 to 4 hr stoppage of WEC |
| 3) | Electrical maintenance | : Average 16 to 20 hr stoppage of WEC |
| 4) | Mechanical maintenance | : Average 16 to 20 hr stop of WEC |

Other than the above mentioned maintenance activity, WEC were generating electricity continuously without any technical fault. Hence, no break down has been noted during the monitoring period.

B.1.1 Forward Action Requests

>>

Not Applicable

##### B.2. Post-Design Certification changes

>>

B.2.1. Temporary deviations from the approved Monitoring & Reporting Plan, methodology or standardized baseline

>>

Not Applicable

B.2.2. Corrections

>>

Not Applicable

>>

There are no changes to the start date of the crediting period.

B.2.4. Permanent changes from the Design Certified monitoring plan, applied methodology or applied standardized baseline

>>

Previous:

- Permanent changes to the registered monitoring plan were approved by UNFCCC on 1 Aug 2013 (Reference: PRC- 4700 -001).

2)  A Post Registration Change (PRC) was made related to change in calibration frequency from once in three year to once in five year. The PRC request was made via registered PDD version 6, dated 09/04/2021 (Reference: PRC- 4700 -002).

B.2.5. Changes to project design of approved project

>>

Not Applicable

#### SECTION C. DESCRIPTION OF MONITORING SYSTEM APPLIED BY THE PROJECT

>>

Approved   monitoring   methodology   ACM0002   Version 11 Sectoral Scope: 1, “Consolidated baseline methodology for grid-connected electricity generation from renewable sources”, by CDM - Meth Panel is proposed to be used to monitor the emission reductions.

Wind World (India) Limited is the O&M contractor for the project activity. Wind World (India) Limited will be responsible for maintaining all the monitoring data on behalf of VIPCPL in respect of the project activity. Wind World (India) Limited has implemented the management structure for managing the monitored data.

The approved monitoring methodology requires monitoring of the following:

•   Electricity generation from the project activity; and

•   Operating margin emission factor and build margin emission factor of the grid, where ex post determination of grid emission factor has been chosen

Since the baseline methodology is based on ex ante determination of the baseline, the monitoring of operating margin emission factor and build margin emission factor is not required. Further, wind based electricity generation is not associated with any kind of leakages.

The project activity has seventeen clusters and each cluster has exclusive metering arrangement and the meter readings taken at these metering points have been provided by the representatives of Wind World (India) Limited to GEDA. Line diagram showing the relevant monitoring points is provided at end of this section.

Wind World (India) Limited substation at Sadodar has four main meter(s) also known as revenue meter which is connected to wind turbines installed by the project proponent and wind turbines installed by other project owners. Gujarat Electricity Development Authority (GEDA) apportion the net electricity supplied to the grid at the Wind World (India) Limited substation to all the project owners after adjusting transmission loss to the meter readings taken at dedicated cluster meters of different project owners. The meter reading is being taken jointly by the representatives of Wind World (India) Limited and GEDA/GETCO in the form of JMR. The electricity from Wind World (India) Limited’s substation has been finally supplied to the utility’s substation at Moti Paneli. The net electricity generated by the project owners is being provided by GETCO in the share certificate of electricity generated. The value of the net electricity generated by the project activity has been taken  directly  by  the  project  proponent  from  the  share  certificate  provided  by  GETCO  for calculation of emission reductions.

The allocation plan for the project activity is given below:-

EG GETCO, Export   = Electricity exported, as recorded by the main meter at Wind World (India) Limited substation

EGGETCO, Import   = Electricity imported, as recorded by the main meter at Wind World (India) Limited substation

EGCluster, Export    = Electricity exported by the project activity, as measured at Cluster Meter

EGCluster, Import     = Electricity imported by the project activity, as measured at Cluster Meter

EGCluster, WF, Export = Electricity exported by all the project owners connected to Wind World (India) Limited substation, as measured at Cluster Meter

EGCluster, WF, Import = Electricity imported by all the project owners connected to Wind World (India) Limited substation, as measured at Cluster Meter

EGPJ,export,y = Electricity exported by the project activity to the grid, calculated

EGPJ,import,y  = Electricity imported from the project activity to the grid, calculated

EGPJ,y = Net Electricity exported by the project activity to the grid, calculated

Electricity Exported to the Grid by the project activity

EGPJ,export,y  = EGGETCO, Export X EGCluster, Export/ EGCluster, WF, Export

Electricity Imported from the Grid by the project activity

EGPJ,Import,y  =  EGGETCO, Import X EGCluster, Import / EGCluster, WF, Import

Net Electricity Exported to the grid by the project activity

EGPJ,y = EGPJ,export,y - EGPJ,Import,y

The apportioning procedure for the project activity is done by GEDA (Gujarat Energy Development Agency) based on the meters that are connected to the cluster meter of various project owners connected  to substation  of Wind World (India) Limited based on meter reading  noted at Wind World  (India)  Limited  substation  connecting  all  the  machines  of the  project  activity  and  other project  developers.  The  meter  reading  at  cluster  meter  and  the  Wind  World  (India)  Limited substation is directly monitored and hence the apportioning of the electricity is based on the meter reading that are directly measured.

The operational and management structure implemented for data monitoring is as follows:

The reading is monitored continuously by the online monitoring station (online monitoring station is located at the project site where all the data [historical and instantaneous] from the LCS or panel meters of all WECs is retrieved) at the project site. In case of data loss, the data can be archived from this online monitoring system.

Training imparted to the Personnel

Training on the machine is an essential pre-requisite, to ensure necessary safety of man and machine. Further, in order to maximize the output from the Wind Energy Converters (WECs), it is extremely essential, that the engineers and technicians understand the machines and keep them in good health.  In  order  to  ensure,  that  Wind  World  (India)  Limited’s  service  staffs  is  deft  at handling technical snags on top of the turbine, the necessity of ensuring that they are capable of climbing the tower with absolute ease and comfort has been established. The Wind World (India) Limited Training Academy provides need-based training to meet the training requirements of Wind World (India) Limited projects. The training is contemporary, which results in imparting focused knowledge leading to value addition to the attitude and skills of all trainees. This ultimately leads to creativity in problem solving.

Calibration Details

The metering equipment were inspected & calibrated by state utility. Meter details for the main meters/GETCO meters are as follows:-

| Name of WWIL Substation | (Main Meter/GETCO Meter)Meter Serial No | Type/ Make | Accuracy Class | Frequency of Calibration | Calibration |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Previous Calibration | Validity |
| Sadodar S/s | GJB01470 | Secure | 0.2s | Once in a five years | 30/09/2016 | 29/09/2021 |
|  | GJU04175 | Secure | 0.2s | Once in a five years | 30/09/2016 | 29/09/2021 |
|  | GJU04176 | Secure | 0.2s | Once in a five years | 30/09/2016 | 29/09/2021 |
|  | KAB11082 | Secure | 0.2s | Once in a five years | 30/09/2016 | 29/09/2021 |

Meter Details of Cluster Meters:

| Meter Serial No | Make | Accuracy Class | Calibration dates during meter installation | Calibration during last verification | Latest Calibration dates |
| --- | --- | --- | --- | --- | --- |
| 10059208 | L&T | 0.2s | 24/08/2010 | 25/09/2013 | 24/05/2021 |
| 10059203 | L&T | 0.2s | 24/08/2010 | 25/09/2013 | 24/05/2021 |
| GJU60947 | Secure | 0.2ss | 03/09/2010 | 25/09/2013 | 24/05/2021 |
| GJU61707 | Secure | 0.2s | 29/01/2011 | 25/09/2013 | 24/05/2021 |
| GJU61698 | Secure | 0.2s | 29/01/2011 | 25/09/2013 | 24/05/2021 |
| GJU61321 | Secure | 0.2s | 05/02/2011 | 25/09/2013 | 24/05/2021 |
| GJU61313 | Secure | 0.2s | 18/12/2010 | 25/09/2013 | 24/05/2021 |
| GJU61690 | Secure | 0.2s | 29/01/2011 | 25/09/2013 | 24/05/2021 |
| GJU61699 | Secure | 0.2s | 29/01/2011 | 25/09/2013 | 24/05/2021 |
| GJU61322 | Secure | 0.2s | 09/12/2010 | 26/09/2013 | 25/05/2021 |
| GJU61696 | Secure | 0.2s | 29/01/2011 | 26/09/2013 | 25/05/2021 |
| GJU61310 | Secure | 0.2s | 18/12/2010 | 26/09/2013 | 25/05/2021 |
| GJU61701 | Secure | 0.2s | 29/01/2011 | 26/09/2013 | 25/05/2021 |
| GJU61693 | Secure | 0.2s | 29/01/2011 | 26/09/2013 | 25/05/2021 |
| GJU61692 | Secure | 0.2s | 29/01/2011 | 25/09/2013 | 25/05/2021 |
| GJU61691 | Secure | 0.2s | 29/01/2011 | 26/09/2013 | 25/05/2021 |
| GJU60943 | Secure | 0.2s | 03/09/2010 | 26/09/2013 | 25/05/2021 |

- During the current monitoring period, it has been identified that the last calibration for the cluster meters has been performed in the year 2013, which were valid till September 2018 and afterwards the calibration has been performed in the year 2021. The calibration has not been performed as per the frequency mentioned, i.e. once in five years.

- However, all the meters were operating under permissible error i.e. less than the accuracy class 0.2, hence, under conservative approach, a maximum correction factor (0.2%) has been applied by the PP in the emission reduction calculations for the entire monitoring period, i.e. from 01 April 2018 to 31 December 2019.

Emergency procedure:

If during meter testing the main meter at the Wind World (India) Limited substation is found beyond the permissible limit of error, the meter reading will be taken from the meter located at the utility substation at Moti Panelli after addition of average historical transmission losses.

If during meter testing the cluster meter is found beyond the permissible limit of error, the meter will be replaced by new meter.

The line diagram showing all relevant monitoring points has been provided below:

#### SECTION D. DATA AND PARAMETERS

##### D.1. Data and parameters fixed ex ante or at renewal of crediting period

>>

| Relevant SDG Indicator | 13.2.1 Number of countries that have communicated the establishment or operationalization of an integrated policy/strategy/plan which increases their ability to adapt to the adverse impacts of climate change, and foster climate resilience and low greenhouse gas emissions development in a manner that does not threaten food production (including a national adaptation plan, nationally determined contribution, national communication, biennial update report or other) |
| --- | --- |
| Data/parameter: | EFgrid,OM,y |
| Unit | tCO2e/MWh |
| Description | Operating Margin Emission Factor of NEWNE Regional Electricity Grid |
| Source of data | The “CO2 Baseline Database for Indian Power Sector” is available at http://www.cea.nic.in/reports/planning/cdm_co2/cdm_co2.htm |
| Value(s) applied) | 1.00498 |
| Choice of data or measurement methods and procedures | “CO2 Baseline Database for Indian Power Sector”, version 5 published by the Central Electricity Authority, Ministry of Power, Government of India. |
| Purpose of data | This data is used for baseline emission calculation. |
| Additional comments | The value is calculated on ex-ante basis and it will remain same throughout the crediting period. |

| Relevant SDG Indicator | 13.2.1 Number of countries that have communicated the establishment or operationalization of an integrated policy/strategy/plan which increases their ability to adapt to the adverse impacts of climate change, and foster climate resilience and low greenhouse gas emissions development in a manner that does not threaten food production (including a national adaptation plan, nationally determined contribution, national communication, biennial update report or other) |
| --- | --- |
| Data/parameter: | EFgrid,BM,y |
| Unit | tCO2e/MWh |
| Description | Build Margin Emission Factor of NEWNE Regional Electricity Grid |
| Source of data | The “CO2 Baseline Database for Indian Power Sector” is available at http://www.cea.nic.in/reports/planning/cdm_co2/cdm_co2.htm |
| Value(s) applied) | 0.67518 |
| Choice of data or measurement methods and procedures | “CO2 Baseline Database for Indian Power Sector”, version 5 published by the Central Electricity Authority, Ministry of Power, Government of India. |
| Purpose of data | The data is used to calculate baseline emission reductions. |
| Additional comments | The value is calculated on ex-ante basis and it will remain same throughout the crediting period. |

| Relevant SDG Indicator | 13.2.1 Number of countries that have communicated the establishment or operationalization of an integrated policy/strategy/plan which increases their ability to adapt to the adverse impacts of climate change, and foster climate resilience and low greenhouse gas emissions development in a manner that does not threaten food production (including a national adaptation plan, nationally determined contribution, national communication, biennial update report or other) |
| --- | --- |
| Data/parameter: | EFgrid,,CM,y |
| Unit | tCO2e/MWh |
| Description | Combined Margin Emission Factor of NEWNE Regional Electricity Grid |
| Source of data | The “CO2 Baseline Database for Indian Power Sector” is available at http://www.cea.nic.in/reports/planning/cdm_co2/cdm_co2.htm |
| Value(s) applied) | 0.92252 |
| Choice of data or measurement methods and procedures | “CO2 Baseline Database for Indian Power Sector”, version 5 published by the Central Electricity Authority, Ministry of Power, Government of India. |
| Purpose of data | The data is used to calculate baseline emission reductions. |
| Additional comments | The value is calculated on ex-ante basis and it will remain same throughout the crediting period. |

##### D.2 Data and parameters monitored

>>

| Relevant SDG Indicator | 7.2.1Renewable energy share in the total final energy consumption |
| --- | --- |
| Data/parameter: | EGPJ,y |
| Unit | MWh (Mega-watt hour) |
| Description | Net Quantity of Electricity exported to the grid |
| Measured/calculated/default | The net electricity supplied to the grid by the wind farm is calculated by GEDA on the basis of GETCO main meter reading and the meter readings taken at individual cluster meters after adjusting transmission loss. |
| Source of data | Share certificate issued by GETCO |
| Value(s) of monitored parameter | 132,789.152 MWh |
| Monitoring equipment | Calculated as per formulas described under section C. |
| Measuring/reading/recording frequency: | Monthly |
| Calculation method (if applicable): | The procedures for metering have been as per the provisions of the power purchase agreement. The WECs of a single customer (VIPCPL in this case) has been divided into clusters and each cluster has dedicated metering system. Different clusters are connected to different Vacuum Circuit Breaker metering yards (VCB) which ultimately lead to the shared main GETCO meter (also known as revenue meter) at the Sadodar substation maintained by Wind World (India) Limited. Data monitoring takes place at the cluster metering points and GETCO main meter at the WWIL substation. The net electricity supplied to the grid by the wind farm has been calculated by GEDA on the basis of GETCO main meter reading and the meter readings taken at individual cluster meters after adjusting transmission loss. For adjustment of transmission loss, the electricity metered at the GETCO meter has been proportionally divided by GEDA among the customers connected to the revenue meter on the basis of the pro rata readings taken at the cluster meters metering point. The net electricity generated by the project activity has been taken directly from the share certificate issued by GETCO on monthly basis. |
| QA/QC procedures: | Refer section C for an illustration of the provisions for QA/QC procedures. |
| Purpose of data: | This data is directly used for baseline estimation |
| Additional comments: | The data will be archived for the entire crediting period plus two years. |

| Relevant SDG Indicator | 7.2.1Renewable energy share in the total final energy consumption |
| --- | --- |
| Data/parameter: | EGGETCO, Export |
| Unit | kWh |
| Description | Net Electricity export recorded at Wind World (India) Limited Substation |
| Measured/calculated/ Default | Measured at Main Meter |
| Source of data | Joint Meter Reading (JMR) |
| Value(s) of monitored parameter | 1,093,782,552 kWh |
| Monitoring equipment | Please refer section C under heading calibration details of the MR |
| Measuring/reading/recording frequency | Monthly |
| Calculation method (if applicable) | Monitoring: Electricity export to the grid is recorded by the main meter at Wind World (India) Limited Substation. Frequency of recording data: Monthly Recording: The values of electricity exports to the grid are sourced from JMR. Responsibility: Joint responsibility of Wind World (India) Limited and state utility |
| QA/QC procedures | Refer section C for an illustration of the provisions for QA/QC procedures. |
| Purpose of data/parameter | Baseline Emissions |
| Additional comments | The data will be archived for the entire crediting period plus two years. |

| Relevant SDG Indicator | 7.2.1Renewable energy share in the total final energy consumption |
| --- | --- |
| Data/parameter: | EGGETCO, Import |
| Unit | kWh |
| Description | Net Electricity import recorded at Wind World (India) Limited Substation |
| Measured/calculated/default | Measured at Main Meter |
| Source of data | Joint Meter Reading (JMR) |
| Value(s) of monitored parameter | 25,506,000 kWh |
| Monitoring equipment | Please refer section C under heading calibration details of the MR |
| Measuring/reading/recording frequency: | Monthly |
| Calculation method (if applicable): | Monitoring: Electricity import to the grid is recorded by the main meter at Wind World (India) Limited Substation. Refer section C & D.2 for an illustration of the provisions for QA/QC procedures. Frequency of recording data: Monthly Recording: The values of electricity exports to the grid are sourced from JMR. Responsibility: Joint responsibility of Wind World (India) Limited and state utility. |
| QA/QC procedures: | Refer section C for an illustration of the provisions for QA/QC procedures. |
| Purpose of data: | Baseline Emissions |
| Additional comments: | The data will be archived for the entire crediting period plus two years. |

Sustainability Monitoring Plan:

| Relevant SDG Indicator | 8.5.2 Unemployment rate, by sex, age and persons with disabilities |
| --- | --- |
| Data/parameter: | Quality of Employment |
| Unit | Number of Training Programmes provided to the employees and the O & M staff |
| Description | Training records, categories of jobs created, occupational health management, safeguards put in place. Project developer has comprehensive internal systems in place wherein all essential norms pertaining to safety, occupational health and working conditions are being followed |
| Measured/calculated/default | Measured for duration 01/04/2018 to 31/12/2019 |
| Source of data | Documentation pertaining to training programmes, awareness generation activities, photographs, interviews etc. |
| Value(s) of monitored parameter | Following training programs have been conducted in order to enhance the safety awareness , operational skill levels and occupational health management for the local staff: - Safety week Trainings conducted at Samana Site on 09/03/2018 - Safety week Trainings conducted at Samana Site on 08/03/2019 The training programmes help in making the workforce efficient and skilled at their job. This not only helps the company but also adds to growth of individual employees. Thus, the project has a positive impact on the parameter. |
| Monitoring equipment | - |
| Measuring/reading/recording frequency: | Annually |
| Calculation method (if applicable): | Manually by PP representative |
| QA/QC procedures: | Continuation of regular trainings/workshops for employees & O&M staffs |
| Purpose of data: | Assessment of contribution towards ‘Safeguarding Principles’ and to monitor the contribution to SDG 8 |
| Additional comments: | None |

| Relevant SDG Indicator | 8.5.2 Unemployment rate, by sex, age and persons with disabilities |
| --- | --- |
| Data/parameter: | Quantitative employment and income generation |
| Unit | Number of Employees |
| Description | Number of jobs generated during the operation of the wind farm. |
| Measured/calculated/default | Measured from 01/04/2018 to 31/12/2019 |
| Source of data | Attendance Sheet and employment records maintained by Project Developer |
| Value(s) of monitored parameter | The project activity is located at Samana Site & connected to Sadodar Substation. At Sadodar Substation various sites are connected where local people are employed. The number of jobs created due to the project activity is divided under following categories: i. Service Staff ii. Other Staff During current monitoring period, total of 31 people have been employed at project site. Out of these 31 employees, seven people have been employed for the project activity during the current monitoring period. In addition to this, local villagers were engaged for short time jobs because of the infrastructure development (e.g. road construction, road repairs, cleaning of sub-station etc.) which leading to income generation. Further, for local transportation, PP is using local vehicles which have also created a revenue stream for the local people. Since this is an indirect benefit of the project activity in terms of income generation, it is not possible to exactly quantify the same. However, it can be conservatively stated that around 30 people have received additional livelihood/income generation opportunities because of the project activity. Same can be established through interviews with various local stakeholders by the DOE. Corresponding attendance sheets and other relevant records on annual monitoring have been submitted to the DOE. |
| Monitoring equipment | Attendance Sheet and employment records maintained by Project Developer |
| Measuring/reading/recording frequency: | Annually |
| Calculation method (if applicable): | Measured, Manually by Project Developer |
| QA/QC procedures: | Additional job opportunities created for the local population depending upon project site requirement for the continuous operation and maintenance of WECs. Income generation to be enhanced by creating relatively high value job opportunities through training and capacity building. |
| Purpose of data: | To monitor the contribution to SDG 8 |
| Additional comments: | None |

##### D.3. Comparison of monitored parameters with last monitoring period

| Data/Parameter | Value obtained in this monitoring period | Value obtained last monitoring period |
| --- | --- | --- |
| - SDG 7 : Affordable and Clean Energy | - 132,789.152 MWh | 81,173.394 MWh |
| - SDG 8 Decent Work and Economic Growth | 2 trainings 31 employees | 3 trainings 24 employees |
| - SDG 13 Climate Action | 119,302 tCO2 | 74,881 tCO2 |

##### D.4. Implementation of sampling plan

>>

Not Applicable

#### SECTION E. CALCULATION OF SDG IMPACTS

##### E.1. Calculation of baseline value or estimation of baseline situation of each SDG Impact

>>

According to the approved methodology ACM0002 (Version 11) Emission Reductions are calculated as:-

ERy = BEy – PEy

Where:

BEy Baseline Emissions in year y (t CO2e/yr)

PEy Project Emissions in year y (t CO2e/yr)

Baseline emissions:

Baseline emission factor (Combined Margin) (EFy)

= 0.92252 tCO2e/MWh

Baseline Emissions Reduction: ERy = EFgrid,CM,y * EGPJ,y

Where,

ERy   is baseline emissions in year y, tCO2e

EGPJ,y  is the net electricity supplied to the grid in year y and is applied directly from GEDA sharing certified by state utility. This value can also be cross-checked from the invoice.

EFy or EFgrid,CM,y is the CO2 emission factor of the grid (0.92252 tCO2e/MWh fixed ex-ante)

ERy = EFgrid,CM,y * EGPJ,y = 0.92252 *   132,789.152 = 119,302 tCO2e

Baseline Emission Reductions calculation for project activity:-

| Duration | Net electricity generation supplied to the grid by the Project activity [MWh] | Baseline Emission Factor (tCO2e/MWh) | Baseline Emissions (tCO2e) |
| --- | --- | --- | --- |
|  | [EGy] | [EFy] | [BEy] = [EGy] * [EFy] |
| 01/04/2018 to 31/12/2019 | 132,789.152 | 0. 92252 | 119,302 |
| Total | 132,789.152 | 0. 92252 | 119,302 |

| - Item | Baseline estimate |
| --- | --- |
| - SDG 7: Affordable and Clean Energy | No Activities in the baseline |
| - SDG 8: Decent Work and Economic Growth | No Activities in the baseline |
| - SDG 13: Climate Action | Emission of 119,302 tCO2 |

##### E.2. Calculation of project value or estimation of project situation of each SDG Impact

>>

Based on the surveys, PP identifies and works on several scope(s) of developmental activities such as health camps, distribution of furniture & sports kits in schools, toilet requirements in government schools, drinking water requirements etc. Apart from these activities, some or all of the following SDGs will be conducted in any given year:

| SDG Goal | Monitoring Plan |
| --- | --- |
| SDG 7: Ensure access to affordable, reliable, sustainable and modern energy for all | Method: The net electricity supplied to the grid by the wind farm has been calculated by GEDA on the basis of GETCO main meter reading and the meter readings taken at individual cluster meters after adjusting transmission loss. The net electricity generated by the project activity has been taken directly from the share certificate issued by GETCO on monthly basis. Frequency: Monthly QA/QC procedures: Net electricity supplied to the grid by the project activity has been cross checked with invoices submitted to EB. The meter(s) has been calibrated once in five years basis. Purpose: To measure the electricity produced and supplied to the grid |
| SDG 8: Promote sustained, inclusive and sustainable economic growth, full and productive employment and decent work for all | Method: Training records, Attendance Sheet, Employment records maintained by Project Developer with a comprehensive internal system in place wherein all essential norms pertaining to safety, occupational health and working conditions are being followed. Frequency: Annual QA/QC procedures: Continuation of regular trainings/workshops for employees & O&M staffs Purpose: To identify and record the number of trainings provided to the employees and employment generated due to the project. |
| SDG 13: Take urgent action to combat climate change and its impacts | Method: Using processes and equations provided under “Tool to calculate the emission factor for an electricity system”, and referencing data from CEA database. Frequency: Every monitoring period (whereas Emission Factor calculated based on combined margin approach is fixed ex-ante for the entire crediting period) QA/QC procedures: Transparent data collection, analysis, calculation and reporting as per CEA database which is publicly available. Purpose: To calculate emissions avoided due to the project activity. |

##### E.3. Calculation of leakage

>>

Not Applicable

##### E.4. Calculation of net benefits or direct calculation for each SDG Impact

| SDG | SDG Impact | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- | --- |
| SDG 7 | - Affordable and Clean Energy | 0 | 132,789.152 MWh | 132,789.152 MWh |
| - SDG 8 | - Decent Work and Economic Growth | 0 | 2 trainings 31 employees | 2 trainings 31 employees |
| SDG 13 | - Climate Action | 122,500 tCO2 | 0 | 119,302 tCO2 |

##### E.5. Comparison of actual SDG Impacts with estimates in approved PDD

| SDG | Values estimated in ex ante calculation of approved PDD for this monitoring period | Actual values achieved during this monitoring period |
| --- | --- | --- |
| - SDG 7 : Affordable and Clean Energy | MWh | 173,306.545 MWh |
| - SDG 8 Decent Work and Economic Growth | 2 trainings 24 employees | 2 trainings 31 employees |
| - SDG 13 Climate Action | 186,526 tCO2 | 119,302 tCO2 |

E.5.1. Explanation of calculation of value estimated ex ante calculation of approved PDD for this monitoring period

>>

The actual emission reduction achieved is 34.33% less than the estimated figure as per registered PDD. This is due to lower electricity generation compared to estimated generation in registered PDD (for the equivalent period) during the monitoring period. Further, as the actual generation is less than the estimated generation mentioned in registered PDD, thus, further justification is not necessary.

##### E.6. Remarks on increase in achieved SDG Impacts from estimated value in approved PDD

>>

As shown in calculation in previous section, there is no increase in achieved SDG impact for SDG 13.

#### SECTION F. SAFEGUARDS REPORTING

>>

Not applicable.

Safeguard principles were assessed in detail and reported under the registered GS-Passport and Transition document.

#### SECTION G. STAKEHOLDER INPUTS AND LEGAL DISPUTES

##### G.1. List all Inputs and Grievances which have been received via the Continuous Input and Grievance Mechanism together with their respective responses/mitigations.

>>

No grievances received during the monitoring period together with their respective answers/actions.

##### G.2. Report on any stakeholder mitigations that were agreed to be monitored.

>>

No grievances received in the previous monitoring period, thus no follow up required.

##### G.3. Provide details of any legal contest that has arisen with the project during the monitoring period

>>

No legal contest or dispute that has arisen with the project during the monitoring period.

Appendix 1

| Sr. No. | Location No. | WTG-ID No. | Village | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- |
| 1 | 3020 | EIL/800/10-11/1826 | Machharda | N22 O 06’ 19.0” | E70 O 18’ 45.7” |
| 2 | 3021 | EIL/800/10-11/1827 | Machharda | N22 O 06’ 23.5” | E70 O 18’ 43.7” |
| 3 | 3022 | EIL/800/10-11/1828 | Machharda | N22 O 06’ 29.7” | E70 O 18’ 44.6” |
| 4 | 3072 | EIL/800/09-10/1738 | Padavala | N21 O 57’ 19.6” | E70 O 15’ 05.0” |
| 5 | 3073 | EIL/800/09-10/1739 | Padavala | N21 O 57’ 14.9” | E70 O 15’ 11.7” |
| 6 | 3075 | EIL/800/09-10/1740 | Padavala | N21 O 56’43.1” | E70 O 15’ 20.6” |
| 7 | 3076 | EIL/800/09-10/1741 | Padavala | N21 O 55’ 59.2” | E70 O 15’ 33.7” |
| 8 | 3088 | EIL/800/09-10/1742 | Padavala | N21 O 56’ 19.3” | E70 O 14’ 38.0” |
| 9 | 62 | EIL/800/09-10/1766 | Chattar | N22 O 07’ 40.2” | E70 O 15’ 10.7” |
| 10 | 63 | EIL/800/09-10/1767 | Chattar | N22 O 07’ 46.6” | E70 O 15’00.6” |
| 11 | 64 | EIL/800/09-10/1768 | Chattar | N22 O 07’ 53.3” | E70 O 14’ 57.1” |
| 12 | 539 | EIL/800/09-10/1789 | Seth Wadala | N22 O 04’ 46.7” | E70 O 05’ 34.3” |
| 13 | 540 | EIL/800/09-10/1790 | Seth Wadala | N22 O 04’ 33.3” | E70 O 05’ 43.1” |
| 14 | 541 | EIL/800/09-10/1791 | Seth Wadala | N22 O 04’ 27.4” | E70 O 05’ 47.6” |
| 15 | 543 | EIL/800/09-10/1792 | Seth Wadala | N22 O 04’ 17.3” | E70 O 05’ 53.7” |
| 16 | 544 | EIL/800/09-10/1793 | Seth Wadala | N22 O 04’ 13.5” | E70 O 06’ 00.7” |
| 17 | 545 | EIL/800/09-10/1794 | Seth Wadala | N22 O 03’ 31.5” | E70 O 05’ 32.6” |
| 18 | 546 | EIL/800/09-10/1795 | Jam Ambardi | N22 O 03’ 40.2” | E70 O 05’ 31.0” |
| 19 | 547 | EIL/800/09-10/1796 | Jam Ambardi | N22 O 03’ 45.3” | E70 O 05’ 31.9” |
| 20 | 548 | EIL/800/09-10/1797 | Jam Ambardi | N22 O 03’ 50.7” | E70 O 05’ 34.2” |
| 21 | 903 | EIL/800/09-10/1747 | Mevasa/ Haripar | N22 O 01’ 23.0” | E70 O 15’ 35.2” |
| 22 | 904 | EIL/800/09-10/1748 | Mevasa/ Haripar | N22 O 01’ 30.2” | E70 O 15’ 41.0” |
| 23 | 905 | EIL/800/09-10/1749 | Mevasa/ Haripar | N22 O 01’ 36.6” | E70 O 15’ 27.2” |
| 24 | 906 | EIL/800/09-10/1750 | Mevasa/ Haripar | N22 O 01’ 30.7” | E70 O 14’ 55.0” |
| 25 | 907 | EIL/800/09-10/1751 | Mevasa/ Haripar | N22 O 01’ 37.9” | E70 O 14’ 56.8” |
| 26 | 908 | EIL/800/09-10/1752 | Mevasa/ Haripar | N22 O 01’ 44.8” | E70 O 14’ 54.1” |
| 27 | 909 | EIL/800/09-10/1753 | Mevasa/ Haripar | N22 O 01’ 51.2” | E70 O 14’ 51.2” |
| 28 | 910 | EIL/800/09-10/1754 | Mevasa/ Haripar | N22 O 01’ 57.7” | E70 O 14’ 55.7” |
| 29 | 912 | EIL/800/09-10/1746 | Dhun Dhoraji | N22 O 02’ 09.1” | E70 O 15’ 04.4” |
| 30 | 926 | EIL/800/09-10/1769 | Chattar | N22 O 06’ 57.6” | E70 O 16’ 33.0” |
| 31 | 927 | EIL/800/09-10/1770 | Chattar | N22 O 06’ 59.3” | E70 O 16’ 23.3” |
| 32 | 928 | EIL/800/09-10/1771 | Chattar | N22 O 07’ 10.0” | E70 O 16’ 16.5” |
| 33 | 929 | EIL/800/09-10/1772 | Chattar | N22 O 07’ 15.9” | E70 O 16’ 11.3” |
| 34 | 931 | EIL/800/10-11/1870 | Chattar | N22 O 07’ 12.7” | E70 O 15’ 23.5” |
| 35 | 932 | EIL/800/09-10/1773 | Chattar | N22 O 07’ 05.5” | E70 O 15’ 27.2” |
| 36 | 933 | EIL/800/09-10/1774 | Chattar | N22 O 06’ 59.3” | E70 O 15’ 31.5” |
| 37 | 934 | EIL/800/09-10/1775 | Chattar | N22 O 06’ 53.9” | E70 O 15’ 27.9” |
| 38 | 935 | EIL/800/09-10/1776 | Chattar | N22 O 06’ 46.0” | E70 O 15’ 22.7” |
| 39 | 936 | EIL/800/09-10/1777 | Chattar | N22 O 06’ 40.3” | E70 O 15’ 25.7” |
| 40 | 937 | EIL/800/09-10/1778 | Chattar | N22 O 06’ 32.0” | E70 O 15’ 23.4” |
| 41 | 938 | EIL/800/09-10/1779 | Chattar | N22 O 06’ 25.7” | E70 O 15’ 22.1” |
| 42 | 939 | EIL/800/09-10/1760 | Jamvadi | N22 O 08’ 19.5” | E70 O 19’ 02.3” |
| 43 | 941 | EIL/800/09-10/1761 | Jamvadi | N22 O 08’ 07.2” | E70 O 18’ 57.8” |
| 44 | 942 | EIL/800/09-10/1762 | Jamvadi | N22 O 08’ 08.6” | E70 O 19’ 30.2” |
| 45 | 943 | EIL/800/09-10/1763 | Jamvadi | N22 O 08’ 00.9” | E70 O 19’ 25.4” |
| 46 | 944 | EIL/800/09-10/1764 | Jamvadi | N22 O 07’ 53.9” | E70 O 19’ 26.0” |
| 47 | 945 | EIL/800/09-10/1765 | Jamvadi | N22 O 07’49.5” | E70 O 19’ 31.4” |
| 48 | 947 | EIL/800/09-10/1755 | Moti Vavdi | N22 O 06’ 04.0” | E70 O 18’ 16.9” |
| 49 | 948 | EIL/800/09-10/1756 | Moti Vavdi | N22 O 05’ 57.0” | E70 O 18’ 17.8” |
| 50 | 950 | EIL/800/09-10/1757 | Moti Vavdi | N22 O 05’ 45.7” | E70 O 18’ 21.5” |
| 51 | 951 | EIL/800/09-10/1758 | Moti Vavdi | N22 O 05’ 38.3” | E70 O 18’ 18.4” |
| 52 | 952 | EIL/800/09-10/1759 | Moti Vavdi | N22 O 05’ 31.6” | E70 O 18’ 16.9” |
| 53 | 958 | EIL/800/09-10/1743 | Dhun Dhoraji | N22 O 02’ 32.4” | E70 O 16’ 42.8” |
| 54 | 959 | EIL/800/09-10/1744 | Dhun Dhoraji | N22 O 02’ 26.2” | E70 O 16’ 44.6” |
| 55 | 960 | EIL/800/09-10/1745 | Dhun Dhoraji | N22 O 02’ 19.0” | E70 O 16’ 44.4” |
| 56 | 992 | EIL/800/09-10/1782 | Sadodar | N22 O 03’ 13.6” | E70 O 10’ 37.3” |
| 57 | 993 | EIL/800/09-10/1783 | Sadodar | N22 O 03’ 09.5” | E70 O 10’ 40.0” |
| 58 | 994 | EIL/800/09-10/1784 | Sadodar | N22 O 02’ 59.6” | E70 O 10’ 36.4” |
| 59 | 995 | EIL/800/09-10/1785 | Sadodar | N22 O 02’ 54.2” | E70 O 10’ 33.5” |
| 60 | 996 | EIL/800/09-10/1786 | Sadodar | N22 O 02’ 47.4” | E70 O 10’ 22.2” |
| 61 | 997 | EIL/800/09-10/1787 | Sadodar | N22 O 02’ 41.3” | E70 O 10’ 32.4” |
| 62 | 1028 | EIL/800/09-10/1788 | Seth Wadala | N22 O 03’ 06.0” | E70 O 08’ 36.9” |
| 63 | 1045 | EIL/800/09-10/1780 | Bodi | N22 O 08’ 43.4” | E70 O 15’ 11.4” |
| 64 | 1046 | EIL/800/09-10/1781 | Bodi | N22 O 08’ 48.8” | E70 O 15’ 08.5” |

Revision History

| Version | Date | Remarks |
| --- | --- | --- |
| 1.1 | 14 October 2020 | Hyperlinked section summary to enable quick access to key sections Improved clarity on Key Project Information Section for POA monitoring Forward action request section Improved Clarity on SDG contribution/SDG Impact term used throughout Clarity on safeguard reporting Clarity on design changes Leakage section added for VER/CER projects Addition of Comparison of monitored parameters with last monitoring period Provision of an accompanying Guide to help the user understand detailed rules and requirements |
| 1.0 | 10 July 2017 | Initial adoption |

For the delay in site visit, the emission reductions can only be claimed in the period of three years prior to onsite verification date 22/07/2021, i.e., from 22/07/2018. PP has foregone generation for the period of 22/07/2018 to 31/07/2018 to simplify calculation of emission reductions. The credits are claimed from 01/08/2018 to 31/12/2019.

The previous verification visit was performed on 31/05/2018 and for this monitoring period on 22/07/2021, there is more than 3 years gap between the verification visits, following the guideline, PP cannot claim credits from 01/04/2018 to 27/07/2018. Although the billing cycle for the project is 1st of every month to last day, hence didn’t claim credits from 01/04/2018 to 31/07/2018 and considered the generation zero from April 2018 to July 2018.

For the delay in site visit, the emission reductions can only be claimed in the period of three years prior to onsite verification date 22/07/2021, i.e., from 22/07/2018. PP has foregone generation for the period of 22/07/2018 to 31/07/2018 to simplify calculation of emission reductions. The credits are claimed from 01/08/2018 to 31/12/2019.

- After applying the error factor

- This reading is used for calculation of transmission loss by GEDA and is not directly used for calculation of emission reductions.

- This reading is used for calculation of transmission loss by GEDA and is not directly used for calculation of emission reductions.

For the delay in site visit, the emission reductions can only be claimed in the period of three years prior to onsite verification date 22/07/2021, i.e., from 22/07/2018. PP has foregone generation for the period of 22/07/2018 to 31/07/2018 to simplify calculation of emission reductions. The credits are claimed from 01/08/2018 to 31/12/2019.

After applying the error factor

After 2% deduction on account of UNFCCC Adaptation Fund

The previous verification visit was performed on 31/05/2018 and for this monitoring period on 22/07/2021, there is more than 3 years gap between the verification visits, following the guideline, PP cannot claim credits from 01/04/2018 to 27/07/2018. Although the billing cycle for the project is 1st of every month to last day, hence didn’t claim credits from 01/04/2018 to 31/07/2018 and considered the generation zero from April 2018 to July 2018.

After 2% deduction on account of UNFCCC Adaptation Fund

After 2% deduction on account of UNFCCC Adaptation Fund

After 2% deduction on account of UNFCCC Adaptation Fund