Gold standard for the global goals

Monitoring report

June 2017, version 1

Title of the project; Bergres Wind Power Project, Turkey. Gold Standard project id; GS 5229. Version number of the monitoring report; 04. Completion date of the monitoring report; 17/08/2020. Date of project design certification; 17/04/2017. Start date of crediting period; 21/04/2017. Duration of this monitoring period; 01/05/2019 to 30/04/2020 (both days are included, 1. Crediting period, 3. Monitoring period). Duration of previous monitoring period; 01/05/2018 to 30/04/2019 (both days are included, 1. Crediting period, 2. Monitoring period). Project representative(s); Bergres Elektrik Üretim A.Ş.. Host Country; Turkey. Certification pathway (activity certification/impact certification); Impact Certification. SDG Contributions targeted (as per approved PDD); 1 – SDG 13- Climate Action 2 – SDG 7- Affordable and Clean Energy 3 – SDG 8- Decent Work and Economic Growth 4 – SDG 6- Clean Water and Sanitation. Gold Standard statement/product certification sought (GSVER/ADALYs/RECs etc.); GSVER. Selected methodology(ies); ACM0002 Version 17. Estimated amount of annual average certified SDG impact (as per approved PDD); 1 – SDG 13: CO2: 131,241 tCO2/year, CO: 36.3 and NMVOC: 7.7 tons 2 – SDG 7: Avoidance 23,294 m3 natural gas import 3 – SDG 8: At least 2 local people will be employed – SDG 8: Health & Safety trainings are to be held for employees at the plant. Training records will be provided for all employees. 4 – SDG 6: Avoidance of around 4,912.3 m3/year wastewater discharge. Total amount of certified SDG impact (as per approved methodology) achieved in this monitoring period; SDG 13: Emission Reduction: 127,325 tCO2 Air Quality: CO: 35.2 tons/y NMVOC: 7.5 tons/y; SDG 6: Water Quality: No wastewater discharge to the environment in the project area. Avoidance of around 4,756.088 m3 wastewater discharge to the environment per year.. SDG 8: Quality of Employment: All the workers that are employed by Bergres Enerji Santrali Üretim A.Ş have taken a series of training courses including technical capacity development and personal development. Occupational health and safety trainings have been provided in 2019 and 11 employees are attended to the trainings. Training records for the employees are provided. Quantitative Employment and Income Generation: 11 employees work at the WPP. Out of these 11 employees, 6 have been employed by Bergres Elektrik. The rest has been employed by the other companies (Securitas and GE) from which services are taken. There are 5 local employees. Current employment records are now presented.; SDG 7: Balance of payments and investment: 7.1 - Avoidance of around 22,998 m3 natural gas import each year by generation of project activity, which is worth about 6,256,187.65 EUR. 7.2 - Energy is generated by using renewable energy systems throughout the project (Wind Power Plant). There is no electricity generation in the baseline scenario in which Bergres WPP is not being implemented. The generation amount was achieved as 230,915 MWh..

- Description of project

  - Purpose and general description of project

Bergres Elektrik Üretim A.Ş. (hereafter referred to as “Bergres Elektrik”) is investing into a new Wind Power project called Bergres Wind Power Project (hereafter referred to as the “Project” or “Bergres WPP”), which involves installation and operation of 69,95 MWm/69,95 MWe wind power plant. The project consists of 21 turbines with 3.2 MWm/MWe and 1 turbine with 2.75 MWm/MWe capacity. Energy Market Regulatory Authority (EMRA) issued the licence of the project on February 09 of 2012. The generated energy is fed to the grid at Bergres RES transmission line.

Table 1: Dates of Installation of Turbines

| No. of Turbine | Date of Installation |
| --- | --- |
| T1 | 26.05.2017 |
| T2 | 26.05.2017 |
| T3 | 26.05.2017 |
| T4 | 26.05.2017 |
| T5 | 26.05.2017 |
| T6 | 26.05.2017 |
| T7 | 20.04.2017 |
| T8 | 20.04.2017 |
| T9 | 20.04.2017 |
| T10 | 20.04.2017 |
| T11 | 20.04.2017 |
| T12 | 20.04.2017 |
| T13 | 20.04.2017 |
| T14 | 20.04.2017 |
| T15 | 20.04.2017 |
| T16 | 26.05.2017 |
| T17 | 26.05.2017 |
| T18 | 26.05.2017 |
| T19 | 26.05.2017 |
| T20 | 26.05.2017 |
| T21 | 26.05.2017 |
| T22 | 26.05.2017 |

An estimated electricity net generation of 238,000 MWh per year by the efficient utilization of the available wind energy by project activity replaces the grid electricity, which is constituted of different fuel sources, mainly fossil fuels. The electricity produced by project activity results in annual estimated emission reduction of 131,241 tonnes of CO2/year. Moreover, project activity contributes further dissemination of wind energy and extension of national power generation. The generation of electricity starts on 21/04/2017.

Crediting period of Bergres WPP started on 21/04/2017 and the crediting period is 7 years which will end on 20/04/2024. The third monitoring period is between 01/05/2019 to 30/04/2020. In this monitoring period, 230,915 MWh of net electricity is supplied to the grid and 127,325 tCO2 emission reduction has achieved.

Bergres WPP completed its transition process to Gold Standard for the Global Goals on 22/11/2018. Environmental and social monitoring parameters are revised in transition process and are connected to specific SGD parameters. Bergres WPP is now compatible with Gold Standard for the Global Goals and this monitoring report has taken revised monitoring parameters and SGDs into consideration.

The purpose of the project is to generate electricity and to feed it into the grid. Bergres WPP was registered as a Voluntary Emission Reduction project in order to enable the project implementation by means of financial inflows coming from the credits sale. Because of its significant contribution to climate protection and to sustainable development in the region, this project fulfils the requirements of the Gold Standard Organization.

The project helps Turkey to stimulate and commercialise the use of grid connected renewable energy technologies and markets. Furthermore, the project demonstrates the viability of grid connected wind farms which can support improved energy security, improved air quality, alternative sustainable energy futures, improved local livelihoods and sustainable renewable energy industry development. The specific goals of the project are to:

- reduce greenhouse gas emissions in Turkey compared to the business-as-usual scenario;

- help to stimulate the growth of the wind power industry in Turkey;

- create local employment during the construction and the operation phase of the wind farm;

- reduce other pollutants resulting from power generation industry in Turkey, compared to a business-as-usual scenario;

- help to reduce Turkeys increasing energy deficit;

- and differentiate the electricity generation mix and reduce import dependency.

As the project developer, Bergres Elektrik believes that efficient utilization of all kinds of natural resources with a harmony coupled with responsible environmental considerations is vital for sustainable development of Turkey and the World. This has been a guiding factor for the shareholders towards the concept of designation and installation of a wind power project. Other than the objective of climate change mitigation through significant reduction in greenhouse gas (GHG) emissions, the project has been carried out to provide social and economic contribution to the region in a sustainable way. The benefits that will be gained by the realization of the project compared to the business-as-usual scenario can be summarized under four main indicators:

Environmental

The project activities replaces the grid electricity, which is constituted of different fuel sources causing greenhouse gas emissions. By replacing in the consumption of these fuels, it contributes to conservation of water, soil, flora and faunas and transfers these natural resources and also the additional supply of these primary energy sources to the future generations. In the absence of the project activity, an equivalent amount of electricity would have been generated from the power plants connected to the grid, majority of which are based on fossil fuels. Thus, the project is replacing the greenhouse gas emissions (CO2, CH4) and other pollutants (SOX, NOX, particulate matters) occurring from extraction, processing, transportation and burning of fossil-fuels for power generation connected to the national grid.

Economical

Firstly, the project helps to accelerate the growth of the wind power industry and stimulate the designation and production of renewable energy technologies in Turkey. Then, other entrepreneurs irrespective of sector will be encouraged to invest in wind power generations. It also assists to reduce Turkey’s increasing energy deficit and diversify the electricity generation mix while reducing import dependency, especially natural gas. Importantly, rural development is maintained in the areas around the project site by providing infrastructural investments to these remote villages.

Social

Local employment is enhanced by all project activities during construction and operation of wind farm. As a result, local poverty and unemployment are partially eliminated by increased job opportunities and project business activities. Construction materials for the foundations, cables and other auxiliary equipment are preferentially sourced locally. Moreover as contribution of the project to welfare of the region, the quality of the electricity consumed in the region will be increased by local electricity production, which also contributes decreasing of distribution losses.

Technological

Implementation of the proposed project contributes to wider deployment of wind power technology in local and national level. It demonstrates the viability of larger grid connected wind farms, which support improved energy security, alternative sustainable energy, and also renewable energy industry development. This also strengthens pillars of Turkish electricity supply based on ecologically sound technology.

  - Location of project

The host country is Republic of Turkey. Project area is in Aegean Region, İzmir province. The project is close to Bergama town. Location of the project site is presented in Figure 1.

Figure 1: Map of the Project Area

The project site is located about 17 km south-east of the town of Bergama and approximately 60 km north of the city of Izmir in western Turkey. The wind farm area is located between the villages Avunduk and Balaban in the west, Yuntdag and Bayramcılar in the south-east and Hacılar in the east. The closest villages are Bayramcılar and Balaban villages and the distance between the villages and the project area is 500m.

The coordinates of the turbines are given below. The project coordinates were provided below which are under ED 50, zone 35 coordinate system.

Table 2: Coordinates of the Turbines

| Turbine No. | Easting | Northing |
| --- | --- | --- |
| T1 | 518750,00 | 4315500,00 |
| T2 | 518820,00 | 4315000,00 |
| T3 | 518250,00 | 4313770,00 |
| T4 | 518119,00 | 4311650,00 |
| T5 | 518243,00 | 4311226,00 |
| T6 | 519019,00 | 4311718,00 |
| T7 | 519169,00 | 4312506,00 |
| T8 | 519019,00 | 4312774,00 |
| T9 | 519770,00 | 4312470,00 |
| T10 | 520120,00 | 4312325,00 |
| T11 | 520350,00 | 4312175,00 |
| T12 | 520590,00 | 4312000,00 |
| T13 | 520768,00 | 4311832,00 |
| T14 | 521062,00 | 4311756,00 |
| T15 | 521321,00 | 4311023,00 |
| T16 | 520609,00 | 4309927,00 |
| T17 | 520759,00 | 4309697,00 |
| T18 | 520968,00 | 4309525,00 |
| T19 | 521255,00 | 4309305,00 |
| T20 | 521523,00 | 4309200,00 |
| T21 | 521819,00 | 4309625,00 |
| T22 | 522020,00 | 4309400,00 |

  - Reference of applied methodology

For the determination of the baseline, the official methodology ACM0002 version 17.0.0, “Large-scale Grid-connected electricity generation from renewable sources”, is applied, using conservative options and data as presented in the following section. This methodology refers to five Tools, which are:

- Tool to calculate the emission factor for an electricity system (Version 05.0.0);

- Tool for the demonstration and assessment of additionality (Version 07.0.0);

- Combined tool to identify the baseline scenario and demonstrate additionality (Version 06.0.0);

- Tool to calculate project or leakage CO2 emissions from fossil fuel combustion (Version 02.0.0).

- Tool to determine the remaining lifetime of the equipment

For baseline calculation the first tool, for additionality assessment the second tool is used. As third tool is the combination of the first and second tool, it is not used. Since no project emission or leakage calculation is required for wind power project fourth tool is not used, and finally to determine the remaining lifetime of the equipment fifth tool is used.

  - Crediting period of project

21/04/2017 - 20/04/2024 (Crediting period of the project is 7 years which is twice renewable.)

- Implementation of project

  - Description of implemented project

According to the Generation License, 21 wind turbines with unit capacity of 3,2 kWm/3,2 kWe and 1 turbine with unit capacity of 2,75 kWm/kWe were selected for the project. GE Wind Energy is decided as equipment provider due to the outstanding features of its product regarding safety factors, simple durable design for low maintenance and long life operation, high efficiency, and also for fine visual appearance.

The key parameters about the technical design of the selected model GE-3.2-103 turbines are listed below in Table 3.

Table 3: Technical specifications of GE-3.2-103 turbines

| Specifications | GE-3.2-103 |
| --- | --- |
| Rated Power (kW) | 3200 |
| Rotor Diameter (m) | 103 |
| Hub Height (m) | 85 |
| Num. of Blades | 3 |
| Hub Height wind speed (m/s) | 8.4 |

The key parameters about the technical design of the selected model GE-2.75-103 turbines are listed below in Table 4.

Table 4: Technical specifications of GE-2.75-103 turbines

| Specifications | GE-2.75-103 |
| --- | --- |
| Rated Power (kW) | 2750 |
| Rotor Diameter (m) | 103 |
| Hub Height (m) | 85 |
| Num. of Blades | 3 |
| Hub Height wind speed (m/s) | 8.4 |

Selected these turbines, whose technical specifications are given above and having unit capacity of 3,2 kWm/kWe and 2,75 kWm/kWe, both have the same technical specifications. The electricity generation of the 3,2 kWm/kWe turbine is limited to 2,75 kWm/kWe. For this reason, the technical specifications of the GE-2.75-103 turbine are the same as GE-3.2-103 turbines. The only difference is that this turbine operates at a lower capacity (2750 kW) than the others.

A general operation diagram of the project is given in Figure 2.

Figure 2: Operation diagram of the project

The project installation is completed according to the description in the PDD and completely operational. The project consists only one site. The most important milestones are included in the following table:

Table 5: Milestones of the Bergres Wind Power Project

| Date (DD/MM/YYYY) | Milestone |
| --- | --- |
| 10/01/2012 | EIA Not Required Certificate |
| 09/02/2012 | Issuance of the generation license |
| 12/11/2015 | Agreement with Equipment Provider (GE) (Investment decision date) |
| 18/02/2016 | Start Date of Construction |
| 28/03/2016 | Credit Agreement |
| 15/06/2016 | Agreement with Life Enerji for Carbon Consultancy |
| 17/04/2017 | Registration date |
| 20/04/2017 | First commissioning dates of the turbines |
| 21/04/2017 | Electricity generation date/Start date of first monitoring period and crediting period |
| 26/05/2017 | Last commissioning dates of the turbines |
| 30/04/2018 | End date of first monitoring period |
| 01/05/2018 | Start date of the second monitoring period |
| 16/08/2018 | Start date of the transition process |
| 22/11/2018 | End date of the transition process |
| 30/04/2019 | End date of the second monitoring period |
| 01/05/2019 | Start date of the third monitoring period |
| 30/04/2020 | End date of the third monitoring period |
| 20/04/2024 | End date of the first crediting period |

  - Post-registration changes

    - Temporary deviations from Certified Key Project Information, Project Design Document, Monitoring & Reporting Plan, applied methodology or applied standardized baseline

None

    - Corrections

None

    - Changes to start date of crediting period

N/A

    - Permanent changes from registered monitoring plan, applied methodology or applied standardized baseline

None

    - Changes to project design of approved project

None

- Description of monitoring system applied by the project

The first crediting period started on 21/04/2017. The second monitoring period started on 01/05/2018. As the crediting period started on 21/04/2017, the estimated emission reductions from the first and last year of the crediting period sum up to the amount of the estimated yearly reductions.

- C.1 	Data Processing and Archiving

- C.1.1. Data Processing

Data handling was slightly revised due to innovation in technology that enables TEİAŞ to read meters remotely. Thus, under this process, the meter is read remotely at the end of every month and published through website of TEİAŞ which can be accessed by project owner to check the correctness of the data. Besides, project owner carries monthly protocol to cross-check the data. By this way, a cross check is done. Once the meter data is correct then the amount is also published in EPİAŞ website that is also accessible by project owner. When all data is correct, an invoice (receipt of sale) is prepared by Bergres and delivered to TEIAŞ. The electricity generation amount, the collected data, are kept in both hardcopy and softcopy in daily and monthly basis indefinitely. Moreover, these data are submitted to several authorized people. Thus, several additional copies of these data are stored at different computers. The project owner will keep the collected data during the monitoring period at least two years after the last issuance of VERs.

As the necessary baseline emission factors are all defined ex ante, the only information to be monitored is the amount of electricity fed into the grid by Bergres WPP. The data which is base for the invoices and published in EPİAŞ website is used as input for emission reduction calculation.

Electricity transfer from turbine to transmission line (line diagram) can be seen in Figure 3. SCADA system and remote control of meters can be seen from Figure 4 and monitoring system can be visualized.

Figure 3: Line Diagram of the Bergres WPP

Figure 4: Electricity transmission from turbine to transmission line

Note: On 01/09/2015, PMUM was renamed as EPİAŞ.

- C.1.2. A Backup Unit for Systematic Storage

The fact that two reliable best practice meters are installed in a redundant manner keeps the uncertainty level of the only parameter for baseline calculation low. High data quality of this parameter is in the interest of not only the emission reduction monitoring, but also paramount for the business relation between the plant operator and the electricity buyer. Since the meters are reading electricity supplied to the system and withdrawn from the system separately, the net electricity amount supplied to the grid will be calculated by electricity supplied minus electricity withdrawn that will be taken from monthly settlement notifications.

Monthly data from TEIAS are stored electronically on TEIAS website and this data can be accessed by login of Project Developer. Bergres also stores a hardcopy of the monthly meter reading protocols for back up of systematic storage of data.

Moreover, there are always internal reviews of the meters data which is checked by different parties. First of all, data of the meters is collected by technicians daily in written forms. The data collected daily is saved in plant manager computer and backed up and shared by headquarter of Bergres in Istanbul. Besides the data that can be get from meters, production amount can be checked from SCADA system. SCADA figures differ a bit with meter data due to internal losses.

Operation and management diagram is presented in Figure 5.

Figure 5: Operation and management diagram of the Bergres WPP

C.1.3   Calibration Procedures:

The calibration of the monitoring equipment was carried out according to the information provided in the GS-VER PDD (version 08, dated 21/09/2017). The GS-VER PDD mainly includes the following obligation for the calibration of the appropriate meters:

“The Turkish Electricity Market Regulation Agency (EPDK) sets rules on the accuracy of electricity meters that are used by power plants feeding into the grid. The rules are part of the EPDK regulation 25056 from 22 March 2003. The table in Article 11 of the regulation specifies the use of electricity meters of the accuracy class 0.5S for power plants between 10 MW and 100 MW and refers to compliance with International Electro technical Commission’s norm EN 60687. TEIAŞ conducts remote reading. In addition, operative manager, employee of Bergres Enerji, conducts reading from the project site and sends to TEIAŞ. By this way, a cross-check is done. Also, with the readings from project site, continuously these two meters are compared with each other and if any differences are detected the necessary control measures are taken.

Table 6: Specification of the electricity meters

| Name | Serial Number | Brand –Model | Year of Manufacture | Accuracy Class |
| --- | --- | --- | --- | --- |
| Main Meter | 5704873 | EMH-LZQ-XC | 2014 | Active C (0.5) – Reactive 2 |
| Backup meter | 5704874 | EMH-LZQ-XC | 2014 | Active C (0.5) – Reactive 2 |

First calibration date: 21/04/2017

Installation Date of the meters: 21/04/2017

*No test is performed during this monitoring period.

According to the Article 2 of the Communiqué: ‘The meters to be used in the electricity market shall be compliant with the standards of Turkish Standards Institute or IEC and have obtained “Type and System Approval” certificate from the Ministry of Trade and Industry.’ Therefore, Ministry of Trade and Industry (Ministry) is responsible from control and calibration of the meters.

paragraph b) of the Article 9 of the 'Regulation of Metering and Testing of Metering Systems' 7 (Regulation) of Ministry states that: ‘ b) Periodic tests of meters of electricity, water, coal gas, natural gas and current and voltage transformers are done every 10 years’. Therefore, periodic calibration of the meters will be done every 10 years. Therefore, the next calibration will be applied in 2027.

C.1.4. 	Troubleshooting Procedures

As the measuring devices are sealed by TEIAŞ, Bergres cannot intervene with the devices. In case of unforeseen problems or failures of the meters or if any differences occur between primary and secondary devices TEIAŞ has to be informed for necessary maintenance and calibration. There is an agreement between Bergres and TEIAŞ that in case of problems or failures of the meters TEIAŞ reacts as fast as possible to solve the problem.

For this monitoring period, there have not been an accident or an incident, which may affect the electricity generation of the project.

- Data and parameters

  - Data and parameters fixed ex ante or at renewal of crediting period

| Relevant SDG Indicator | SDG 13- Climate Action Target: 13.3 by emission reduction Indicator: 13.3.1 |
| --- | --- |
| Data/parameter: | EFgrid,CM,y |
| Unit | tCO2/MWh |
| Description | Combined margin CO2 emission factor for grid connected power generation in year y calculated using the “Tool to calculate the emission factor for an electricity system (Version 05.0.0)” |
| Source of data | Tool to calculate the emission factor for an electricity system (Version 05.0.0) |
| Value(s) applied) | 0.5514 tCO2/MWh |
| Choice of data or measurement methods and procedures | Combined Margin Emission Factor (EFgrid,CM,y) is calculated ex ante as the weighted average CO2 of Operating Margin Emission Factor (EFgrid,OM,y) and Build Margin Emission Factor (EFgrid,BM,y). Please see registered PDD and Transition Annex document. |
| Purpose of data | Calculation of baseline emissions |
| Additional comments | Other parameters are not specified as they are used in calculations in accordance with the registered PDD. Please see registered PDD and Transition Annex document. |

  - Data and parameters monitored

| Relevant SDG Indicator/Safeguarding Principle | SDG 13: Climate Action Target: 13.3 by emission reduction Indicator: 13.3.1 |
| --- | --- |
| Data / Parameter | Air quality |
| Unit | tons of CO2e |
| Description | Amount of CO2 emission |
| Source of data | Monthly meter readings |
| Value(s) applied | Reductions of proportionate amount in described emissions during operational life of the project activity. When electricity generation amount (230,915 MWh/y) is considered for the project activity then annual CO2 emission avoidances with project implementation becomes 127,325 tCO2e/yr respectively. |
| Measurement methods and procedures | Amount of annual net electricity generation, which is calculated by monthly settlement notifications of PMUM based on monthly meter readings, will be used to calculate estimated CO2 emission reduction by project activity. |
| Monitoring frequency | Annually. |
| QA/QC procedures |  |
| Purpose of data | Emission reduction. |
| Additional comment | - |

| Relevant SDG Indicator/Safeguarding Principle | SDG 13: Climate Action Target: 13.3 by emission reduction Indicator: 13.3.1 |
| --- | --- |
| Data / Parameter | Air quality |
| Unit | tons/GWh |
| Description | Amount of CO and NMVOC emissions |
| Source of data | Monthly meter readings |
| Value(s) applied | According to latest official data CO and NMVOC emissions due to electricity generation in 2014 are: 0.152 tons/GWh and 0.033 tons/ GWh respectively. No quantitative information is available for projection of described emission rates. Continuation of current emission rates is estimated in case of baseline situation. Reductions of proportionate amount in described emissions during operational life of the project activity. When electricity generation amount (230.915 GWh/y ) is considered for the project activity then expected annual emission avoidances with project implementation becomes 35.2 and 7.5 tons respectively for each parameter. |
| Measurement methods and procedures | Amount of annual net electricity generation, which is calculated by monthly settlement notifications of PMUM based on monthly meter readings, will be used to calculate estimated CO and NMVOC emission reductions by project activity. |
| Monitoring frequency | Annually. |
| QA/QC procedures |  |
| Purpose of data | Emission reduction. |
| Additional comment | - |

| Relevant SDG Indicator | SDG 6- Clean Water and Sanitation Target: 6.3 by using renewable energy systems Indicator: 6.3.1 |
| --- | --- |
| Data/parameter: | Water Quality and Quantity |
| Unit | m3 |
| Description | Amount of wastewater to be discharged to the environment |
| Source of data | Monthly meter readings |
| Value(s) applied) | In 2014; 20.6 m3 wastewater is discharged the environment due to each GWh electricity produced. Avoidance of around 4,756.088 m3 wastewater discharge to the environment per year. |
| Measurement methods and procedures | Amount of annual net electricity generation, which is calculated by monthly settlement notifications of PMUM based on monthly meter readings, will be used to calculate estimated amount of avoided wastewater discharge by project activity. (Note: PMUM has been changed as EPİAŞ on 01/09/2015) Records of transfer of waste water from power plant by sewage truck, if it was performed, will be used to demonstrate proper waste water management. |
| Monitoring frequency | Anually. |
| QA/QC procedures |  |
| Purpose of data | Avoidance wastewater discharge. |
| Additional comments | - |
| Repeat for each parameter |  |
| Description | Avoidance of waste oil and domestic solid waste disposal into natural bodies by removing them in line with the regulations |
| Source of data | Removal collection receipts |
| Value(s) applied | a) Waste oil etc. (gearbox oil waste, oil filters etc.) will be collected on site and disposed by licensed companies according to legal requirements. Waste oil transfer receipts are provided in this monitoring period. b) Avoidance of domestic waste disposal into natural bodies by removing them in line with the regulations. All domestic waste is being collected in garbage bins and then they are collected by Bergama Municipality. Letter from the municipality (dated 29/11/2019) and garbage bin’s photo are provided for this monitoring period. |
| Measurement methods and procedures | Waste oil: By checking waste oil disposal invoices. Domestic waste : Letter from the municipality and garbage bins’ photos. |
| Monitoring frequency | Continuous. |
| QA/QC procedures | - |
| Purpose of data | Soil Condition |
| Additional comment |  |

| Relevant SDG Indicator/Safeguarding Principle | SDG 8- Decent Work and Economic Growth Target: 8.8 by employment and decent work Indicator: 8.8.2 |
| --- | --- |
| Data / Parameter | Quality of employment |
| Unit | - |
| Description | Quality of employment |
| Source of data | Health & Safety trainings |
| Value(s) applied | Health & Safety trainings trainings have been held during the this monitoring period for employees at the plant. Training records are provided for all employees. |
| Measurement methods and procedures | Training attendance list and/or certificates. Safety measures and equipments will be shown to DOE. |
| Monitoring frequency | Annually (Once at the end of the each monitoring period). After first verification period, only new cases will be reported. Safety measures and equipments will be shown to DOE during each site visit for verification. |
| QA/QC procedures |  |
| Purpose of data | Employment and decent work for all. |
| Additional comment | - |

| Relevant SDG Indicator/Safeguarding Principle | SDG 8- Decent Work and Economic Growth Target: 8.8 by employment and decent work Indicator: 8.8.2 |
| --- | --- |
| Data/parameter: | Quantitative employment and income generation |
| Unit | - |
| Description | Number of employment |
| Source of data | For number of local employment: social insurance registries and receipts of employees. After first verification, only changes in employees will be reported. |
| Value(s) applied | The number of local employees is equal to the number of local employees in the previous monitoring period. Totally, 11 employees are working within the plant and 5 of them are local. |
| Measurement methods and procedures | Training attendance list and/or certificates. Safety measures and equipments will be shown to VVB. |
| Monitoring frequency | Number of local employment: annually |
| QA/QC procedures |  |

| Relevant SDG Indicator/Safeguarding Principle | SDG 7: Affordable and clean energy Target: 7.2 by using renewable energy systems Indicator: 7.2.1 |
| --- | --- |
| Data / Parameter | Balance of payments and investment |
| Unit | - |
| Description | Avoidance of natural gas import. |
| Source of data | Monthly meter readings |
| Value(s) applied | According to TEİAŞ 25,426,014 thousand m3 natural gas is consumed for electricity generation in 2014. In 2014 Electricity generation amount from natural gas is 120,576 GWh and share of natural gas in the electricity mix is 47.85%. Since Turkey imports 98.6% of consumed natural gas, it is calculated that for 1 GWh electricity generation 23,294 m3 natural gas is imported in 2014. Avoidance of around 22,998 m3 natural gas import each year by generation of project activity, which is worth about 6,256,187.65 EUR per year. |
| Measurement methods and procedures | TEİAŞ statistics for natural gas share in the electricity mix shall help to demonstrate the high import dependency. Amount of annual net electricity generation, which is calculated by monthly settlement notifications of PMUM (now EPİAŞ) based on monthly meter readings, will be used to calculate correspondent amount of currency saved by project activity with help of above calculated factor. |
| Monitoring frequency | Annually. |
| QA/QC procedures |  |
| Purpose of data | Ensure access to affordable, reliable, sustainable and modern energy. |
| Additional comment | - |

| Relevant SDG Indicator | SDG 7- Affordable and Clean Energy Target: 7.2 by using renewable energy systems Indicator: 7.2.1 |
| --- | --- |
| Data/parameter: | EGfacility,y |
| Unit | MWh/yr |
| Description | Quantity of net electricity generation supplied by the project plant to the grid in year y |
| Measured/calculated/default | Measured |
| Source of data | Electricity meter(s) |
| Value(s) of monitored parameter | Months; Electricity supplied to the grid (MWh) (1); Electricity consumption from the grid (MWh) (2); Net electricity supplied to the grid[MWh] (3) =(1)-(2). May.19; 8,653.97; 23.89; 8,630.08. Jun.19; 22,076.75; 11.43; 22,065.32. Jul.19; 21,553.13; 8.16; 21,544.97. Aug.19; 31,065.79; 0.73; 31,065.06. Sep.19; 23,584.68; 9.88; 23,574.80. Oct.19; 11,550.62; 34.38; 11,516.24. Nov.19; 10,936.42; 48.49; 10,887.93. Dec.19; 17,638.73; 35.54; 17,603.19. Sum 2019; 147,060.09; 172.50; 146,887.59. Jan.20; 24,896.90; 23.60; 24,873.30. Feb.20; 20,253.46; 14.09; 20,239.37. Mar.20; 18,542.44; 14.83; 18,527.61. Apr.20; 20,383.49; 16.09; 20,367.40. Sum 2020; 84,076.31; 68.62; 84,007.69. Total Sum (01/05/2019-30/04/2020); 231,136.38; 221.11; 230,915.27. |
| Monitoring equipment | Two electricity meters will be placed (one main and one reserve) at the substation. These meters are sealed by TEIAS and intervention by project proponent is not possible. The fact that two meters are installed in a redundant manner keeps the uncertainty level of the only parameter for baseline calculation low. High data quality of this parameter is in the interest of not only the emission reduction monitoring, but paramount for the business relation between the plant operator and the electricity buyers. EPİAŞ readings will be used as a base. The above described measurement method follows Article 81 of the official regulation “Electricity Market Balancing And Settlement Regulation” EPİAŞ reading will be used as a base datum on the other hand; electricity meter reading will be used to crosscheck the accuracy of the data. Authenticity will be increased by the aid of data crosschecks between EPİAŞ datum and electricity meter readings. (Note: PMUM has been changed as EPİAŞ on 01/09/2015) However, as of 01/09/2015, EPIAS took over the PMUM procedures from TEIAS. After that date, EPIAS will supply the data. This means that the main source is the EPIAS data. TEIAS notices are used to cross-check the EPİAŞ records. TEIAS sends an electronic spreadsheet that includes daily and monthly electricity generation and withdrawn amounts for each power plant. Thus, cross-check source is the TEIAS meter readings. Since the meters are reading electricity supplied to the system and withdrawn from the system separately, the net electricity amount supplied to the grid will be calculated by electricity supplied minus electricity withdrawn. Thus, with this procedure, monitoring is sufficient and no extra monitoring has to be implemented. The specification of electricity meters are provided below: Name; Serial Number; Brand –Model; Year of Manufacture; Accuracy Class. Main Meter; 5704873; EMH-LZQJ-XC; 2014; Active C (0.5) – Reactive 2. Backup meter; 5704874; EMH-LZQJ-XC; 2014; Active C (0.5) – Reactive 2. First calibration date: 21/04/2017 Installation Date of the meters: 21/04/2017 |
| Measuring/reading/recording frequency: | Continuous measurement and at least monthly readings |
| Calculation method (if applicable): | - |
| QA/QC procedures: | According to the Article 2 of the Communiqué of Meters in Electricity Sector: ‘The meters to be used in the electricity market shall be compliant with the standards of Turkish Standards Institute or IEC and have obtained “Type and System Approval” certificate from the Ministry of Trade and Industry.’ Therefore, Ministry of Trade and Industry (Ministry) is responsible from control and calibration of the meters. Also according to Article 11 of this Communiqué, meters shall be in class of 0.5s, which means error interval for measuring is in +-0.5% range which is well acceptable according to rules. Paragraph b) of the Article 9 of the 'Regulation of Metering and Testing of Metering Systems' (Regulation) of Ministry states that: ‘ b) Periodic tests of meters of electricity, water, coal gas, natural gas and current and voltage transformers are done every 10 years.’ Therefore periodic calibration of the meters will be done every 10 years. Also according to Article 67 (page 20) of this regulation, the calibration shall be done in calibration stations which have been tested and approved by Ministry of Trade and Industry. Article 10 d) of Communiqué requires the meters shall be three phase four wire and Article 64 of Regulation clearly states how calibration shall be performed for this kind of meters. As above mentioned, the data acquisition and management and quality assurance procedures that are anyway in place, no additional procedures have to be established for the monitoring plan. |
| Purpose of data: | Calculation of emission reduction of the project |
| Additional comments: | Plant Manager will be responsible for monitoring data. |

| Relevant SDG Indicator/Safeguarding Principle | Safeguarding Principle 6.1 - Labour Rights |
| --- | --- |
| Data / Parameter | Other |
| Unit | - |
| Description | a) Average weekly working hours of the staff. b) Working conditions of the staff. c) Wage amount compared to regional/national standards for the plant staff. |
| Source of data | a) Letter from the Project Proponent is provided. b) Interviews with personnel. c) Letter from the Project Proponent is provided. |
| Value(s) applied | a) Weekly working hours of the labours in the Bergres WPP are 40 hours and do not exceed the legal period. Related document has been provided to VVB for this monitoring period. b) No negative feedback has been received from personnel or project proponent regarding with this issue for this monitoring period. c) Wage amount of staff of power plant comply with minimum wage legislation of the Republic of Turkey. Related document (payroll) has been provided to VVB for this monitoring period. |
| Measurement methods and procedures | a) By assigned technician by Plant Manager or assigned carbon consultant. b) Interviews to check whether the process is carried out properly. c) By assigned technician by Plant Manager or assigned carbon consultant. |
| Monitoring frequency | Annually |
| QA/QC procedures | - |
| Purpose of data | Respect and promote worker’s rights, promote the right to decent work, fair treatment, non-discrimination, and equal opportunity for workers. |
| Additional comment | - |

| Relevant SDG Indicator/Safeguarding Principle | Safeguarding Principle 3.3 – Community Health, Safety and Working Conditions |
| --- | --- |
| Data / Parameter | Other |
| Unit | - |
| Description | Fatal or non-fatal occupational Injuries for the plant staff. |
| Source of data | Project Proponent |
| Value(s) applied | There is no record regarding with fatal or non-fatal injuries for this monitoring period. |
| Measurement methods and procedures | By assigned technician by Plant Manager or assigned carbon consultant. |
| Monitoring frequency | Annually |
| QA/QC procedures | - |
| Purpose of data | Respect and promote worker’s rights, promote the right to decent work, fair treatment, non-discrimination, and equal opportunity for workers. |
| Additional comment | - |

  - Implementation of sampling plan

N/A

SECTION.E Calculation of SDG outcomes

E.1 Calculation of baseline value or estimation of baseline situation of each SDG outcome

Used Formulas:

FOR SDG 13

CO2  Emission Avoidance via CM calculation

According to the applied methodology version, the emission reduction is the baseline emissions calculated as the net electricity supplied to the grid multiplied by the grid emission factor. In other saying, the value of  “Combined margin CO2 emission factor for grid connected power generation in year y (EFgrid, CM,y) in needed to multiply with the value of Net electricity supplied to the Grid (EGy) in order to reach Baseline emissions (BEy). Calculations for reaching EFgrid, CM,y and (EGy) values have already been explained in PDD.

BEy = EGy x EFgrid,CM,y  (3)

Where:

BEy = Baseline emissions in tonnes CO2

EFgrid, CM,y 	= Combined margin CO2 emission factor for grid connected power generation in year y calculated using the latest version of the “Tool to calculate the emission factor for an electricity system”   [tCO2/MWh].

EGy 		= Net electricity supplied to the Grid in MWh (ID 1)

y		= Refers to a given period

CO Emission Avoidance

In order to calculate CO emission from the facility, the first step is to calculate a factor showing CO emission per GWh generated electricity. This factor is derived by dividing “Emission Amount due to Electricity Amount in related year (2014 in this case) with “Net electricity generation in the related year in GWh”. Then, the CO emission in the baseline scenario is reached by multipliying this factor with the estimated electricity generation of the WPP. This output also refers to Avoided CO Emission Amount. (Please see Bergres WPP_ER_v1_31032020 excel file.)

Calculation of CO emission :

CO Avoidance =  Emission Amount due to Electricity Amount in related year (2014 in this case)Net electricity generation in the related year in GWh ×Electrcity Gen. of Bergres WPP

NMVOC Emission Avoidance

In order to calculate NMVOC emission from the facility, the first step is to calculate a factor showing NMVOC emission per GWh generated electricity. This factor is derived by dividing “Emission Amount due to Electricity Amount in related year (2014 in this case) with “Net electricity generation in the related year in GWh”. Then, the NMVOC emission in the baseline scenario is reached by multipliying this factor with the estimated electricity generation of the WPP. This output also refers to Avoided CO Emission Amount. (Please see Bergres WPP_ER_v1_31032020 excel file.)

Calculation of NMVOC Avoidance :

NMVOC Avo. = Emission Amount due to Electricity Amount in related year (2014 in this case)Net electricity generation in the related year in GWh ×Electrcity Generation of Bergres WPP

FOR SDG 6

Wastewater Avodiance

In order to calculate wastewater avoidance from the facility (by taking cooling water discharge in thermal power plants into account), the first step is to calculate a factor showing estimated wastewater discharge per GWh generated electricity. This factor is derived by dividing “Total Waste Water Discharged by Thermal Power Plants in the related year (2014 in this case)” with “Total Electricty Generation in 2014 (GWh) in the related year in GWh”. Then, the wastewater discharge in the baseline scenario is reached by multipliying this factor with the estimated electricity generation of the WPP. This output also refers to Avoided Wastewater Discharge Amount. (Please see Bergres WPP_ER_v1_31032020 excel file.)

Calculation of Wastewater Avodiance :

Total Waste Water Discharged by Thermal Power Plants in the related year (2014)Total Electricty Generation in 2014 (GWh) in the related year  ×Elecrtricity Generation of Bergres WPP

FOR SDG 7

The project’s contribution to SDG 7 is determined via a simple calculation that provides the overall renewable energy generation fed into the grid. This approach is based on the following calculation:

Quantity of net electricity generation supplied by the project plant to the grid in the related year has been calculated by substracting the value of “Electricity consumption from the grid (MWh)” from “Electricity supplied to the grid (MWh)”

Calculation of Electricity Generation of Bergres WPP :

Electricity Generation of Bergres WPP = Electricity supplied to the grid (MWh) - Electricity consumption from the grid (MWh)

FOR SDG 8

Health and safety (HSE) training certificates for all employees will be verified.

E.2 Calculation of project value or estimation of project situation of each SDG outcome

FOR SDG 13

SDG 13 parameter has been evaluated and also explained in the registered PDD registered PDD (version 08, dated 21/09/2017) and Transition Annex (version 3, dated 07/11/2018).

CO2 = 131,241 tCO2

NMVOC = 7.7 ton/ y

CO = 36.3 ton/ y

FOR SDG 8

The employment of local people that have necessary technical qualifications for the required post will be the priority and enhanced by all project activities during construction and operation of wind farm. As a result, local poverty and unemployment will be partially eliminated by increased job opportunities and project business activities. Construction materials for the foundations, cables and other auxiliary equipment will preferentially be sourced locally. Moreover as contribution of the project to welfare of the region, the quality of the electricity

consumed in the region will be increased by local electricity production, which also contributes decreasing of distribution losses.

The number of employees will be reached via the Social Security Institution records.

FOR SDG 7

Firstly, the project has helped accelerating the growth of the wind power industry and stimulate the designation and production of renewable energy technologies in Turkey. Then, other entrepreneurs irrespective of sector have been encouraged to invest in wind power generations. It has also assisted reducing Turkey’s increasing energy deficit and diversify the electricity generation mix while reducing import dependency, especially natural gas. Importantly, rural development has been maintained in the areas around the project site by providing infrastructural investments to these remote villages.

Implementation of the proposed project has contributed to wider deployment of wind power technology in local and national level. It has demonstrated the viability of larger grid connected wind farms, which will support improved energy security, alternative sustainable energy, and also renewable energy industry development. This will also strengthen pillars of Turkish electricity supply based on ecologically sound technology.

The project’s contribution to SDG 7 is determined via a simple calculation that provides the overall renewable energy generation fed into the grid. This approach is based on the following calculation:

Quantity of net electricity generation supplied by the project plant to the grid in the related year has been calculated by substracting the value of “Electricity consumption from the grid (MWh)” from “Electricity supplied to the grid (MWh)”

Electricity Generation of Bergres WPP = 238,000 MWh/y registered PDD (version 08, dated 21/09/2017) and Transition Annex (version 3, dated 07/11/2018).

SDG 6

During operation of project activity only small amount of waste water to be discharged environment which is due to daily consumption of workers. In the project area, one septic tank has been digged for collection of wastewater. This wastewater periodically will be transferred to the wastewater treatment centers via sewage truck. Avoidance of around 4,912.3 m3 wastewater discharge to the environment per year in the registered registered PDD (version 08, dated 21/09/2017) and Transition Annex (version 3, dated 22/11/2018).

E.3 Calculation of net benefits as difference of baseline and project values or direct calculation for each SDG outcome

For SDG 13:

- Emission Reduction : 127,325 tCO2

- CO Avoidance: 35.2 tons/y

- NMVOC Avoidance: 7.5 tons/y

For SDG 8: 11 workers are employed.

For SDG 7: 230,915 MWh electricity has been generated for this verification period.

For SDG 6: 4,756.088 m3 wastewater has been avoided.

Emission reduction values for period of 01/05/2019-30/04/2020 is presented below:

| Dates | Emission Reduction (tCO2) |
| --- | --- |
| 01.05.2019 - 31.05.2019 | 4,759 |
| 01.06.2019 - 30.06.2019 | 12,167 |
| 01.07.2019 - 31.07.2019 | 11,880 |
| 01.08.2019 - 31.08.2019 | 17,129 |
| 01.09.2019 - 30.09.2019 | 12,999 |
| 01.10.2019 - 31.10.2019 | 6,350 |
| 01.11.2019 - 30.11.2019 | 6,004 |
| 01.12.2019 - 31.12.2019 | 9,706 |
| 01.01.2020 - 31.01.2020 | 13,715 |
| 01.02.2020 - 28.02.2020 | 11,160 |
| 01.03.2020 - 31.03.2020 | 10,216 |
| 01.04.2020- 30.04.2020 | 11,231 |
| Total | 127,325 |

E.4 Summary of ex-post values of each SDG outcome for the current monitoring period

| - Item | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- |
| - SDG 13 | Emission Reduction: 0 tCO2 CO2: 0 ton/ y CO: 0 ton/ y NMVOC: 0 ton/ y | Emission Reduction: 131,241 tCO2 Air Quality: CO: 36.3 tons/y NMVOC: 7.7 tons/y | Emission Reduction: 127,325 tCO2 Air Quality: CO: 35.2 tons/y NMVOC: 7.5 tons/y |
| - SDG 6 | N/A | Water Quality: With this project, no wastewater will be discharged to the environment. | No wastewater is being discharged to the environment in the project area. Avoidance of around 4,756.088 m3 wastewater discharge to the environment per year. |
| - SDG 7 | Energy is generated by using renewable energy systems throughout the project (Wind Power Plant). There is no electricity generation in the baseline scenario in which Bergres WPP is not being implemented. | The generation amount was estimated as 238,000 MWh | Energy (230,915 MWh) is generated by using renewable energy systems throughout the project (Wind Power Plant). |
| - SDG 8 | N/A | Quality of Employment: A set of trainings (eg. Health and safety (HSE) trainings, working at height, first aid) are foreseen to be given to all employees at the plant. Quantitative Employment and Income Generation: Current employment rates will be presented. | Quality of Employment: All the workers that are employed by Bergres Enerji Santrali Üretim A.Ş have taken a series of training courses including technical capacity development and personal development. Occupational health and safety trainings have been provided in 2019 and 11 employees are attended to the trainings. Training records for the employees are provided such as occupational health and safety trainings. Quantitative Employment: 11 employees work at the WPP. Out of these 11 employees, 6 have been employed by Bergres Elektrik. The rest has been employed by the other companies (Securitas, GE) from which services are taken. There are 5 local employees. Current employment records are now presented. |

E.5 Comparison of actual value of outcomes with estimates in approved PDD

| Item | Values estimated in ex ante calculation of approved PDD | Actual values achieved during this monitoring period |
| --- | --- | --- |
| - SDG 13 | Emission Reduction: 131,241 tCO2 Air Quality: CO: 36.3 tons/y NMVOC: 7.7 tons/y | Emission Reduction: 127,325 tCO2 Air Quality: CO: 35.2 tons/y NMVOC: 7.5 tons/y |
| - SDG 6 | Water Quality: With this project, no wastewater will be discharged to the environment Avoidance of around 4,912,300 m3 wastewater discharge to the environment per year. | Water Quality: No wastewater is being discharged to the environment in the project area. Avoidance of around 4,756.088 m3 wastewater discharge to the environment in this monitoring period. |
| - SDG 8 | Quality of Employment: Health and safety (HSE) training certificates for all employees are provided. Quantitative Employment and Income Generation: Current employment records are presented. There will be at least 2 local employees at the plant. | Quality of Employment: All the workers that are employed by Bergres Enerji Santrali Üretim A.Ş have taken a series of training courses including technical capacity development and personal development. Occupational health and safety trainings have been provided in 2019 and 11 employees are attended to the trainings. Training records for the employees are provided such as occupational health and safety trainings. Quantitative Employment and Income Generation: 11 employees work at the WPP. Out of these 11 employees, 6 have been employed by Bergres Elektrik. The rest has been employed by the other companies (Securitas and GE) from which services are taken. There are 5 local employees. Current employment records are now presented. |
| - SDG 7 | 7.1 - Balance of payments and investment: With this project, avoidance of around 23,294 m3 natural gas will be import each year by generation of project activity, which is worth about 6,311,043 EUR per year. 7.2 - Energy is generated by using renewable energy systems throughout the project (Wind Power Plant). There is no electricity generation in the baseline scenario in which Bergres WPP is not being implemented. The generation amount was estimated as 238,000 MWh | Balance of payments and investment: Avoidance of around 22,998 m3 natural gas is imported by generation of project activity, which is worth about 6,256,187.65 EUR. Energy (230,915 MWh) is generated by using renewable energy systems throughout the project (Wind Power Plant). |

Names of Employees

| BERGRES |  |
| --- | --- |
| Name and Surname | Local Or Not |
| Erhan Biçer | Not Local |
| Gökhan Erol | Not Local |
| Ahmet Kuşçu | Local |
| Oğuz Yorgun | Not Local |
| Cengizhan Aygibi | Not Local |
| Mustafa Ulaş | Not Local |
| SECURITAS |  |
| Name and Surname | Local Or Not |
| Aziz Korkut | Local |
| Hasan Ali Demir | Local |
| İbrahim Karakuzu | Local |
| Serdar Gökçe | Local |
| GE |  |
| Name and Surname | Local Or Not |
| Niyazi Türktür | Not Local |

Training Records of Employees

| BERGRES |  |  |
| --- | --- | --- |
| Name and Surname | Date | Training |
| Erhan Biçer | 26.03.2019/23.07.2019 | Health and Safety |
| Ahmet Kuşçu | 26.03.2019/23.07.2019 | Health and Safety |
| Gökhan Erol | 26.03.2019/23.07.2019 | Health and Safety |
| Cengizhan Aygibi | 29.05.2019/27.08.2019 | Health and Safety |
| Mustafa Ulaş | 26.03.2019/23.07.2019 | Health and Safety |
| Oğuz Yorgun | 26.03.2019/23.07.2019 | Health and Safety |
| SECURITAS |  |  |
| Name and Surname | Date | Training |
| İbrahim Karakuzu | 12.11.2019/13.11.2019 | Health and Safety |
| Aziz Korkut | 12.11.2019/13.11.2019 | Health and Safety |
| Serdar Gökçe | 12.11.2019/13.11.2019 | Health and Safety |
| Hasan Ali Demir | 12.11.2019/13.11.2019 | Health and Safety |
| GE |  |  |
| Name and Surname | Date | Training |
| Niyazi Türktür | 24.05.2019 | Health and Safety |

Occupational health and safety trainings have been provided in 2019 and 11 employees are attended to the trainings.

E.6 Remarks on difference from estimated value in approved PDD

Estimated emission reduction in ex ante calculation of registered PDD is 131,241 tCO2. Moreover, actual emission reduction achieved during this monitoring period is 127,325 tCO2. When they are compared, actual emission reduction achieved during this monitoring period is less than the estimated figure. This situation is arisen from the unexpected wind trend in the project location.  Wind potential might have changed. However, when it is compared, actual emission reduction achieved during this monitoring period is acceptable. Because, net difference between estimated and actual emission reduction for the project activity is 2.98 % and difference between this two emission reduction values is lower than 10% range calculated in the registered PDD.

An estimated electricity net generation is 238,000 MWh per year in registered PDD. In this monitoring period, 230,915 MWh of net electricity is supplied to the grid. Net difference between estimated and actual electricity generation for the project activity is 2.98 %.

SECTION F. Stakeholder inputs and legal disputes

F.1 List all inputs/grievances which have been received for the project during the monitoring period together with their respective answers/actions

For the Bergres Wind Power Project, stakeholder engagement procedure was conducted at the with the made man of Hacılar village head and the local stakeholders (from Bayramcılar and Balaban villages) via teleconference on 28th of April 2020.

As validated during the site visit, muhtar (head of the village) had already taken the contact information of Company Executive of the company so that the local stakeholders will be able to reach company executive whenever they have any complaints, suggestions or ideas about the project.

Since muhtar and made men are the head of the village, they will be the contact persons between the project owner and the local stakeholders. Muhtar will make sure that there’s continuous communication between the two parties. During the site visit, it is validated that there were no negative comments from the stakeholders.

Since this project is about the complete its 3th verification of first crediting period and it does not result in different impacts with respect to different vintages, the grievance mechanism does mainly rely on the existed grievance mechanism by the use of logbook. There would not requirement to use further grievance tools throughout the project life cycle.

In addition, logbook was checked to see whether there has been any complaints from the local stakeholders and it is seen that there were no negative comments from the local stakeholders during this monitoring period.

F.2  List all inputs/grievances from previous monitoring period where follow up action is to be verified in this monitoring period

Observations of the personnel in the site has been received and no nest or carcass in the project area was observed during the operation.

According to the labour law legislation of the Republic of Turkey, weekly working hour are specified as 45 hours. As can be seen from the document provided, weekly working hours of the labours in the Bergres WPP are 40 hours and do not exceed the legal period. As per GS4GG Safeguarding Principles & Requirements section 3.6.1 para 3, working hours must not exceed 48 hours per week on a regular basis and weekly working hours of the labours in the Bergres WPP do not exceed this period. So, working hours of power plant comply with this rule. In addition to these, related parameter has been added in the section D.2 of the MR and also necessary document has been now provided to VVB.

According to the minimum wage legislation of the Republic of Turkey, workers in all business lines must be employed with a wage amount that will not be below the minimum wage. There is no staff working below the minimum wage amount in Bergres WPP. So, wage amount of staff of power plant comply with this rule.                 A substitutive document has been provided from the project proponent. In addition to these, related parameter has been added in the section D.2 of the MR and necessary document has been now provided to VVB.

Opinions of the staff in the site have been taken via telephone and no complaints or problems were recorded regarding the working conditions. During discussion with the staffs, it has been stated their health is not put at risk and that they work under safe and suitable conditions. So, no negative feedback has been received from personnel or project proponent regarding with this issue. In addition to these, related parameter has been added in the section D.2 of the MR.

No fatal or non-fatal occupational injuries have been occurred to the plant staff. In case of such situations, a report is kept by the Plant Manager. When the field records are examined, there is no record regarding with fatal or non-fatal injuries for this monitoring period. During discussion with the project proponent, it has been stated that no fatal or non-fatal occupational injuries have been occurred for this monitoring period. So, no negative feedback has been received from plant manager or project proponent regarding with this issue. In addition to these, related parameter has been added in the section D.2 of the MR.

Disposal of waste oil, hazardous/non-hazardous materials have been monitored for this monitoring period and related parameter has been added in the section D.2 (under SDG 6) of the MR and also necessary documents (waste oil receipts, letter from Municipality and garbage bin’s photo) have been now provided to VVB.

Necessary document (logbook) is now provided to VVB. Thus, when the supporting documents are examined, it is seen that the logbook has been available and this document has been followed up regularly since the beginning of this monitoring period.

F.3 Provide details of any legal contest or dispute that has arisen with the project during the monitoring period

N.A.

- Please see; generation licence

Please see; provisional acceptance protocols of Bergres WPP

See, Generation License for coordinates and http://www.rcn.montana.edu/Resources/Converter.aspx for the conversion.

ACM0002 Version 17: https://cdm.unfccc.int/filestorage/D/5/Y/D5YFS9I3VKBT18MQNGX0LPZ6U7AWCO/ACM0002_%28v17%200%29_clean.pdf?t=OWF8cGJkMDNvfDCaXD0VeKrfjdDN1RPb4oxZ

See; https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-07-v5.0.pdf

See; http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-01-v7.0.0.pdf

See; https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-02-v6.0.pdf

See; http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-03-v2.pdf

See; http://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-10-v1.pdf

Deutsche WindGuard Bergres Energy Yield Assessment, Page 8

Related documents have been submitted to VVB.

- See: 1st Index Protocol

- See, http://www.sanayi.gov.tr/download/osgm/olcu_aletleri_muayene_yonetmelik.zip , page 2

The unit emissions are calculated as dividing emission amount for each parameter with net electricity amount. (For CO and NMVOC emission amounts  see :

http://www.tuik.gov.tr/PreHaberBultenleri.do?id=16174 http://www.tuik.gov.tr/PreHaberBultenleri.do?id=16174

Calculation is further substantiated in Monitoring Plan of CM Calculation Worksheet, which is available to DOE.

Please see Transition Annex.

The training records have been submitted to VVB.

The Social security records have been submitted to VVB.

See,  http://www.teias.gov.tr/YayinRapor/apk/projeksiyon/index.htm (2015 report, page 53, Table 32)

- See: 1st Index Protocol

See, www.epdk.org.tr/TR/Dokuman/3156

See, http://www.mevzuat.gov.tr/Metin.Aspx?MevzuatKod=7.5.6381&MevzuatIliski=0&sourceXmlSearch=

https://www.mevzuat.gov.tr/MevzuatMetin/1.5.4857.pdf (Article 63)

https://www.mevzuat.gov.tr/mevzuat?MevzuatNo=5454&MevzuatTur=7&MevzuatTertip=5