Monitoring Report

###### PUBLICATION DATE  14.10.2020
VERSION  v. 1.1 
RELATED SUPPORT - TEMPLATE GUIDE Monitoring Report v. 1.1

This document contains the following Sections

Key Project Information

SECTION A - Description of project

SECTION B - Implementation of project

SECTION C - Description of monitoring system applied by the project

SECTION D - Data and parameters

SECTION E - Calculation of SDG Impacts

SECTION F - Safeguards Reporting

SECTION G - Stakeholder inputs and legal disputes

### KEY PROJECT INFORMATION

##### Key Project Information

| GS ID (s) of Project (s) | GS 5229 |
| --- | --- |
| Title of the project (s) covered by monitoring report | Bergres Wind Power Project, Turkey |
| Version number of the PDD/VPA-DD (s) applicable to this monitoring report | 08 |
| Version number of the monitoring report | 04 |
| Completion date of the monitoring report | 27/11/2023 |
| Date of project design certification | 17/04/2017 |
| Date of Last Annual Report | - |
| Monitoring period number | 6th |
| Duration of this monitoring period | 01/05/2022 to 30/04/2023 (both days are included, 1nd Crediting period, 6thMonitoring period) |
| Project Representative | Bergres Elektrik Üretim A.Ş. |
| Host Country | Turkey |
| Activity Requirements applied | Community Services Activities Renewable Energy Activities Land Use and Forestry Activities/Risks & Capacities N/A |
| Methodology (ies) applied and version number | ACM0002 Version 17 |
| Product Requirements applied | GHG Emissions Reduction & Sequestration Renewable Energy Label N/A |

##### Table 1 - Sustainable Development Contributions Achieved

| Sustainable Development Goals Targeted | SDG Impact | Amount Achieved | Units/ Products |
| --- | --- | --- | --- |
| SDG 13 Climate Action | Emissions Reductions | ## Emission Reduction: 132,070 tCO2 ## Air Quality: ## CO: 38.37 tons/y NMVOC: 8.19 tons/y | VERs |
| SDG 8 Decent work and economic growth | The project provides employment. Trainings to be held. | 12 employees work at the WPP. There are 6 local employees. HSE trainings are given to 12 employees at the plant. Also, fire awareness and risk assessment are given to 6 employees. | Training and SGK records |
| SDG 7 Affordable and Clean Energy | MWh of renewable energy generated | 239,519.893 MWh/annually renewable energy. | MWh/y |
| SDG 7 Balance of payments and investment | Avoidance of natural gas import | Avoidance of around 16.301,93 m3 natural gas is imported by generation of project activity, which is worth about 6.489.312, 46 EUR. | m3/y |
| SDG 6 Clean water | Amount of wastewater to be discharged to the environment | Avoidance of around 4,933.348 m3 wastewater discharge to the environment per year. | m3/y |

##### Table 2 – Product Vintages

|  |  | Amount Achieved |  |  |
| --- | --- | --- | --- | --- |
| Start Dates | End Dates | VERs | … | … |
| 01/05/2022 | 31/12/2022 | 94,927 |  |  |
| 01/01/2023 | 30/04/2023 | 37,143 |  |  |

#### SECTION A. DESCRIPTION OF PROJECT

##### A.1. General description of project

Bergres Elektrik Üretim A.Ş. (hereafter referred to as “Bergres Elektrik”) is investing into a new Wind Power project called Bergres Wind Power Project (hereafter referred to as the “Project” or “Bergres WPP”), which involves installation and operation of 69,95 MWm/69,95 MWe wind power plant. The project consists of 21 turbines with 3.2 MWm/MWe and 1 turbine with 2.75 MWm/MWe capacity. Energy Market Regulatory Authority (EMRA) issued the licence of the project on February 09 of 2012. The generated energy is fed to the grid at Bergres RES transmission line.

An estimated electricity net generation of 238,000 MWh per year by the efficient utilization of the available wind energy by project activity replaces the grid electricity, which is constituted of different fuel sources, mainly fossil fuels. The electricity produced by project activity results in annual estimated emission reduction of 131,241 tonnes of CO2/year. Moreover, project activity contributes further dissemination of wind energy and extension of national power generation. The generation of electricity starts on 21/04/2017.

Crediting period of Bergres WPP started on 21/04/2017 and the crediting period is 7 years which will end on 20/04/2024. The sixth monitoring period is between 01/05/2022 to 30/04/2023. In this monitoring period, 239,519.893 MWh of net electricity is supplied to the grid and 132,070 tCO2 emission reduction has achieved.

Bergres WPP completed its transition process to Gold Standard for the Global Goals on 22/11/2018. Environmental and social monitoring parameters are revised in transition process and are connected to specific SGD parameters. Bergres WPP is now compatible with Gold Standard for the Global Goals and this monitoring report has taken revised monitoring parameters and SGDs into consideration.

The purpose of the project is to generate electricity and to feed it into the grid. Bergres WPP was registered as a Voluntary Emission Reduction project in order to enable the project implementation by means of financial inflows coming from the credits sale. Because of its significant contribution to climate protection and to sustainable development in the region, this project fulfils the requirements of the Gold Standard Organization.

The project helps Turkey to stimulate and commercialise the use of grid connected renewable energy technologies and markets. Furthermore, the project demonstrates the viability of grid connected wind farms which can support improved energy security, improved air quality, alternative sustainable energy futures, improved local livelihoods and sustainable renewable energy industry development. The specific goals of the project are to:

- reduce greenhouse gas emissions in Turkey compared to the business-as-usual scenario;

- help to stimulate the growth of the wind power industry in Turkey;

- create local employment during the construction and the operation phase of the wind farm;

- reduce other pollutants resulting from power generation industry in Turkey, compared to a business-as-usual scenario;

- help to reduce Turkeys increasing energy deficit;

- and differentiate the electricity generation mix and reduce import dependency.

As the project developer, Bergres Elektrik believes that efficient utilization of all kinds of natural resources with a harmony coupled with responsible environmental considerations is vital for sustainable development of Turkey and the World. This has been a guiding factor for the shareholders towards the concept of designation and installation of a wind power project. Other than the objective of climate change mitigation through significant reduction in greenhouse gas (GHG) emissions, the project has been carried out to provide social and economic contribution to the region in a sustainable way. The benefits that will be gained by the realization of the project compared to the business-as-usual scenario can be summarized under four main indicators:

Environmental

The project activities replaces the grid electricity, which is constituted of different fuel sources causing greenhouse gas emissions. By replacing in the consumption of these fuels, it contributes to conservation of water, soil, flora and faunas and transfers these natural resources and also the additional supply of these primary energy sources to the future generations. In the absence of the project activity, an equivalent amount of electricity would have been generated from the power plants connected to the grid, majority of which are based on fossil fuels. Thus, the project is replacing the greenhouse gas emissions (CO2, CH4) and other pollutants (SOX, NOX, particulate matters) occurring from extraction, processing, transportation and burning of fossil-fuels for power generation connected to the national grid.

Economical

Firstly, the project helps to accelerate the growth of the wind power industry and stimulate the designation and production of renewable energy technologies in Turkey. Then, other entrepreneurs irrespective of sector will be encouraged to invest in wind power generations. It also assists to reduce Turkey’s increasing energy deficit and diversify the electricity generation mix while reducing import dependency, especially natural gas. Importantly, rural development is maintained in the areas around the project site by providing infrastructural investments to these remote villages.

Social

Local employment is enhanced by all project activities during construction and operation of wind farm. As a result, local poverty and unemployment are partially eliminated by increased job opportunities and project business activities. Construction materials for the foundations, cables and other auxiliary equipment are preferentially sourced locally. Moreover, as contribution of the project to welfare of the region, the quality of the electricity consumed in the region will be increased by local electricity production, which also contributes decreasing of distribution losses.

Technological

##### Implementation of the proposed project contributes to wider deployment of wind power technology in local and national level. It demonstrates the viability of larger grid connected wind farms, which support improved energy security, alternative sustainable energy, and also renewable energy industry development. This also strengthens pillars of Turkish electricity supply based on ecologically sound technology.

##### A.2. Location of project

The host country is Republic of Turkey. Project area is in Aegean Region, İzmir province. The project is close to Bergama town. Location of the project site is presented in Figure 1.

Figure 1: Map of the Project Area

The project site is located about 17 km south-east of the town of Bergama and approximately 60 km north of the city of Izmir in western Turkey. The wind farm area is located between the villages Avunduk and Balaban in the west, Yuntdag and Bayramcılar in the south-east and Hacılar in the east. The closest villages are Bayramcılar and Balaban villages and the distance between the villages and the project area is 500m.

The coordinates of the turbines are given below. The project coordinates were provided below which are under ED 50, zone 35 coordinate system.

Figure 2. Coordinates of the Turbines

| Turbine No. | Easting | Northing |
| --- | --- | --- |
| T1 | 518750,00 | 4315500,00 |
| T2 | 518820,00 | 4315000,00 |
| T3 | 518250,00 | 4313770,00 |
| T4 | 518119,00 | 4311650,00 |
| T5 | 518243,00 | 4311226,00 |
| T6 | 519019,00 | 4311718,00 |
| T7 | 519169,00 | 4312506,00 |
| T8 | 519019,00 | 4312774,00 |
| T9 | 519770,00 | 4312470,00 |
| T10 | 520120,00 | 4312325,00 |
| T11 | 520350,00 | 4312175,00 |
| T12 | 520590,00 | 4312000,00 |
| T13 | 520768,00 | 4311832,00 |
| T14 | 521062,00 | 4311756,00 |
| T15 | 521321,00 | 4311023,00 |
| T16 | 520609,00 | 4309927,00 |
| T17 | 520759,00 | 4309697,00 |
| T18 | 520968,00 | 4309525,00 |
| T19 | 521255,00 | 4309305,00 |
| T20 | 521523,00 | 4309200,00 |
| T21 | 521819,00 | 4309625,00 |
| T22 | 522020,00 | 4309400,00 |

##### A.3. Reference of applied methodology

For the determination of the baseline, the official methodology ACM0002 version 17.0, “Large-scale Consolidated baseline methodology for grid-connected electricity generation from renewable sources”, is applied, using conservative options and data as presented in the following section. This methodology refers to five Tools, which are:

- Tool to calculate the emission factor for an electricity system (Version 05.0.0);

- Tool for the demonstration and assessment of additionality (Version 07.0.0) ;

- Combined tool to identify the baseline scenario and demonstrate additionality (Version 06.0.0);

- Tool to calculate project or leakage CO2 emissions from fossil fuel combustion (Version 02.0.0);

- Tool to determine the  lifetime of the equipment

For baseline calculation the first tool, for additionality assessment the second tool is used. As third tool is the combination of the first and second tool, it is not used. Since no project emission or leakage calculation is required for wind power project fourth tool is not used, and finally to determine the remaining lifetime of the equipment fifth tool is used.

##### A.4. Crediting period of project

21/04/2017-20/04/2024 both dates are included. (Crediting period of the project is 7 years which is renewable.)

#### SECTION B. IMPLEMENTATION OF PROJECT

##### B.1. Description of implemented project

According to the Generation License, 21 wind turbines with unit capacity of 3.2 kWm/3.2 kWe and 1 turbine with unit capacity of 2.75 kWm/kWe were selected for the project. GE Wind Energy is decided as equipment provider due to the outstanding features of its product regarding safety factors, simple durable design for low maintenance and long life operation, high efficiency, and also for fine visual appearance.

The key parameters about the technical design of the selected model GE-3.2-103 turbines are listed below in Figure 3.

Figure 3: Technical specifications of GE-3.2-103 turbines

| Specifications | GE-3.2-103 |
| --- | --- |
| Rated Power (kW) | 3200 |
| Rotor Diameter (m) | 103 |
| Hub Height (m) | 85 |
| Num. of Blades | 3 |
| Hub Height wind speed (m/s) | 8.4 |

The key parameters about the technical design of the selected model GE-2.75-103 turbines are listed below in Figure 4.

Figure 4: Technical specifications of GE-2.75-103 turbines

| Specifications | GE-2.75-103 |
| --- | --- |
| Rated Power (kW) | 2750 |
| Rotor Diameter (m) | 103 |
| Hub Height (m) | 85 |
| Num. of Blades | 3 |
| Hub Height wind speed (m/s) | 8.4 |

Selected these turbines, whose technical specifications are given above and having unit capacity of 3.2 kWm/kWe and 2.75 kWm/kWe, both have the same technical specifications. The electricity generation of the 3.2 kWm/kWe turbine is limited to 2.75 kWm/kWe. For this reason, the technical specifications of the GE-2.75-103 turbine are the same as GE-3.2-103 turbines. The only difference is that this turbine operates at a lower capacity (2750 kW) than the others.

A general operation diagram of the project is given in Figure 5.

Figure 5: Operation diagram of the project

The project installation is completed according to the description in the PDD and completely operational. The project consists only one site.

The most important milestones are included in the following table:

Figure 6.  Milestones of the Bergres Wind Power Project

| Date (DD/MM/YYYY) | Milestone |
| --- | --- |
| 10/01/2012 | EIA Not Required Certificate |
| 09/02/2012 | Issuance of the generation license |
| 12/11/2015 | Agreement with Equipment Provider (GE) (Investment decision date) |
| 18/02/2016 | Start Date of Construction |
| 28/03/2016 | Credit Agreement |
| 15/06/2016 | Agreement with Life Enerji for Carbon Consultancy |
| 17/04/2017 | Registration date |
| 20/04/2017 | First commissioning dates of the turbines |
| 21/04/2017 | Electricity generation date/Start date of first monitoring period and crediting period |
| 26/05/2017 | Last commissioning dates of the turbines |
| 30/04/2018 | End date of first monitoring period |
| 01/05/2018 | Start date of the second monitoring period |
| 16/08/2018 | Start date of the transition process |
| 22/11/2018 | End date of the transition process |
| 30/04/2019 | End date of the second monitoring period |
| 01/05/2019 | Start date of the third monitoring period |
| 30/04/2020 | End date of the third monitoring period |
| 20/04/2024 | End date of the first crediting period |
| 01/05/2020 | Start date of the fourth monitoring period |
| 30/04/2021 | End date of the fourth monitoring period |
| 01/05/2021 | Start date of the fifth monitoring period |
| 30/04/2022 | End date of the fifth monitoring period |
| 01/05/2022 | Start date of the sixth monitoring period |
| 30/04/2023 | End date of the sixth monitoring period |

B.1.1 Forward Action Requests

##### VVB shall interview with the stakeholders from all villagers and check whether any complaint is received.

Response: All the mukhtar (Balaban, Hacılar, Avunduk, Bayramcılar) and stakeholders from villages interviewed to verify the project.

##### B.2. Post-Design Certification changes

B.2.1. Temporary deviations from the approved Monitoring & Reporting Plan, methodology or standardized baseline

N/A

B.2.2. Corrections

N/A

B.2.3. Changes to start date of crediting period

N/A

B.2.4. Permanent changes from the Design Certified monitoring plan, applied methodology or applied standardized baseline

N/A

B.2.5. Changes to project design of approved project

N/A

#### SECTION C. DESCRIPTION OF MONITORING SYSTEM APPLIED BY THE PROJECT

Data handling was slightly revised due to innovation in technology that enables TEİAŞ to read meters remotely. Thus, under this process, the meter is read remotely at the end of every month and published through website of TEİAŞ which can be accessed by project owner to check the correctness of the data. Besides, project owner carries monthly protocol to cross-check the data. By this way, a cross check is done. Once the meter data is correct then the amount is also published in EPİAŞ website that is also accessible by project owner. When all data is correct, an invoice (receipt of sale) is prepared by Bergres Elektrik and delivered to TEİAŞ. The electricity generation amount, the collected data, are kept in both hardcopy and softcopy in daily and monthly basis indefinitely. Moreover, these data are submitted to several authorized people. Thus, several additional copies of these data are stored at different computers. The project owner will keep the collected data during the monitoring period at least two years after the last issuance of VERs.

As the necessary baseline emission factors are all defined ex ante, the only information to be monitored is the amount of electricity fed into the grid by Bergres WPP. The data which is base for the invoices and published in EPİAŞ website is used as input for emission reduction calculation.

Electricity transfer from turbine to transmission line can be seen in Figure 6. SCADA system and remote control of meters can be seen from Figure 6 and monitoring system can be visualized.

Figure 7. Line Diagram of the Bergres WPP

Note: On 01/09/2015, PMUM was renamed as EPİAŞ.

The fact that two reliable best practice meters are installed in a redundant manner keeps the uncertainty level of the only parameter for baseline calculation low. High data quality of this parameter is in the interest of not only the emission reduction monitoring, but paramount for the business relation between the plant operator and the electricity buyer. Since the meters are reading electricity supplied to the system and withdrawn from the system separately, the net electricity amount supplied to the grid will be calculated by electricity supplied minus electricity withdrawn that will be taken from monthly settlement notifications.

Monthly data from TEİAS are stored electronically on TEİAS website and this data can be accessed by login of Project Developer. Bergres Elektrik also stores a hardcopy of the monthly meter reading protocols for back up of systematic storage of data.

Moreover, there are always internal reviews of the meters data that is checked by different parties. First of all, data of the meters is collected by technicians daily in written forms. The data collected daily is saved in plant manager computer and backed up. Besides the data that can be get from meters, production amount can be checked from SCADA system of GE. SCADA figures differ a bit with meter data due to internal losses.

Operation and management diagram is presented in Figure 8.

Figure 8 : Operation and management diagram of the Bergres WPP

The calibration of the monitoring equipment was carried out according to the information provided in the GS-VER PDD (version 08, dated 21/09/2017). The GS-VER PDD mainly includes the following obligation for the calibration of the appropriate meters:

“The Turkish Electricity Market Regulation Agency (EPDK) sets rules on the accuracy of electricity meters that are used by power plants feeding into the grid. The rules are part of the EPDK regulation 25056 from 22 March 2003. The table in Article 11 of the regulation specifies the use of electricity meters of the accuracy class 0.5S for power plants between 10 MW and 100 MW and refers to compliance with International Electro technical Commission’s norm EN 60687.  TEIAŞ conducts remote reading. And another technician, employee of Bergres Elektrik, conducts reading from the project site and sends to TEIAŞ. By this way, a cross-check is done. Also, with the readings from project site, continuously these two meters are compared with each other and if any differences are detected the necessary control measures are taken.”

The specification of electricity meters are provided below:

Figure 9. Specification of the Electricity Meters

| Name | Serial Number | Brand –Model | Accuracy Class | Year of Manufacture |
| --- | --- | --- | --- | --- |
| Main Meter | 5704873 | EMH-LZQ-XC | Active C (0.5) – Reactive 2 | 2016 |
| Backup meter | 5704874 | EMH-LZQ-XC | Active C (0.5) – Reactive 2 | 2016 |

Installation and First Calibration Date of the meters: 21/04/2017

According to the Article 2 of the Communiqué: ‘The meters to be used in the electricity market shall be compliant with the standards of Turkish Standards Institute or IEC and have obtained “Type and System Approval” certificate from the Ministry of Trade and Industry.’ Therefore, Ministry of Trade and Industry (Ministry) is responsible from control and calibration of the meters.

paragraph b) of the Article 9 of the 'Regulation of Metering and Testing of Metering Systems' 7 (Regulation) of Ministry states that: ‘b) Periodic tests of meters of electricity, water, coal gas, natural gas and current and voltage transformers are done every 10 years’. Therefore, periodic calibration of the meters will be done every 10 years. Therefore, the next calibration will be applied in 2027.

As stated in the related letter of TEİAŞ dated 2022; It is stated that the periodic inspection and testing of the meters should be done every 2 years. The last periodic control of the electricity meters have been done at: 25/09/2022

As the measuring devices are sealed by TEIAŞ, Bergres cannot intervene with the devices. In case of unforeseen problems or failures of the meters or if any differences occur between primary and secondary devices TEIAŞ has to be informed for necessary maintenance and calibration. There is an agreement between Bergres and TEIAŞ that in case of problems or failures of the meters TEIAŞ reacts as fast as possible to solve the problem.

For this monitoring period, turbines did not stop and there have not been an accident or an incident, which may affect the electricity generation of the project. Also, test of the electricity meter was not performed for this monitoring period.

#### SECTION D. DATA AND PARAMETERS

##### D.1. Data and parameters fixed ex ante or at renewal of crediting period

| Data/parameter: | EFgrid,CM,y (SDG 13) |
| --- | --- |
| Unit | tCO2/MWh |
| Description | Combined margin CO2 emission factor for grid connected power generation in year y calculated using the “Tool to calculate the emission factor for an electricity system (Version 05.0.0)” |
| Source of data | Tool to calculate the emission factor for an electricity system (Version 05.0.0) |
| Value(s) applied) | 0.5514 tCO2/MWh |
| Measurement methods and procedures | Combined Margin Emission Factor (EFgrid,CM,y) is calculated ex ante as the weighted average CO2 of Operating Margin Emission Factor (EFgrid,OM,y) and Build Margin Emission Factor (EFgrid,BM,y). Please see registered PDD and Transition Annex document. |
| Monitoring frequency | Annually |
| QA/QC procedures | - |
| Purpose of data | Calculation of baseline emissions |
| Additional comments | Other parameters are not specified as they are used in calculations in accordance with the registered PDD. Please see registered PDD and Transition Annex document. |

##### D.2 Data and parameters monitored

| Data / Parameter | Air quality (SDG 13) |
| --- | --- |
| Unit | tons of CO2e |
| Description | Amount of CO2 emissions |
| Source of data | Monthly meter readings |
| Value(s) applied | Reductions of proportionate amount in described emissions during operational life of the project activity. When electricity generation amount (239,519.893 MWh/y) is considered for the project activity then annual CO2 emission avoidances with project implementation becomes 132,070 tCO2e/yr respectively. |
| Measurement methods and procedures | Amount of annual net electricity generation, which is calculated by monthly settlement notifications of EPIAS based on monthly meter readings, will be used to calculate estimated CO2 emission reduction by project activity. |
| Monitoring frequency | Annually |
| QA/QC procedures |  |
| Purpose of data | Emission reduction. |
| Additional comment | - |

| Data/parameter: | Air quality (emissions other than GHGs) (SDG 13) |
| --- | --- |
| Unit | tons/GWh |
| Description | Amount of CO and NMVOC emissions |
| Source of data | Monthly meter readings |
| Value(s) applied) | According to latest official data CO and NMVOC emissions due to electricity generation in 2014 are: 0.152 tons/GWh and 0.033 tons/ GWh respectively. Reductions of proportionate amount in described emissions during operational life of the project activity. When electricity generation amount (239.519 GWh/y) is considered for the project activity then expected annual emission avoidances with project implementation becomes 38.37 and 8.19 tons respectively for each parameter. |
| Measurement methods and procedures | Amount of annual net electricity generation, which is calculated by monthly settlement notifications of EPİAŞ based on monthly meter readings, will be used to calculate estimated CO and NMVOC emission reduction by project activity. |
| Monitoring frequency | Annually |
| QA/QC Procedures |  |
| Purpose of data | Emission reduction. |
| Additional comments | - |

| Data / Parameter | Quality of employment (SDG 8) |
| --- | --- |
| Unit | - |
| Description | Quality of employment |
| Source of data | Health & Safety trainings |
| Value(s) applied | Health & Safety trainings have been held during this monitoring period for 12 employees at the plant. Training records are provided for 12 employees. |
| Measurement methods and procedures | Training attendance list and/or certificates. Safety measures and equipments will be shown to VVB. |
| Monitoring frequency | Annually (Once at the end of each monitoring period). After first verification period, only new cases will be reported. Safety measures and equipments will be shown to VVB during each site visit for verification. |
| QA/QC procedures |  |
| Purpose of data | Employment and decent work for all. |
| Additional comment | - |

| Data / Parameter | Quantitative employment and income generation (SDG 8) |
| --- | --- |
| Unit | - |
| Description | Number of employees |
| Source of data | For number of local employment: social insurance registries and receipts of employees. After first verification, only changes in employees will be reported. |
| Value(s) applied | Totally, 12 employees are working within the plant and 6 of them are local. |
| Measurement methods and procedures | Training attendance list and/or certificates. Safety measures and equipments will be shown to VVB. |
| Monitoring frequency | Number of local employment: annually |
| QA/QC procedures |  |
| Purpose of data | Employment and decent work for all. |
| Additional comment | - |

| Data/parameter: | EGfacility,y |
| --- | --- |
| Unit | MWh/yr |
| Description | Quantity of net electricity generation supplied by the project plant to the grid in year y |
| Measured/calculated/default | Measured |
| Source of data | Electricity meter(s) |
| Value(s) of monitored parameter | Months; Electricity supplied to the grid (MWh) (1); Electricity consumption from the grid (MWh) (2); Net electricity supplied to the grid [MWh] (3) = (1)-(2). May.22; 13,666.880; 22.420; 13,644.460. June.22; 24,377.410; 4.320; 24,373.090. July.22; 36,247.370; 0.210; 36,247.160. Aug.22; 21,572.610; 15.430; 21,557.180. Sept.22; 15,137.710; 10.560; 15,127.150. Oct.22; 26,633.530; 9.330; 26,624.200. Nov.22; 20,103.780; 7.770; 20,096.010. Dec.22; 14,522.220; 33.670; 14,488.550. Jan.23; 16,057.810; 29.090; 16,038.720. Feb.23; 20,098.090; 11.280; 20,086.810. Mar.23; 19,671.340; 15.600; 19,655.740. Apr.23; 11,610.540; 19.700; 11,590.840. Sum; 239,699.290; 179.380; 239,529.800. |
| Measurement methods and procedures | Two electricity meters will be placed (one main and one reserve) at the substation. These meters are sealed by TEIAS and intervention by project proponent is not possible. The fact that two meters are installed in a redundant manner keeps the uncertainty level of the only parameter for baseline calculation low. High data quality of this parameter is in the interest of not only the emission reduction monitoring, but paramount for the business relation between the plant operator and the electricity buyers. EPİAŞ readings will be used as a base. The above described measurement method follows Article 81 of the official regulation “Electricity Market Balancing And Settlement Regulation” . EPİAŞ reading will be used as a base datum on the other hand; electricity meter reading will be used to crosscheck the accuracy of the data. Authenticity will be increased by the aid of data crosschecks between EPİAŞ datum and electricity meter readings. (Note: PMUM has been changed as EPİAŞ on 01/09/2015) However, as of 01/09/2015, EPIAS took over the PMUM procedures from TEIAS. After that date, EPIAS will supply the data. This means that the main source is the EPIAS data. TEIAS notices are used to cross-check the EPİAŞ records. TEIAS sends an electronic spreadsheet that includes daily and monthly electricity generation and withdrawn amounts for each power plant. Thus, cross-check source is the TEIAS meter readings. Since the meters are reading electricity supplied to the system and withdrawn from the system separately, the net electricity amount supplied to the grid will be calculated by electricity supplied minus electricity withdrawn. Thus, with this procedure, monitoring is sufficient and no extra monitoring has to be implemented. The specification of electricity meters are provided below: Name; Serial Number; Brand –Model; Year of Manufacture; Accuracy Class. Main Meter; 5704873; EMH-LZQJ-XC; 2016; Active C (0.5) – Reactive 2. Backup meter; 5704874; EMH-LZQJ-XC; 2016; Active C (0.5) – Reactive 2. Installation Date of the meters: 21/04/2017 |
| Measuring/reading/recording frequency: | Continuous measurement and at least monthly readings |
| Calculation method (if applicable): | - |
| QA/QC procedures: | According to the Article 2 of the Communiqué of Meters in Electricity Sector: ‘The meters to be used in the electricity market shall be compliant with the standards of Turkish Standards Institute or IEC and have obtained “Type and System Approval” certificate from the Ministry of Trade and Industry.’ Therefore, Ministry of Trade and Industry (Ministry) is responsible from control and calibration of the meters. Also according to Article 11 of this Communiqué, meters shall be in class of 0.5s, which means error interval for measuring is in +-0.5% range which is well acceptable according to rules. Paragraph b) of the Article 9 of the 'Regulation of Metering and Testing of Metering Systems' (Regulation) of Ministry states that: ‘ b) Periodic tests of meters of electricity, water, coal gas, natural gas and current and voltage transformers are done every 10 years.’ Therefore periodic calibration of the meters will be done every 10 years. Also according to Article 67 (page 20) of this regulation, the calibration shall be done in calibration stations which have been tested and approved by Ministry of Trade and Industry. Article 10 d) of Communiqué requires the meters shall be three phase four wire and Article 64 of Regulation clearly states how calibration shall be performed for this kind of meters. As above mentioned, the data acquisition and management and quality assurance procedures that are anyway in place, no additional procedures have to be established for the monitoring plan. As stated in the related letter of TEİAŞ dated 2022; It is stated that the periodic inspection and testing of the meters should be done every 2 years. The last periodic control of the electricity meters have been done at: 25/09/2022 |
| Purpose of data: | Calculation of emission reduction of the project |
| Additional comments: | Plant Manager will be responsible for monitoring data. |

| Data / Parameter | Water Quality and Quantity (SDG 6) |
| --- | --- |
| Unit | m3 |
| Description | Amount of wastewater to be discharged to the environment |
| Source of data | Institution of Turkey Statistics |
| Value(s) applied | In 2014; 20.6 m3 wastewater is discharged the environment due to each GWh electricity produced. Avoidance of around 4,933.348 m3 wastewater discharge to the environment per year. |
| Measurement methods and procedures | It does not provide cooling water to the receiving environment since the project does not need cooling water like thermic, which is a WPP. In addition, the wastewater from the project site is collected and sent with a sewage truck, which does not harm the environment. |
| QA/QC procedures | - |
| Purpose of data | Avoidance wastewater discharge. |
| Additional comment | - |

| Data / Parameter | Safeguarding Principle: 9.4 Release of pollutants and 9.5 Hazardous and Non-hazardous Waste |
| --- | --- |
| Unit | - |
| Description | Avoidance of waste oil and domestic solid waste disposal into natural bodies by removing them in line with the regulations |
| Source of data | Removal collection receipts |
| Value(s) applied | a) Waste oil etc. (gearbox oil waste, oil filters etc.) is collected on site and disposed by licensed companies according to legal requirements. Waste declaration form is provided in this monitoring period. b) Avoidance of domestic waste disposal into natural bodies by removing them in line with the regulations. All domestic waste is being collected in garbage bins and then they are collected by Bergama Municipality. Letter from the municipality (dated 02/08/2022) and garbage bin’s photo are provided for this monitoring period. |
| Measurement methods and procedures | a)Waste oil: By checking waste oil disposal invoices. b)Domestic waste : Letter from the municipality and garbage bins’ photos. |
| Monitoring frequency | On each verification |
| QA/QC procedures |  |
| Purpose of data | Soil Condition |
| Additional comment | - |

| Data / Parameter | Balance of payments and investment (SDG 7) |
| --- | --- |
| Unit | - |
| Description | Avoidance of natural gas import. |
| Source of data | Monthly meter readings |
| Value(s) applied | According to TEİAŞ 22,932,272 thousand m3 natural gas is consumed for electricity generation in 2021. In 2021 Electricity generation amount from natural gas is 111,180.80GWh and share of natural gas in the electricity mix is 33.22%. Since Turkey imports 99.33% of consumed natural gas, it is calculated that for 1 GWh electricity generation 99.33 m3 natural gas is imported in 2021. Avoidance of around 16,301.93 m3 natural gas import each year by generation of project activity, which is worth about 6.489.312,46 EUR per year. |
| Measurement methods and procedures | TEİAŞ statistics for natural gas share in the electricity mix shall help to demonstrate the high import dependency. Amount of annual net electricity generation, which is calculated by monthly settlement notifications of PMUM (now EPİAŞ) based on monthly meter readings, will be used to calculate correspondent amount of currency saved by project activity with help of above calculated factor. |
| Monitoring frequency | Annually. |
| QA/QC procedures |  |
| Purpose of data | Ensure access to affordable, reliable, sustainable and modern energy. |
| Additional comment | - |

##### D.3. Comparison of monitored parameters with last monitoring period

| Data/Parameter | Value obtained in this monitoring period | Value obtained last monitoring period |
| --- | --- | --- |
| N/A | N/A | N/A |

This section is Not Applicable since no community service activities are involved in this project activity

##### D.4. Implementation of sampling plan

N/A

#### SECTION E. CALCULATION OF SDG IMPACTS

##### E.1. Calculation of baseline value or estimation of baseline situation of each SDG Impact

Used formulas;

For SDG 13

CO2 Emission Avoidance via CM calculation

According to the applied methodology version, the emission reduction is the baseline emissions calculated as the net electricity supplied to the grid multiplied by the grid emission factor. In other saying, the value of  “Combined margin CO2 emission factor for grid connected power generation in year y (EFgrid, CM,y) in needed to multiply with the value of Net electricity supplied to the Grid (EGy) in order to reach Baseline emissions (BEy). Calculations for reaching EFgrid, CM,y and (EGy) values have already been explained in PDD.

BEy = EGy x EFgrid,CM,y  (3)

Where:

BEy = Baseline emissions in tonnes CO2

EFgrid, CM,y 	= Combined margin CO2 emission factor for grid connected power generation in year y calculated using the latest version of the “Tool to calculate the emission factor for an electricity system”   [tCO2/MWh].

EGy 		= Net electricity supplied to the Grid in MWh (ID 1)

y		= Refers to a given period

The project activity is the installation of a new grid-connected renewable power plant so,

ERy = BEy = EGy * EFgrid,CM,y = 239,519.893 MWh/year * 0.5514 tCO2/MWh = 132,070 tCO2/year

CO Emission Avoidance

In order to calculate CO emission from the facility, the first step is to calculate a factor showing CO emission per GWh generated electricity. This factor is derived by dividing “Emission Amount due to Electricity Amount in related year (2021 in this case) with “Net electricity generation in the related year in GWh”. Then, the CO emission in the baseline scenario is reached by multipliying this factor with the estimated electricity generation of the WPP. This output also refers to Avoided CO Emission Amount. (Please see BERGRES WPP_ER_v2_080623 excel file.)

Calculation of CO emission :

CO Avoidance =  Emission Amount due to Electricity Amount in related year (2014 in this case)Net electricity generation in the related year in GWh ×Electrcity Gen. of Bergres WPP

NMVOC Emission Avoidance

In order to calculate NMVOC emission from the facility, the first step is to calculate a factor showing NMVOC emission per GWh generated electricity. This factor is derived by dividing “Emission Amount due to Electricity Amount in related year (2014 in this case) with “Net electricity generation in the related year in GWh”. Then, the NMVOC emission in the baseline scenario is reached by multipliying this factor with the estimated electricity generation of the WPP. This output also refers to Avoided CO Emission Amount. (Please see BERGRES WPP_ER_v2_080623 excel file.)

Calculation of NMVOC Avoidance :

Then, the emissions of these compounds are calculated as follow:

| Emission Amount by Project Activity per (tons) | = | Electricity Generation of Project Activity | x | Emission per GWh (tons/GWh) |
| --- | --- | --- | --- | --- |

This yields as:

- 38.37 tons/year for CO

- 8.19   tons/year for NMVOC

For SDG 6

Wastewater Avodiance

In order to calculate wastewater avoidance from the facility (by taking cooling water discharge in thermal power plants into account), the first step is to calculate a factor showing estimated wastewater discharge per GWh generated electricity. This factor is derived by dividing “Total WasteWater Discharged by Thermal Power Plants in the related year (2021 in this case)” with “Total Electricty Generation in 2021 (GWh) in the related year in GWh”. Then, the wastewater discharge in the baseline scenario is reached by multipliying this factor with the estimated electricity generation of the WPP. This output also refers to Avoided Wastewater Discharge Amount. (Please see BERGRES WPP_ER_v2_080623 excel file.)

Calculation of Wastewater Avodiance :

Total Waste Water Discharged by Thermal Power Plants in the related year (2014)Total Electricty Generation in 2014 (GWh) in the related year  ×Elecrtricity Generation of Bergres WPP

Baselinewastewater = 4,933.348 m3/y

FOR SDG 7

Renewable Electricity Generation

The project’s contribution to SDG 7 is determined via a simple calculation that provides the overall renewable energy generation fed into the grid. This approach is based on the following calculation:

Quantity of net electricity generation supplied by the project plant to the grid in the related year has been calculated by substracting the value of “Electricity consumption from the grid (MWh)” from “Electricity supplied to the grid (MWh)”

Calculation of Electricity Generation of Bergres WPP:

Electricity Generation of Bergres WPP = Electricity supplied to the grid (MWh) - Electricity consumption from the grid (MWh)

Baselineelectricity generation = 0

Balance of payments and investment:

Amount of Avoided Natural Gas Imported by Project Activity per year and also, during monitoring period can be calculated as well:

| Amount of Avoided Imported Natural Gas by Project Activity per year (m3) | = | Net Annual Electricity Generation of Project Activity per year (GWh/y) | x | Amount of Avoided Imported Natural Gas by each GWh of Project Electricity Generation in 2021 (m3/GWh) |
| --- | --- | --- | --- | --- |

By considering the natural gas in baseline year the avoided money for natural gas use is calculated as follow:

| Amount of Avoided Money for Imported Natural Gas by Project Activity during the MP (€) | = | Amount of Avoided Imported Natural Gas by Project Activity Monitoring Period (m3) | x | Natural gas unit price (€/m3) |
| --- | --- | --- | --- | --- |

Baselinenatural gas = 16,301.93 m3

##### SDG 8

If this power plant has not been installed, there would be no employment in the project area or around the project. So, employment opportunity will be provided with this project.

Without this project, there would be no employment and no training. Therefore, it would not be possible to contribute to education. It is aimed to provide training to both local employees and all workers working in the plant. The objective is to be achieved by ensure that everyone is trained in the plant. Health and safety (HSE) training certificates for all employees will be verified and also, all employees are to be trained according to the requirements.

Baselineemployment,training = 0

##### E.2. Calculation of project value or estimation of project situation of each SDG Impact

FOR SDG 13

CO2 Emissions:

The proposed project activity involves the generation of electricity by development of a wind farm. The generation of electricity does not result in greenhouse gas emissions and therefore is taken as 0 tCO2/year.

Indirect GHG Emissions- Air Quality:

There is not CO and NMVOC emissions due to the project activity.

FOR SDG 8

The employment of local people that have necessary technical qualifications for the required post has been the priority and enhanced by all project activities during construction and operation of wind farm. As a result, local poverty and unemployment has been partially eliminated by increased job opportunities and project business activities. Moreover, as contribution of the project to welfare of the region, the quality of the electricity consumed in the region has been increased by local electricity production, which also contributes decreasing of distribution losses.

In this monitoring period, 12 workers have been employed. 6 of them were locally performed. Also, HSE trainings have been given to 12 employees at the plant.

At this monitoring period, Risk Assessment, Fire Awareness and Health and Safety trainings have been provided to employees. In addition, the LOTO training was not provided to all employees is that LOTO training is a one-time training given to employees when they first get a job. During this monitoring period, no LOTO training was given as no new recruits were hired.

FOR SDG 7

Renewable Electricity Generation

Firstly, the project has helped accelerating the growth of the wind power industry and stimulate the designation and production of renewable energy technologies in Turkey. Then, other entrepreneurs irrespective of sector have been encouraged to invest in wind power generations. It has also assisted reducing Turkey’s increasing energy deficit and diversify the electricity generation mix while reducing import dependency, especially natural gas. Importantly, rural development has been maintained in the areas around the project site by providing infrastructural investments to these remote villages.

Implementation of the proposed project has contributed to wider deployment of wind power technology in local and national level. It has demonstrated the viability of larger grid connected wind farms, which will support improved energy security, alternative sustainable energy, and also renewable energy industry development. This will also strengthen pillars of Turkish electricity supply based on ecologically sound technology.

The project’s contribution to SDG 7 is determined via a simple calculation that provides the overall renewable energy generation fed into the grid. This approach is based on the following calculation:

Quantity of net electricity generation supplied by the project plant to the grid in the related year has been calculated by substracting the value of “Electricity consumption from the grid (MWh)” from “Electricity supplied to the grid (MWh)”

For Bergres WPP: This yields as 239,519.893 MWh/year.

Access to Investment

The amount of Avoided Natural Gas Imported by Project Activity per year and, during the monitoring period can be calculated as below.

# Amount of Avoided Imported Natural Gas by Project Activity m3year=Net Annual Electricity Generation of Project Activity GWhyear*Amount of Avoided Imported Natural Gas by each GWh of Project Electricity Generation in 2012 (m3GWh)

For Bergres WPP: This yields as 7,937,958.79m3 natural gas per year and for this Monitoring Period.

Regarding the natural gas in the baseline year, the avoided money for natural gas use is calculated as follow;

# Amount of Avoided Money for Imported Natural Gas by Project Activity during the Monitorinf Period €=Amount of Avoided Imported Natural Gas by Project Activity during Monitoring Period m3*Natural Gas unit price in 2012 (€m3)

For Bergres WPP: The overall results are 2,756,140.47 €/year and 2,756,140.47 €/Monitoring Period.

Balance of payments and investment:

For Bergres WPP: This yields as 16,301.93 m3 natural gas per year.

For Bergres WPP: The overall results are € 6.489.312,46 /year

FOR SDG 6

During operation of project activity only small amount of wastewater to be discharged environment which is due to daily consumption of workers. In the project area, one septic tank has been digged for collection of wastewater. This wastewater periodically will be transferred to the wastewater treatment centers via sewage truck.

Quantity of net avoidance of wastewater discharge to the environment has been calculated by subtracting the value of amount of wastewater discharged by the project from total amount of wastewater avoided.

In this monitoring period there is no need to discharge wastewater. The calculation of domestic wastewater discharge that is occurred from the project activity is followed below.

Amount of Domestic Wastewater Discharged by Project Activity per year m3year=Number of Employees person*Daily water usage per employee m3personday*365 day/year

For Bergres WPP: Avoidance of around 4,933.348m3 wastewater discharge to the environment for this monitoring period.

##### E.3. Calculation of leakage

LEy is 0, as it is not considered according to ACM0002(version 17) page 21. PEy is 0 because the project is a wind power generation activity (Only for geothermal and solar thermal projects, it should be considered according to ACM0002 page21).

##### E.4. Calculation of net benefits or direct calculation for each SDG Impact

| SDG | SDG Impact | Baseline estimate | Project estimate | Net benefit |
| --- | --- | --- | --- | --- |
| 13 | CO2 Emission Reduction & Air Quality | 132,070 tCO2 CO: 38.37 tons/y NMVOC: 8.19 tons/y | 0 tCO2 CO: 0 ton/ y NMVOC: 0 ton/ y | 132,070 tCO2 CO: 38.37 tons/y NMVOC: 8.19 tons/y |
| 8 | Quantitative Employ-ment And Income Genera-tion & Quality of Employ-ment | Quantitative Employment: No employment. Quality of Employment: No training | Quantitative Employment: 12 employees work at the WPP. Out of these 12 employees, 6 have been employed by Bergres Elektrik. The rest has been employed by the other companies (Securitas, GE) from which services are taken. There are 6 local employees. Quality of Employment: Occupational health and safety trainings have been provided in 2022/2023 and 12 employees are attended to the HSE trainings. | Quantitative Employment: 12 employees work at the WPP. Out of these 12 employees, 6 have been employed by Bergres Elektrik. The rest has been employed by the other companies (Securitas, GE) from which services are taken. There are 6 local employees. Current employment records are now presented. Quality of Employment: Occupational health and safety trainings have been provided in 2022/2023 and 12 employees are attended to the HSE trainings |
| 7 | Electricity Generation | Energy is generated by using renewable energy systems throughout the project. There is no electricity generation in the baseline scenario in which Bergres WPP is not being implemented. | Energy (239,519.893 MWh) is generated by using renewable energy systems throughout the project (Wind Power Plant). | Energy (239,519.893 MWh) is generated by using renewable energy systems throughout the project (Wind Power Plant). |
| 7 | Balance of payments and investment | No Natural Gas Use | Avoidance of around 16.301,93 m3 natural gas is imported by generation of project activity, which is worth about 6.489.312,46 EUR. | Avoidance of around 16.301,93 m3 natural gas is imported by generation of project activity, which is worth about 6.489.312,46 EUR. |
| 6 | Amount of wastewater to be discharged to the environment | No Wastewater avoidance | Avoidance of around 4,933.348m3 wastewater discharge to the environment per year. | Avoidance of around 4,933.348m3 wastewater discharge to the environment per year. |

##### E.5. Comparison of actual SDG Impacts with estimates in approved PDD

| SDG | Values estimated in ex ante calculation of approved PDD for this monitoring period | Actual values achieved during this monitoring period |
| --- | --- | --- |
| 13 | Emission Reduction: 131,241 tCO2 Air Quality: CO: 36.3 tons/y NMVOC: 7.7 tons/y | Emission Reduction: 132,070 tCO2 Air Quality: CO: 38.37 tons/y NMVOC: 8.19 tons/y |
| 8 | Quality of Employment: Health and safety (HSE) training certificates for all employees are provided. Quantitative Employment and Income Generation: Current employment records are presented. There will be at least 2 local employees at the plant. | Quality of Employment: All the workers that are employed by Bergres Elektrik Üretim A.Ş have taken Occupational health and safety trainings in 2022/2023 and 12 employees are attended to the trainings. Training records for the employees are provided. Quantitative Employment and Income Generation: 12 employees work at the WPP. Out of these 12 employees, 6 have been employed by Bergres Elektrik. The rest has been employed by the other companies (Securitas and GE) from which services are taken. There are 6 local employees. Current employment records are now presented. |
| 7 | Energy is generated by using renewable energy systems throughout the project (Wind Power Plant). This generation amount was estimated as 238,000 MWh. Balance of payments and investment: With this project, avoidance of around 23,294 m3 natural gas will be import each year by generation of project activity, which is worth about 6,311,043 EUR per year | Energy (239,519.893 MWh) is generated by using renewable energy systems throughout the project (Wind Power Plant). Balance of payments and investment: Avoidance of around 16.301,93 m3 natural gas is imported by generation of project activity, which is worth about 6.489.312,46 EUR. |
| 6 | Water Quality: With this project, no wastewater will be discharged to the environment Avoidance of around 4,912.300 m3 wastewater discharge to the environment per year. | Water Quality: No wastewater is being discharged to the environment in the project area. Avoidance of around 4,933.348m3 wastewater discharge to the environment in this monitoring period. |

Names of Employees:

| BERGRES |  |
| --- | --- |
| Name and Surname | Local Or Not |
| Gökhan EROL | Not Local |
| Ahmet KUŞCU | Local |
| Mustafa ULAŞ | Not Local |
| Mustafa UYSAL | Not Local |
| Emin ÇAKAR | Not Local |
| Öncül ELBER | Not Local |
| SECURITAS |  |
| Name and Surname | Local Or Not |
| Aziz Korkut | Local |
| Hasan Ali Demir | Local |
| İbrahim Karakuzu | Local |
| Serdar Gökçe | Local |
| GE |  |
| Name and Surname | Local Or Not |
| Niyazi Türktür | Not Local |
| Özgür Gergin | Local |

Training records of employees:

| BERGRES |  |  |  |
| --- | --- | --- | --- |
| Name and Surname | Date |  | Training |
| Gökhan EROL | 27.03.2023 |  | Health and Safety |
| Ahmet KUŞCU |  |  |  |
| Mustafa ULAŞ |  |  |  |
| Mustafa UYSAL |  |  |  |
| Emin ÇAKAR |  |  |  |
| Öncül ELBER |  |  |  |
| Gökhan EROL | 28/06/2022 |  | Fire Awareness |
| Ahmet KUŞCU |  |  |  |
| Mustafa ULAŞ |  |  |  |
| Mustafa UYSAL |  |  |  |
| Emin ÇAKAR |  |  |  |
| Öncül ELBER |  |  |  |
| Gökhan EROL | 30/05/2022 |  | Risk Assessment Training |
| Ahmet KUŞCU |  |  |  |
| Mustafa ULAŞ |  |  |  |
| Mustafa UYSAL |  |  |  |
| Emin ÇAKAR |  |  |  |
| Öncül ELBER |  |  |  |
| Mustafa UYSAL | 04/04/2023 |  | LOTO Training |
| Emin ÇAKAR |  |  |  |
| Öncül ELBER |  |  |  |
| GE |  |  |  |
| Name and Surname | Date |  | Training |
| Niyazi TÜRKTÜR | 1/03/2023 |  | Health and Safety |
| Özgür GIRGIN | 10/08/2022 |  |  |
| SECURITAS |  |  |  |
| Name and Surname | Date |  | Training |
| Aziz KORKUT | 6/04/2023 | 7/04/2023 | Health and Safety |
| Hasan Ali DEMIR | 3/04/2023 | 4/04/2023 |  |
| İbrahim KARAKUZU | 3/04/2023 | 4/04/2023 |  |
| Serdar GÖKÇE | 24/08/2022 | 25/08/2022 |  |

Occupational Health and safety (HSE) trainings have been provided in 2022/2023 and 12 employees are attended to the trainings.

In Monitoring Period; There have 4 various trainings been provided such as; Health and Safety, Lock Out Tag Out (LOTO) Training, Risk Assessment Training and Fire Awareness training.

Health and Safety Training is routine training which is mandatory with regulations.

LOTO, Risk Assessment and Fire awareness trainings were taken to increase awareness and the quality of the staff. All personnel under our SGK (social security institution), attended Risk Assessment and Fire Awareness Trainings. But in LOTO; who did not participate in this training under SGK (Social Security Institution), attended these trainings.

E.5.1. Explanation of calculation of value estimated ex ante calculation of approved PDD for this monitoring period

Estimated emission reduction in ex ante calculation of registered PDD is 131,241 tCO2, which corresponds to 131,241 tCO2 for 365 days. And actual emission reduction achieved during this monitoring period is 132,070 tCO2 (for (01/05/2022-30/04/2023), 365 days).

##### E.6. Remarks on increase in achieved SDG Impacts from estimated value in approved PDD

Estimated emission reduction in ex ante calculation of registered PDD is 131,241 tCO2. Moreover, actual emission reduction achieved during this monitoring period is 132,070 tCO2. When the actual emission reduction achieved during this monitoring period and estimated figure ​​were compared, it was seen that these values are very close to each other. As a result, the actual emission reduction during this monitoring period is acceptable. Because net difference between estimated and actual emission reduction for the project activity is 0.63%.

An estimated electricity net generation is 238,000 MWh per year in registered PDD. In this monitoring period, 239,519.893 MWh of net electricity is supplied to the grid. Net difference between estimated and actual electricity generation for the project activity is 0.64%.

#### SECTION F. SAFEGUARDS REPORTING

| Principles | Mitigation Measures were added to the Monitoring Plan |
| --- | --- |
| Principle 1 Human Rights | Not Required |
| Principle 2 Gender Equality and Women’s Rights | Not Required |
| Principle 3 Community Health, Safety and Working Conditions | HSE trainings are given to all the employees at the plant regularly. |
| Principle 4 Cultural Heritage, Indigenous Peoples Displacement and Resettlement | Not Required |
| Principle 5 Corruption | Not Required |
| Principle 6 Economic Impacts | Not Required |
| Principle 7 Climate and Energy | The project activities has replaced the grid electricity, which is constituted of different fuel sources causing greenhouse gas emissions. By replacing in the consumption of these fuels, it contributes to conservation of water, soil, flora and faunas and transfers these natural resources and the additional supply of these primary energy sources to the future generations as well. In the absence of the project activity, an equivalent amount of electricity would have been generated from the power plants connected to the grid, majority of which are based on fossil fuels. Thus, the project is replacing the greenhouse gas emissions (e.g CO2) occurring from extraction, processing, transportation and burning of fossil-fuels for power generation connected to the national grid. |
| Principle 8 Water | There would be a potential fossil fuel generation activity if Bergres WPP would not come into implementation. In this scenario of electricity generation, there will be a significant wastewater that will be discharged into natural environments. Wastewater discharge was avoided thanks to Bergres WPP, which was implemented instead of a Thermal Power Plant. This is a positive contribution for Principle 8. |
| Principle 9 Environment, Ecology and Land Use | Air Quality (CO and NMVOC): This project is replacing the greenhouse gas emissions (CO2, CH4) and other pollutants (CO and NMVOC) occurring from extraction, processing, transportation and burning of fossil-fuels for power generation connected to the national grid. |

There are no assessment that have been 'potentially' answered or where requirements require regular re-assessment. (Please see approved GS Transition document)

#### SECTION G. STAKEHOLDER INPUTS AND LEGAL DISPUTES

##### G.1. List all Inputs and Grievances which have been received via the Continuous Input and Grievance Mechanism together with their respective responses/mitigations.

##### For the Bergres Wind Power Project, stakeholder engagement procedure was conducted at the muhktar of Hacılar village head and the local stakeholders (from Avunduk, Hacılar, Balaban villages) face to face on 08/06/2023. During the meeting, the stakeholders stated that they had no complaints.

##### Since muhktar is the head of the village, he will be the contact persons between the project owner and the local stakeholders. Muhtar will make sure that there’s continuous communication between the two parties. On site visit, it is verified that there were no negative comments from the stakeholders.

##### Since this project is about the complete its 6th verification of first crediting period and it does not result in different impacts with respect to different vintages, the grievance mechanism does mainly rely on the existed grievance mechanism by the use of logbook. There would not requirement to use further grievance tools throughout the project life cycle.

##### In addition, logbook was checked to see whether there has been any complaints from the local stakeholders and it is seen that there were no negative comments from the local stakeholders during this monitoring period.

##### G.2. Report on any stakeholder mitigations that were agreed to be monitored.

There have not been stakeholder mitigations that were agreed to be monitored.

##### G.3. Provide details of any legal contest that has arisen with the project during the monitoring period

There is no legal contest or conflict that has occurred with the project during this monitoring period

Revision History

| Version | Date | Remarks |
| --- | --- | --- |
| 1.1 | 14 October 2020 | Hyperlinked section summary to enable quick access to key sections Improved clarity on Key Project Information Section for POA monitoring Forward action request section Improved Clarity on SDG contribution/SDG Impact term used throughout Clarity on safeguard reporting Clarity on design changes Leakage section added for VER/CER projects Addition of Comparison of monitored parameters with last monitoring period Provision of an accompanying Guide to help the user understand detailed rules and requirements |
| 1.0 | 10 July 2017 | Initial adoption |

Please see; Generation Licence

See, Generation License for coordinates and http://rcn.montana.edu/Resources/Converter.aspx for the conversion.

ACM 0002 version 17: https://cdm.unfccc.int/filestorage/D/5/Y/D5YFS9I3VKBT18MQNGX0LPZ6U7AWCO/ACM0002_%28v17%200%29_clean.pdf?t=NjV8cjdoeW02fDBkSikLmV7rg4J_9oPRwGpT

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-07-v4.0.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-01-v7.0.0.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-02-v5.0.0.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-03-v2.pdf

https://cdm.unfccc.int/methodologies/PAmethodologies/tools/am-tool-10-v1.pdf

- Deutsche WindGuard Bergres Energy Yield Assessment, Page 8

- Related documents have been submitted to VVB.

Related document has been provided to VVB.

http://www.sanayi.gov.tr/download/osgm/olcu_aletleri_muayene_yonetmelik.zip , page 2

For CO and NMVOC emission amounts  see :  http://www.tuik.gov.tr/PreHaberBultenleri.do?id=16174

- www.epdk.org.tr/TR/Dokuman/3156

See, http://www.mevzuat.gov.tr/Metin.Aspx?MevzuatKod=7.5.6381&MevzuatIliski=0&sourceXmlSearch=

Please see Transition Annex

- Please see  https://webim.teias.gov.tr/file/619d9ebb-06b1-4a96-9ce4-14eab1aa87d3?download

- Whenever emission reductions are capped, both the original and capped values used for calculations must be transparently reported.  Use brackets to denote original values.