Shizaiqian MSW landfill site LFG recovery to power project

Document Prepared by Yongmei Fang,Quanzhou BCCY New Energy Co.,Ltd.

| Project Title | Shizaiqian MSW landfill site LFG recovery to power project |
| --- | --- |
| Version | 01 |
| Date of Issue | 18-06-2020 |
| Prepared By | Quanzhou BCCY New Energy CO.,Ltd. |
| Contact | Physical address:No.111-2,Wanghong Road,Xinnan community,Shuangyang Subdistrict,Luojiang District,Quanzhou,Fujian Province,P.R.China Telephone :+81 371 56737901 Email:ymfang@bccynewpower.com Website: http://www.bccynewpower.com/ |

Contents

1	Project Details	4

1.1	Summary Description of the Project	4

1.2	Sectoral Scope and Project Type	4

1.3	Project Eligibility	4

1.4	Project Design	4

1.5	Project Proponent	4

1.6	Other Entities Involved in the Project	5

1.7	Ownership	5

1.8	Project Start Date	5

1.9	Project Crediting Period	5

1.10	Project Scale and Estimated GHG Emission Reductions or Removals	5

1.11	Description of the Project Activity	6

1.12	Project Location	8

1.13	Conditions Prior to Project Initiation	8

1.14	Compliance with Laws, Statutes and Other Regulatory Frameworks	9

1.15	Participation under Other GHG Programs	9

1.16	Other Forms of Credit	9

2	Safeguards	10

2.1	No Net Harm	10

2.2	Local Stakeholder Consultation	10

2.3	Environmental Impact	11

2.4	Public Comments	11

2.5	AFOLU-Specific Safeguards	12

3	Application of Methodology	12

3.1	Title and Reference of Methodology	12

3.2	Applicability of Methodology	12

3.3	Project Boundary	17

3.4	Baseline Scenario	19

3.5	Additionality	19

3.6	Methodology Deviations	20

4	Quantification of GHG Emission Reductions and Removals	20

4.1	Baseline Emissions	20

4.2	Project Emissions	34

4.3	Leakage	36

4.4	Net GHG Emission Reductions and Removals	36

5	Monitoring	37

5.1	Data and Parameters Available at Validation	37

5.2	Data and Parameters Monitored	45

5.3	Monitoring Plan	50

Appendix	53

Project Details

Summary Description of the Project

Shizaiqian MSW landfill site LFG recovery to power project (hereafter referred to as the project) is located at Shizaiqian MSW landfill site(hereafter referred to as the Shizaiqian landfill site)， in Quanzhou City, Fujian Province, P.R. China. The project is constructed and operated by Quanzhou BCCY New Energy CO.,Ltd. (hereafter referred to as the project owner)

Shizaiqian landfill site started collecting and handling waste in 2011.

The project has a total designed capacity 5MW(5*1MW) and combusts the LFG,which contains nearly 50% of methane,to generate electricity and export it  to the Eastern China Power Grid(ECPG). VCUs will be claimed from the methane destroyed and the grid electricity displaced.The estimated average GHG emission reductions of the project  is 66,269 tCO2e and the total GHG emission reductions and removals are 662,691 tCO2e during the fixed 10 years crediting period.

Scenario existing prior to the implementation of the project (the same as baseline scenario):

LFG from Shizaiqian landfill site is emitted to the atmosphere directly.

Equivalent electricity generated by the project is supplied by the Eastern China Power Grid

(ECPG), which is dominated by fossil fuel based power plants.

Sectoral Scope and Project Type

Sector scope 1:energy (renewable /Non-renewable) and

Sector scope 13:waste handling and disposal

Project Eligibility

The project is not a grouped project.

Project Design

Not applicable

Project Proponent

| Organization name | Quanzhou BCCY New Energy Co.,Ltd. |
| --- | --- |
| Contact person | Lei Wang |
| Title | Minister of carbon emission department |
| Address | Room 1003,Hengmei Building,Dongfeng Road,No.22,Jingshui District,Zhengzhou city,Henan Province,P.R.China |
| Telephone | +81 371 56737901 |
| Email | lwang@bccynewpower.com |

Other Entities Involved in the Project

| Organization name | - |
| --- | --- |
| Role in the project | - |
| Contact person | - |
| Title | - |
| Address | - |
| Telephone | - |
| Email | - |

Ownership

The project owner is Quanzhou BCCY New Energy Co.,Ltd..The approval of Feasibility Study Report(FSR) and Environmental impact Assesment (EIA),and the business license of the project owner are the evidences for legislative right.Besides,the equipment purchasing contract and the purchasing power agreement are the evidences for the ownership of the plant equipment and power generating.

Project Start Date

20/11/2018 (commercial operation started date)

Project Crediting Period

The project crediting period runs from 20th November 2018 through 19th November 2028 for 10 years fixed.

Project Scale and Estimated GHG Emission Reductions or Removals

| Project Scale |  |
| --- | --- |
| Project | √ |
| Large project |  |

| Year | Estimated GHG emission reductions or removals (tCO2e) |
| --- | --- |
| 2018 .11.20-2018.12.31 | 11,975 |
| 2019 | 110,829 |
| 2020 | 117,160 |
| 2021. | 123,214 |
| 2022 | 129,095 |
| 2023 | 96,567 |
| 2024 | 73,851 |
| 2025 | 0 |
| 2026 | 0 |
| 2027 | 0 |
| 2028.1.1-2028.11.19 | 0 |
| Total estimated ERs | 662,691 |
| Total number of crediting years | 10 |
| Average annual ERs | 66,269 |

Description of the Project Activity

GHG emission reductions will be achieved through combustion of the recovered methane gas by gas engines,which would be otherwise emitted to the atmosphere, and the generation of electricity from the landfill gas,which is supplied by the Eastern China Power Grid (ECPG) prior to the implementation of the project.

Description of the technology in the project

The project consists in LFG collection, transmission and treatment system, with subsequent electricity generation and grid connection system.

LFG collection system

LFG is extracted from MSW under negative pressure by blower pumps and moved through wells, then collected by gas collection stations and transferred by sub-pipes and a main pipe to LFG treatment equipment. Flow rate of the LFG is regulated at the collection points in order to always fit with the consumption capacity of the generation engines.

LFG treatment system

Prior to electricity generation, LFG is treated to remove impurities and moisture, to avoid corrosion in the engines. The treatment consists of filtration, de-moisturing, cooling and pressurization.

Electricity generation and grid connection system

5 gas engines of 1MW rated power each are fed with the LFG and generate electricity,which is then exported to the grid.

Table 1.1 Equipment technical parameters

| Main Equipment | Parameter | Value |
| --- | --- | --- |
| Gas engine | Type | H16V190ZLZ-2 |
|  | Manufacturer | CNPC JICHAI POWER EQUIPMENT COMPANY |
|  | Rated capacity | 1200 kW |
|  | Rated rotation speed | 1000 r/min |
| Generator | Type | Siemens technology |
|  | Manufacturer | CNPC JICHAI POWER EQUIPMENT COMPANY |
|  | Rated Capacity | 1000 kW |
|  | Rated rotation speed | 1000 r/min |
|  | Capacity factor | 0.8 (lagging) |

Project Location

The Project is located in Shizaiqian Landfill Site, Luojiang District, Quanzhou City of Fujian Province, P.R. China. The geographical coordinates of the project site are longitude 118°37′45.96″E and latitude 24°57′55.72″N. Figure 1 shows the location of the Project.

Fujian Province

Quanzhou city

The project

d

Figure 1.1 Location of the project

Conditions Prior to Project Initiation

The baseline scenarios are the same as the conditions existing prior to the project initiation.The details are showed in the Section 3.4.

Compliance with Laws, Statutes and Other Regulatory Frameworks

The project is in the field of waste handling and disposal and meets the relevant provision of  the law of the People’s Republic of China on the Prevention and Control of Environmental Pollution by Solid Waste. The generation of power exported to ECPG will meet the requirement of national laws and regulations ,also financially viable.

Participation under Other GHG Programs

Projects Registered (or seeking registration) under Other GHG Program(s)

The project was not registered(or seeking registration) under other GHG Program(s).

Projects Rejected by Other GHG Programs

Not applicable

Other Forms of Credit

Emissions Trading Programs and Other Binding Limits

The project didn’t reduce GHG emissions from activities that are included in an emissions trading program or any other mechanism that includes GHG allowance trading, and include details about any such programs or mechanisms.

Other Forms of Environmental Credit

The project hasn’t sought or received another form of GHG-related environmental credit, including renewable energy certificates.

- Leakage Management

Not applicable

- Commercially Sensitive Information

No commercially sensitive information has been excluded from the public version of the project description.

- Sustainable Development

The project will contribute to sustainable development in the following ways:

Landfill safety

If methane concentration in the air increases by up to 5-15% in volume with the confined space of a building,the risk of explosion is very high.Prior to the implementation of the project,a few venting pipes and wells have been installed for LFG emitting for safety reasons.With the project,a modern gas extraction system has been installed to ensure the effective collection of LFG and also avoid the risk of landfill explosions.

Odour reduction

Odour from landfill negatively impacts on residents around the landfill. Implementation of the project will reduce odours through LFG collection and will thus mitigate the impact of landfill odour on people’s daily lives.

Energy potential

Methane is an ideal clean fuel.Each cubic meter of methane contains about 360000kJ calorific value,LFG recovery and utilization will supplement the energy supply of Quanzhou city.

Provide employment

The project was designed and technically supported by experts.Employment opportunities are created during both the project construction and operation periods.

- Further Information

Not applicable

Safeguards

No Net Harm

Not applicable

Local Stakeholder Consultation

The project owner distributed questionnaires to local residents who may be impacted by the project in order to collect advice for the project. The aim of these questionnaires was to collect opinions concerning the influence the project would have on the local society, environment, economy, daily life etc.

The following questions are from the questionnaires and stakeholder consultancy meeting:

  - What do you think about the environment of surrounding areas?

  - Do you support the construction of this LFG power station?

  - Do you think the implementation of the project will cause positive effect on living of local residents?

  - Do you think the implementation of the project will cause negative effect?

The stakeholders were informed about the stakeholder meeting through posters on  8/10/2018. In the bulletin, the company invited the potential stakeholders to get to know their opinions and/or suggestions about the implementation of the project and VCS application of the project. The stakeholder consultancy meeting for the parties interested in the project was organized at Shizaiqian Landfill site on 23/10/2018 to collect opinions from all stakeholders, such as representatives of local residents,Quanzhou solid waste management station, the project owner and so on. The project owner appointed one person to make a meeting minute.

Environmental Impact

The Environment Impact Assessment (EIA) was finished by Fujian environmental Safety Testing and Assessment Co., Ltd. and approved by Environment Protection Bureau of Quanzhou City on 25/6/2018. The project has no significant impact on environment.

Public Comments

25 questionnaires were distributed, and 25 questionnaires were returned. The age range of questioned people was 24-54 years old.

- About 36% of questioned people think the environment of surrounding areas is bad, and others think it is acceptable and good.

- 100% of questioned people support the construction of the project.

- 92% and 80% of questioned people think the implementation of the project will mitigate air pollution and lack of electricity, respectively.

- 40% of questioned people think the noise of implementation of the project will affect the living of local residents surround.

There are 12 stakeholders attended the stakeholder consultation meeting and the project   owner had taken meeting minute.

During the stakeholder consultancy meeting, some stakeholders worried that the project will bring noise, land occupying and pollution on employees and local residents’ living. For these issues, the project owner will offer some measures: i.e. installation of sound proof devices and plating of green isolation belts are used for mitigating noise; the area the project built on only will not occupy farms of local residents; at the project site, waste recycling will be carried out reduce emit of waste, and the project site meanwhile will be far from the residential area, so the effect of the noise from the plant is little to local residents. All attendances to the meeting were satisfied to the measures.

As a whole, the impacts of construction of the project are basically positive. All stakeholders were pleased with the development of the project. The CDM project would actually facilitate the development of the local economy and increase the income of local residents.

AFOLU-Specific Safeguards

Not applicable

Application of Methodology

Title and Reference of Methodology

Title: Flaring or use of landfill gas --- Version 19.0

Reference : https://cdm.unfccc.int/methodologies/PAmethodologies/approved

The methodology also refers to the latest approved versions of the following tools:

“Combined tool to identify the baseline scenario and demonstrate additionality” (Version 7.0);

“Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation” (Version 3.0);

“Emissions from solid waste disposal sites” (Version 8.0);

“Tool to calculate the emission factor for an electricity system” (Version 07.0);

"Tool to determine the mass flow of a greenhouse gas in a gaseous stream"(Version 3.0);

“Positive lists of technologies” (Version 2.0)

Applicability of Methodology

The selected methodology (ACM0001, version 19.0) is appropriate to LFG project activities, where the baseline scenario is the atmospheric release of the LFG, and that all or part of the electricity exported to the grid is the electricity generation in existing and/or new grid-connected power plants. In this case, the LFG is released to atmosphere prior to the implementation of the project, electricity generated by the project is exported to Eastern China Power Grid (ECPG).

The project fulfils the following applicability conditions of the methodology:

| Applicability conditions | Justifications |
| --- | --- |
| - Install a new LFG capture system in an existing or new(Greenfield) SWDS where no LFG capture system was or would have been installed prior to the implementation of the project activity; or | Applicable. The project involves the installation of a new LFG capture system in an existing SWDS. |
| - Make an Investment into an existing LFG capture system to increase the recovery rate or change the use of the captured LFG, provided that: - The captured LFG was vented or flared and not used prior to implementation of the project activity; and - In the case an existing active LFG capture system for which the amount of LFG cannot be collected separately from the project system after the implementation of the project activity and its efficiency is not impacted on by the project system: historical data on the amount LFG capture and flared is available; | Not applicable. There wasn’t a LFG capture system prior to the implement of the project. |
| - Flare the LFG and/or use the captured LFG in any (combination) of the following ways: - Generation electricity; - Generation heat in a boiler, air or kiln (brick firing only) or glass melting furnace; and/or - Supplying the LFG to consumers through a natural gas distribution network; - Supplying compressed/liquefied LFG to consumers using trucks; - Supplying the LFG to consumers through a dedicated pipeline; | Applicable.The project uses the captured LFG to generate electricity, which corresponds to point(i). |
| - Do not reduce the amount of organic waste that would be recycled in the absence of the project. | The implementation of the project does not reduce the amount of organic waste that would be recycled in the absence of the project. All the solid waste is disposed in the Shizaiqian landfill site. |
| The methodology is only applicable if the application of the procedure to identify the baseline scenario confirms that the most plausible baseline scenario is: - Atmospheric release of the LFG or captured of LFG and destruction through flaring to comply with regulations or contractual requirements, to address safety and odour concerns, or for other reasons; and - In the case that the LFG is used in the project activity for generation electricity and/or generating heat in a boiler, air heater, glass melting furnace on kiln: - For electricity generation: that electricity would be generated in the grid or in captive fossil fuel fired power plants; and/or - For heat generation: that heat would be generated using fossil fuels in equipment located within the project boundary - In the case of LFG supplied to the end-user(s) through natural gas distribution network, trucks or the dedicated pipeline, the baseline scenario is assumed to be displacement of the natural gas. - In the case of LFG from a Greenfield SWDS, the identified baseline scenario is atmospheric release of the LFG or captured of LFG in a managed SWDS and destruction through flaring to comply with regulations or contractual requirements, to address safety and odour concerns, or for other reasons. | The most plausible baseline scenario of the project is : (1)LFG from Shizaiqian landfill site is emitted to the atmosphere directly. (2)Equivalent electricity generated by the project is supplied by the Eastern China Power Grid(ECPG). |
| The methodology is not applicable: - In combination with other approved methodologies. For instance, ACM0001 cannot be used to claim emission reductions for the displacement of fossil fuels in a kiln or glass melting furnace, where the purpose of the CDM project activity is to implement energy efficiency measures at a kiln or glass melting furnace; - If the management of the SWDS in the project activity is deliberately changed during the crediting in order to increase methane generation compared to the situation prior to the implementation of the project activity. | Not applicable. |

Regarding to tool—“Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation”：

| Applicable conditions | Justifications |
| --- | --- |
| If emissions are calculated for electricity consumption,the tool is only applicable if one out of the following three scenarios applied to the sources of the electricity consumption: - Scenario A:Electricity consumption from the grid. The electricity is purchased from the grid only, and either no captive power plant(s) is/are installed at the site of electricity consumption or, if any captive power plant exists on site,it is either not operating or it is not physically able to provide electricity to the electricity consume; - Scenario B: Electricity consumption from (an) off-grid fossil fuel fired captive power plant(s).One or more fossil fuel fired captive power plants are installed at the site of the electricity consume and supply the consumer with electricity.The captive power plant(s) is/are not connected to the electricity grid; or - Scenario C:Electricity consumption from the grid and (a) fossil fuel fired captive power plant(s). One or more fossil fuel fired captive power plants operate at the site of the electricity consumer. The captive power plant(s) can provide electricity consumer. The captive power plant(s) is/are also connected to the electricity grid. Hence, the electricity consumer can be provided with electricity from the captive power plant(s) and the grid. | Scenario A is applicable for the project. The project purchases electricity from the grid during the operation. There is no captive power plant(s) at the site of the project. |
| This tool is not applicable in cases where captive renewable power generation technologies are installed to provide electricity in the project activity, in the baseline scenario or to sources of leakage. | Not applicable. |

Regarding to tool— “Emissions from solid waste disposal sites” (Version 8.0)：

| (a)Application A: The CDM project activity mitigates methane emissions from a specific existing SWDS. Methane emissions are mitigated by capturing and flaring or combusting the methane (e.g. "ACM0001: Flaring or use of landfill gas). The methane is generated from waste disposed in the past, including prior to the start of the CDM project activity. In these cases, the tool is only applied for an ex ante estimation of emissions in the project design document (CDM-PDD). The emissions will then be monitored during the crediting period using the applicable approaches in the relevant methodologies (e. g. measuring the amount of methane captured from the SWDS) | Applicable. The project collects the methane from the Shizaiqian landfill site and use it to generate the electricity. |
| --- | --- |
| (b)Application B: The CDM project activity avoids or involves the disposal of waste ata SWDS. An example of this application of the tool is ACM0022, in which municipal solid waste (MSW)is treated with an alternative option, such as composting or anaerobic digestion, and is then prevented from being disposed of in a SWDS. The methane is generated from waste disposed or avoided from disposal during the crediting period. In these cases, the tool can be applied for both ex ante and ex post estimation of emissions. These project activities may apply the simplified approach detailed in 0 when calculating baseline emissions | Not applicable. |

Project Boundary

Define the project boundary and identify the relevant GHG sources, sinks and reservoirs for the project and baseline scenarios (including leakage if applicable).

| Source |  | Gas | Included? | Justification/Explanation |
| --- | --- | --- | --- | --- |
| Baseline | Emissions from decomposition of waste at the landfill site | CO2 | Yes | The major source of emissions in the baseline. |
|  |  | CH4 | No | N2O emissions are small compared to CH4 emissions from landfills. Exclusion of this gas is conservative. |
|  |  | N2O | No | CO2 emissions from the decomposition of organic waste are not accounted. |
|  | Emission from electricity consumption | CO2 | Yes | Electricity is consumed from the ECPG in the baseline scenario. |
|  |  | CH4 | No | Excluded for simplification,this is conservative. |
|  |  | N2O | No | Excluded for simplification,this is conservative. |
|  | Emissions from thermal energy generation | CO2 | No | No thermal energy generation is included in the project. |
|  |  | CH4 | No | No thermal energy generation is included in the project. |
|  |  | N2O | No | No thermal energy generation is included in the project. |
| Project | On-site fossil fuel consumption due to the project other than for electricity generation | CO2 | No | Excluded because there is no fossil fuel consumption. |
|  |  | CH4 | No | Excluded because there is no fossil fuel consumption. |
|  |  | N2O | No | Excluded because there is no fossil fuel consumption. |
|  | Emission from on-site electricity use | CO2 | CO2 | Main emission source. |
|  |  | CH4 | CH4 | Excluded for siompilification.This emission source is very small compared to CO2 emissions. |
|  |  | N2O | N2O | Excluded for simplification.This emission source is very small compared to CO2 emissions. |

The project boundary is the site of the project activity, Shiziaqian landfill site, where the LFG is captured and used, and also includes all the power sources connected to the ECPG, which expands throughout Shanghai City, Jiangsu Province, Zhejiang Province, Anhui Province, and Fujian Province.

project boundary

Dumped MSW sources

Shizaiqian landfill site

LFG collection system

Gas-generator sets

ECPG

Electricity self-consumption

Figure 3.1 the Diagram of Project Boundary

Baseline Scenario

According to the section 5.3.1 of the methodology ACM0001,the project chooses simplified procedures to identify the baseline scenario.

The LFG from Shizaiqian landfill site is vented to the atmosphere prior the implementation of the project for safety concerns, and the majority electricity generated by the LFG will be exported to the Eastern China Power Grid (ECPG), and a little of which is consumed by the project.

Therefore, the baseline scenario is:

LFG from Shizaiqian landfill site is emitted to the atmosphere directly.

Equivalent electricity generated by the project is supplied by the Eastern China Power Grid        (ECPG), which is dominated by fossil fuel based power plants.

Additionality

According to the section 5.3.1 of the methodology ACM0001,the project chooses simplified procedures to demonstrate additionality. Tool32(Positive lists of technologies) is used to demonstrate additionality.

| Requirements of tool 32(Positive lists of technologies) | The project |
| --- | --- |
| 5.1Waste handling and disposal 5.1.1Landfill gas recovery and its gainful use The project activities and PoAs at new or existing landfills (greenfield or brownfield)are deemed automatically additional if it is demonstrated that prior to the implementation of the project activities and PoAs the landfill gas(LFG)was only vented and or flared (in the case of brownfield projects)or would have been only vented and/or flared(in the case of greenfield projects)but not utilized for energy generation, and that under the project activities and PoAs any of the following conditions are met: (a) The LFG is used to generate electricity in one or several power plants with a total nameplate capacity that equals or is below 10 MW; (b) The LFG is used to generate heat for internal or external consumption (c)The LFG is flared | The LFG from Shizaiqian landfill site is vented to the atmosphere prior the implementation of the project for safety concerns.Total nameplate capacity of the project is 4MW,which is below 10MW. |

Conclusion: the project is automatically additional.

Methodology Deviations

Not applicable

Quantification of GHG Emission Reductions and Removals

Baseline Emissions

The project utilizes the LFG for generation to substitute the equivalent electricity supplied by the grid, resulting in CH4 and CO2 emissions, which will calculated as follows according to the methodology ACM0001(version 19.0):

BEy=BECH4,y+BEEC,y+BEHG,y+BENG,y                                                         Equation(1)

Where:

| BEy | = | Baseline emissions in year y (t CO2e/yr) |
| --- | --- | --- |
| BECH4,y | = | Baseline emissions of methane from the SWDS in year y (t CO2e/yr) |
| BEEC,y | = | Baseline emissions associated with electricity generation in year y (t CO2/yr) |
| BEHG,y | = | Baseline emissions associated with heat generation in year y (t CO2/yr) |
| BENG,y | = | Baseline emissions associated with natural gas use in year y (t CO2/yr) |

The project dosen’t involve heat and natural gas. BEHG,y=0,BENG,y=0.Therefore:

BEy=BECH4,y+BEEC,y                                                                                        Equation(2)

Step A: Baseline emissions of methane from the SWDS (BECH4,y)

BECH4=((1-OXtop_layer)×FCH4,pj,y-FCH,BL,y)×GWPCH4                                  Equation(3)

Where:

| BECH4,y | = | Baseline emissions of methane from the SWDS in year y (tCO2e/yr) |
| --- | --- | --- |
| OXtop_layer | = | Fraction of methane in the LFG that would be oxidized in the top layer of the SWDS in the baseline (dimensionless) |
| FCH4, PJ,y | = | Amount of methane in the LFG which is flared and/or used in the project activity in year y (tCH4/yr) |
| FCH4, BL,y | = | Amount of methane in the LFG that would be flared in the baseline in year y (tCH4/yr) |
| GWPCH4 | = | Global warming potential of CH4 (tCO2e/tCH4) |

Step A.1: Ex post determination of FCH4,PJ,y

During the crediting period, FCH4,PJ,y is determined as the sum of the quantities of methane flared and used in power plant(s), boiler(s), air heater(s), kiln(s) and natural gas distribution network, as follows:

FCH4, PJ,y=FCH4, flared, y+FCH4, EL, y+FCH4, HG, y+FCH4, NG,y                                       Equation(4)

Where:

| FCH4, PJ,y | = | Amount of methane in the LFG which is flared and/or used in the project activity in year y (t CH4/yr) |
| --- | --- | --- |
| FCH4, flared, y | = | Amount of methane in the LFG which is destroyed by flaring in year y (t CH4/yr) |
| FCH4, EL, y | = | Amount of methane in the LFG which is used for electricity generation in year y (t CH4/yr) |
| FCH4, HG, y | = | Amount of methane in the LFG which is used for heat generation in year y (t CH4/yr) |
| FCH4, NG,y | = | Amount of methane in the LFG which is sent to the natural gas distribution network and/or dedicated pipeline and/or to the trucks in year y (t CH4/yr) |

The working hours of the power plant(s), boiler(s), air heater(s) and kiln(s) should be monitored and no emission reduction should be claimed for methane destruction during non-working hours.

The project utilizes the LFG only for power generation. Therefore:

FCH4, PJ,y=FCH4, EL, y                                                                                              Equation(5)

FCH4,EL,y is determined using the “Tool to determine the mass flow of a greenhouse gas in a gaseous stream”. The following requirements apply:

- The gaseous stream the tool shall be applied to is the LFG delivery pipeline to each item of electricity generation or heat generation equipment j, or the natural gas distribution system. FCH4,EL,y then calculated as the sum of mass flows to each item of electricity generation or heat generation equipment j;

- CH4 is the greenhouse gases for which the mass flow should be determined;

- The flow of the gaseous stream should be measured on continuous basis;

- The simplification offered for calculating the molecular mass of the gaseous stream is valid (equations 3 or 17 in the tool); and

- The mass flow should be summed to a yearly unit basis (t CH4/yr).

Amount of the methane used for power generation (FCH4,EL,y)

The “Tool to determine the mass of a greenhouse gas in a gaseous stream” provides following 6 options for measuring mass flow of a greenhouse gas i in a gaseous stream .

Table 4.1 Measurement options

| Option | Flow of gaseous stream | Volumetric Fraction |
| --- | --- | --- |
| A | Volume flow-dry basis | Dry or wet basis |
| B | Volume flow-wet basis | Dry basis |
| C | Volume flow-wet basis | Wet basis |
| D | Mass flow- dry basis | Dry or wet basis |
| E | Mass flow-wet basis | Dry basis |
| F | Mass flow-wet basis | Wet basis |

In the gas stream,only CH4 is the greenhouse. Therefore Fi,t=FCH4,EL,y.

The flow meter and gas analyzer are installed by the project participant to monitor the volumetric flow of the LFG and the volumetric fraction of CH4 in the LFG,with both parameters monitored on a dry basis.Therefore,Option A of the” tool to determine the mass of a greenhouse gas in a gaseous stream” will be applied in the project.In addition, if in a particular interval it cannot be demonstrated that the gaseous stream is dry,then the flow measurement shall be assumed to be on a wet basis and the corresponding option from the table 2 showed in the ” tool to determine the mass of a greenhouse gas in a gaseous stream” should be applied instead.

Option A

Option A is applicable where the volumetric flow of the LFG is measured on a dry basis. In order to demonstrate that the LFG gaseous stream is dry, the project participants shall:

（a） Measure the moisture content of the gaseous stream (CH2O,t,db,n) and demonstrate that this is less or equal to 0.05 kg H2O/m3 dry gas; or

(b)   Demonstrate that the temperature of the gaseous stream (Tt) is less than 60℃ (331.15 K) at the flow measurement point.

For the project, it has been selected to demonstrate it using method (b).

The mass flow of CH4 shall be determined as follows:

Fi,t=Vt,db×vi,t,db×ρi,t                                                                                  Equation(6)

With:

ρi,t=Pt×MMiRu×Tt                                                                                                    Equation(7)

Where:

| Fi,t | = | Mass flow of greenhouse gas I in the gaseous stream in the inerval t (kg gas/h) |
| --- | --- | --- |
| Vt,db | = | Volumetric flow of the gaseous stream in time interval t on a dry basis (m3 dry gas/h) |
| vi,t,db | = | Volumetric fraction of greenhouse gas I in the gaseous stream in a time interval t on a dry basis (m3 gas i/m3 dry gas) |
| ρi,t | = | Density of greenhouse gas i in the gaseous stream in time interval t (kg gas i/m3 dry gas) |
| Pt | = | Absolute pressure of the gaseous stream in time interval t (Pa) |
| MMi | = | Molecular mass of greenhouse gas i (kg/kmol) |
| Ru | = | Universal ideal gases constant(Pa.m3/kmol.k) |
| Tt | = | Temperature of the gaseous stream in time interval t (K) |

All parameters about the LFG which is fed into the gas engines are converted automatically to normal conditions during the monitoring process.Therefore, Pt and Tt don’t need to be monitored.

Step A.1.1: Ex ante estimation of FCH4,PJ,y

An ex ante estimate of FCH4,PJ,y is required to estimate baseline emission of methane from the SWDS (according to equation (3)) in order to estimate the emission reductions of the proposed project activity in the CDM-PDD. It is determined as follows:

FCH4,PJ,y=PJ×BECH4, SWDS,y/GWPCH4                                                                 Equation (8)

Where:

| FCH4,PJ,y | = | Amount of methane in the LFG which is flared and/or used in the project activity in year y (t CH4/yr) |
| --- | --- | --- |
| BECH4, SWDS,y | = | Amount of methane in the LFG that is generated from the SWDS in the baseline scenario in year y (t CO2e/yr) |
| PJ | = | Efficiency of the LFG capture system that will be installed in the project activity |
| GWPCH4 | = | Global warming potential of CH4 (t CO2e/t CH4) |

BECH4,SWDS,y is determined using the methodological tool “Emissions from solid waste disposal sites”. The following guidance should be taken into account when applying the tool:

- fy in the tool shall be assigned a value of 0 because the amount of LFG that would have been captured and destroyed is already accounted for in equation 2 of this methodology; • In the tool, x begins with the year that the SWDS started receiving wastes (e.g. the first year of SWDS operation); and

  - Sampling to determine the fractions of different waste types is not necessary because the waste composition can be obtained from previous studies. Actually, the Research Institute of Mudanjiang Environmental & Health held an investigation in 2007. The waste was sampled and analyzed in the investigation. The data used in the PDD is from the investigation.

BECH4,SWDS,y=φy×(1-fy)×GWPCH4×(1-OX)×1612×F×DOCf,y×MCFy×x=1yj(Wj,x×DOCj×e-kj×y-x×(1-ekj)                                                        Equation (9)

Where:

| BECH4,SWDS,y | = | Baseline methane emissions occurring in the year y |
| --- | --- | --- |
| φy | = | Model correction factor to account for model uncertainties for year y |
| fy | = | Fraction of methane captured at the SWDS and flared,combusted or used in another manner that prevents the emission of methane to the atmosphere in year y |
| GWPCH4 | = | Global Warming Potential of methane |
| OX | = | Oxidation factor(reflecting the amount of methane from SWDS that is oxidized in the soil or other material covering the waste) |
| F | = | Fraction of the methane in the SWDS gas (volume fraction) |
| DOCf,y | = | Fraction of degradable organic carbon(DOC) that decomposes under the specific conditions occurring in the SWDS for year y(weight fraction) |
| MCFy | = | Methane correction factor for year y |
| Wj,x | = | Amount of organic waste type j disposed/prevented from disposal in the SWDS in the year y (t) |
| DOCj | = | Fraction of degradable organic carbon in the waste type j (weight fraction for year y |
| k | = | Decay rate for the waste type(1/yr) |
| j | = | Type of residual waste or types of waste in the MSW |
| x | = | Years of the crediting period for which waste is disposed at the SWDS, extending from the first year in the time period(x=1) to year y(x=y) |
| y | = | Years of the crediting period for which methane emissions are calculated |

Step A.2: Determination of FCH4,BL,y

This section provides a procedure to determine the amount of methane that would have been captured and destroyed (by flaring) in the baseline due to regulatory or contractual requirements, to address safety and odour concerns, or for other reasons (collectively referred to as requirement in this section). The four cases in Table 4.1 are distinguished. The appropriate case should be identified, and the corresponding instructions followed.

Table 4.1.Cases for determining methane captured and destroyed in the baseline

| Situation at the start of the project activity | Requirement to destroy methane | Existing LFG capture and destruction system |
| --- | --- | --- |
| Case 1 | No | No |
| Case 2 | Yes | No |
| Case 3 | No | Yes |
| Case 4 | Yes | Yes |

Currently China has regulations in place to deal with the management of landfills and to encourage utilization of LFG. Those regulations are:

“Standard for Pollution Control on the Landfill Site of Municipal Solid Waste” (GB 16889-2008), which became effective in 2008, issued by the Environment Protection Administration.

“Technical Code for Municipal Solid Waste Sanitary Landfill”（GB 50869-2013）), issued by the Ministry of Construction in 2013.

According to item 5.15 of GB16889-2008, if the designed landfill capacity is more than 2.5 million tons and the landfill thickness is more than 20m, methane utilization facilities or flare burning facilities shall be built to treat the landfill gas containing methane. For municipal solid waste landfills smaller than the above scale, technologies that can effectively reduce methane generation and emission shall be adopted or flare combustion facilities shall be used to treat methane containing landfill gas.

Item 11.1.1 of GB 50869-2013 stipulates that the landfill site must be equipped with effective landfill gas drainage facilities to prevent the natural accumulation and migration of landfill gas, causing fire and explosion. Item 11.1.3 stipulates that if the landfill does not have the conditions for landfill gas utilization, the flare method shall be adopted for combustion treatment, and the process that can effectively reduce the generation and emission of methane shall be adopted. The old landfills that are not safe and stable should be equipped with effective landfill gas drainage facilities. Among them, item 11.1.1 is mandatory and must be strictly implemented.

In fact, the LFG of Shizaiqian landfill is emitted to atmosphere without LFG capture system prior the implementation of the project. Therefore, Case 2 listed in the table 3 is applicable for the project.

The requirements above don’t specify any amount or percentage of LFG that should be destroyed.In this situation:

FCH4,BL,y=FCH4,BL,R,y                                                                                      Equation(10)

FCH4,BL,R,y=0.2×FCH4,PJ,capt,y                                                                        Equation(11)

Where:

| FCH4,BL,R,y | = | Amount of methane in the LFG which is flared in the baseline due to a requirement in year y (tCH4/yr) |
| --- | --- | --- |
| FCH4,PJ,capt,y | = | Amount of methane in the LFG which is captured in the project activity in year y (tCH4/yr) |

Step:B Baseline emissions associated with electricity generation (BEEC,y)

The baseline emissions associated with electricity generation in year y (BEEC,y)shall be  calculated using the "Methodological tool: Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation".

BEEC,y=ECBL,k,y×EFEF,k,y×(1+TDLk,y)                                                             Equation(12)

Where:

| BEEC,y | = | Baseline emission from electricity consumption in year y(t CO2/yr) |
| --- | --- | --- |
| ECBL,k,y | = | Quantity of electricity that would be consumed by the baseline electricity consumer k in year y(MWh/y) |
| EFEF,k,y | = | Emission factor for electricity generation for source k in year y(t CO2/MWh) |
| TDLk,y | = | Average technical transmission and distribution losses for providing electricity to source k in year y |
| k | = | Sources of electricity consumption in the baseline |

When applying the tool:

(a)  The electricity sources k in the tool correspond to the sources of electricity generated        identified in the selection of the most plausible baseline scenario; and

(b) ECBL,k,y in the tool is equivalent to the net amount of electricity generated using LFG in year y (EGPJ,y).

Scenario A of the tool “Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation” is applicable for the project for the electricity is purchased from the grid only,and either no captive power plant is installed at the site of electricity consumption.

According to the tool “Baseline, project and/or leakage emissions from electricity consumption and monitoring of electricity generation”, EFEF,k,y is calculated using the “Tool to calculate the emission factor for an electricity system”(EFEF,k,y=EFgrid,CM,y).

Project participants shall apply the following six steps:

| Step 1: | Identify the relevant electricity systems; |
| --- | --- |
| Step 2: | Choose whether to include off-grid power plants in the project |
| Step 3: | Select a method to determine the operating margin (OM); |
| Step 4: | Calculate the operating margin emission factor according to the |
| Step 5: | Calculate the build margin (BM) emission factor; |
| Step 6: | Calculate the combined margin (CM) emission factor. |

Step 1:Identity the relevant electricity systems

Ministry of Ecology and Environment of the People’s Republic of China has published a delineation of the project electricity system and connected electricity systems, so the project adopt the delineation of project electricity system and connected electricity system published by Ministry of Ecology and Environment of the People’s Republic of China. The power generated by the project displaces the equivalent electricity generated by the Eastern China Power Grid. The Eastern China Power Grid is a larger regional grid, which consists of six sub-grids: Shanghai City, Jiangsu, Zhejiang, Anhui, Fujian Province.

In addition, there is net imported power to the Eastern China Power Grid from the Central China Power Grid and North China Power Grid. According to the “Tool to calculate the emission factor for an electricity system”, use one of the following options to determine the CO2 emission factor for net electricity imports from a connected electricity system:

(a) 0tCO2/MWh; or

(b) The simple operating margin emission rate of the exporting grid, determined as described in Step 4 section 6.4.1, if the conditions for this method, as described in Step 3 below, apply to the exporting grid; or

(c) The simple adjusted operating margin emission rate of the exporting grid, determined as described in Step 4 section 6.4.2 below; or

- (d) The weighted average operating margin (OM) emission rate of the exporting grid, determined as described in Step 4 section 6.4.4 below.

The PD will choose option (b) to calculate the CO2 emission factor for net electricity imports from the Eastern China Power Grid.

According to the available data of 2013-2015, the corresponding marginal emission factors of electricity are calculated, and weighted average is carried out to obtain the marginal emission factors of electricity.

Step 2. Choose whether to include off-grid power plants in the project electricity system (optional).

Option I: Only grid power plants are included in the calculation.

OptionII: Both grid power plants and off-grid power plants are included in the calculation.

Following the calculation of the Ministry of Ecology and Environment of the People’s Republic of China,and statistical data is avalible, Option I is chosen.

Step 3. Select a method to determine the operating margin (OM)

“Tool to calculate the emission factor for an electricity system (Version 7.0)” offers four methods for the calculation of the operating margin emission factor(s) (EFgrid,OM,y):

- Simple OM; or

- Simple adjusted OM; or

- Dispatch data analysis OM; or

- Average OM.

Method (a) -Simple OM is chosen for calculation and low-cost/must-run resources constitute less than 50% of the total grid generation in average of the five most recent years.

For simple OM, the emission factor can be calculated using either of the two following data vintages:

- Ex ante option: If the ex ante option is chosen, the emission factor is determined once at the validation stage, thus no monitoring and recalculation of the mission factor during the crediting period is required. For grid power plants, use a 3-year generation-weighted average, based on the most recent data available at the time of submission of the CDM-PDD to the DOE for validation. For off-grid power plants, use a single calendar year within the 5 most recent calendar years prior to the time of submission of the CDM-PDD for validation;

- Ex post option: If the ex post option is chosen, the emission factor is determined for the year in which the project activity displaces grid electricity, requiring the emissions factor to be updated annually during monitoring. If the data required to calculate the emission factor for year y is usually only available later than six months after the end of year y, alternatively the emission factor of the previous year y-1 may be used. If the data is usually only available 18 months after the end of year y, the emission factor of the year proceeding the previous year y-2 may be used. The same data vintage (y, y-1 or y-2) should be used throughout all crediting periods

Project participant employs ex ante option for its operation margin calculation.

Step 4. Calculate the operating margin emission factor according to the selected method

The simple OM emission factor is calculated as the generation-weighted average CO2 emissions per unit net electricity generation (tCO2/MWh) of all generating power plants serving the system, not including low-cost / must-run power plants / units.

The simple OM may be calculated by one of the following two options:

- Option A: Based on the net electricity generation and a CO2 emission factor of each power unit; or

- Option B: Based on the total net electricity generation of all power plants serving the system and the fuel types and total fuel consumption of the project electricity system.Option B can only be used if:

- The necessary data for Option A is not available: and

- Only nuclear and renewable power generation are considered as low-cost/must-run power sources and the quantity of electricity supplied to the grid by these sources is known; and

- Off-grid power plants are not included in the calculation (i.e., if Option I has been chosen in Step 2).

Since the data of each power plant/unit is unavalible,Option A is not applicable to the project. The project adopts Option B to calculate the operating margin emission factor (EFgrid,OM,y) of ECPG.

EFgrid,OMsimple,y=iFCi,y×NCVi,y×EFCO2,i,yEGy                                                            Equation(13)

Where:

| EFgrid,OMsimple,y | = | Simple operating margin CO2 emission factor in year y (tCO2/MWh) |
| --- | --- | --- |
| FCi,y | = | Amount of fuel type i consumed in the project electricity system in year y (mass or volume unit) |
| NCVi,y | = | Net calorific value (energy content) of fuel type i in year y (GJ/mass or volume unit) |
| EFCO2,i,y | = | CO2 emission factor of fossil fuel type i in year y (tCO2e/GJ), 2006 IPCC Guidelines for default values |
| EGy | = | Net electricity generated and delivered to the grid by all power sources serving the system, not including low-cost/must run power plants/units in year y (MWh) |
| i | = | All fuel types combusted in power sources in the project electricity system in year y |
| y | = | The relevant year as per the dta vintage chosen in Step 3 |

Regarding parameter selection,local values of NCVi,y and EFCO2,i,y should be used where available. If no such values are available ,IPCC default values are preferable.The Net Calorific Value(NCVi,y) of each type of fossil fuel used in the calculation comes from China Energy Statistic Yearbook 2016.Emission factor(EFCO2,i,y) of each type of fossil fuel come from IPCC 2006 default values.

The operating margin emission factor for 2013, 2014and 2015 are calculated based on the data above. The three-year average is calculated as a weighted average of the emission factors. The Operational Margin Emission Factor is 0.8046 tCO2/MWh.

Step 5. Calculate the build margin emission factor

In terms of vintage of data,project participants can choose between one of the following two options:

- Option 1: For the first crediting period, calculate the build margin emission factor ex ante based on the most recent information available on units already built for sample group m at the time of CDM PDD submission to the DOE for validation. For the second crediting period, the build margin emission factor should be updated based on the most recent information available on units already built at the time of submission of the request for renewal of the crediting period to the DOE. For the third crediting period, the build margin emission factor calculated for the second crediting period should be used. This option does not require monitoring the emission factor during the crediting period.

- Option 2: For the first crediting period, the build margin emission factor shall be updated annually, ex-post, including those units built up to the year of registration of the project activity or, if information up to the year of registration is not yet available, including those units built up to the latest year for which information is available. For the second crediting period, the build margin emissions factor shall be calculated ex ante, as described in Option 1 above. For the third crediting period, the build margin emission factor calculated for the second crediting period should be used.

The project applies option 1 to calculate the build margin emission factor ex-ante.

According to the “Tool to calculate the emission factor for an electricity system”, the sample group of power units m used to calculate the build margin should be determined as per the following procedure,consistent with the data vintage selected above:

(a)  Identify the set of five power units, excluding power units registered as CDM project activities, that started to supply electricity to the grid most recently (SET5 units) and determine their annual electricity generation (AEGSET-5-units, in MWh);

(b) Determine the annual electricity generation of the project electricity system,excluding power units registered as CDM project activities (AEGtotal, in MWh). Identify the set of power units, excluding power units registered as CDM project activities, that started to supply electricity to the grid most recently and that comprise 20% of AEGtotal (if 20% falls on part of the generation of a unit, the generation of that unit is fully included in the calculation) (SET≥ 20%) and determine their annual electricity generation (AEGSET-≥ 20%, in MWh);

(c) From SET5-units and SET≥ 20 percent select the set of power units that comprises   the larger annual electricity generation (SETsample);

Identify the date when the power units in SETsample started to supply electricity to the grid. If none of the power units in SETsample started to supply electricity to the grid more than 10 years ago, then use SETsample to calculate the build margin. Ignore steps (d), (e) and (f).

Otherwise:

(d) Exclude from SETsample the power units which started to supply electricity to the grid more

than 10 years ago. Include in that set the power units registered as CDM project activity, starting with power units that started to supply electricity to the grid most recently, until the electricity generation of the new set comprises 20% of the annual electricity generation of the project electricity system (if 20% falls on part of the generation of a unit, the generation of that unit is fully included in the calculation) to the extent is possible. Determine for the resulting set (SETsample-CDM) the annual electricity generation (AEGSET-sample-CDM, in MWh);

If the annual electricity generation of that set is comprises at least 20% of the annual electricity generation of the project electricity system (i.e. AEGSET-sample-CDM≥0.2×AEGtotal), then use the sample group SETsample-CDM to calculate the build margin. Ignore steps (e) and (f).

- Include in the sample group SETsample-CDM the power units that started to supply electricity to the grid more than 10 years ago until the electricity generation of the new set comprises 20% of the annual electricity generation of the project electricity system ( if 20% falls on part of the generation of a unit, the generation of that unit is fully included in the calculation);

- The sample group of power units m used to calculate the build margin is the resulting set (SETsampleCDM>10yrs)

The build margin emissions factor is the generation-weighted average emission factor(tCO2/MWh) of all power units m during the most recent year y for which power generation data is available,calculated as follws:

EFgrid,BM,y=mEGm,y×EFEL,m,ymEGm,y                                                                                Equation(13)

Where:

| EFgrid,BM,y | = | Build margin CO2 emission factor in year y (t CO2/MWh) |
| --- | --- | --- |
| EGm,y | = | Net quantity of electricity generated and delivered to the grid by power unit m in year y (MWh) |
| EFEL,m,y | = | CO2 emission factor of power unit m in year y (t CO2/MWh) |
| m | = | Power units included in the build margin |
| y | = | Most recent historical yea for which electricity generation data is avalible |

Due to the availability of data, calculation of the BM in this PD adopted an alternative approved by CDM Executive Board. According to the alternative, the generation capacity of and the proportions of different generating technologies used in the mostly recent capacity additions would be calculated at first. The weight of each generating technology used in capacity additions can then be worked out and finally the Build Margin emission factor would be calculated with the commercial optimal efficiencies of the generating technologies.

The following method is adopted for the BM calculation. The first step is to use the most recently available data on the energy balance to calculate the proportion of CO2 emissions from solid, liquid, and gas fuels used for power generation in total CO2 emissions for the ECPG in 2015. Then this proportion is applied as a weight and combined with the emission factors of the best commercially available technologies to calculate the thermal emission factor of the ECPG. Finally, the BM factor of the ECPG can be determined by multiplying its thermal emission factor by the capacity-weighted share of thermal power in the newly added 20% of the installed capacity

(1) Calculate the proportion of CO2 emissions from solid, liquid, and gas fuels used for power generation in total CO2 emissions for the ECPG in the base year (2015)

λCoal,y=i∈COAL,jFi,j,y×NCVi,y×EFCO2,i,j,yi,jFi,j,y×NCVi,y×EFCO2,i,j,y                                                                  Equation(14)

λOil,y=i∈Oil,jFi,j,y×NCVi,y×EFCO2,i,j,yi,jFi,j,y×NCVi,y×EFCO2,i,j,y                                                                                        Equation(15)

λGas,y=i∈Gas,jFi,j,y×NCVi,y×EFCO2,i,j,yi,jFi,j,y×NCVi,y×EFCO2,i,j,y                                                                                      Equation(16)

Where:

| Fi,j,y | = | The amount of fuel i(in a mass or volume unit) consumed by province |
| --- | --- | --- |
| NCVi,y | = | Net calorific value(energy content) of fossil fuel type i(GJ/mass or volume unit) |
| COAL,OIL&GAS | = | The aggregation of various kinds of coal,oil,and gas as fossil fuels. |

(2) Calculate the corresponding emission factor for fossil fuel fired power generation.

EFThermal=λCoal×EFCoal,Adv+λOil×EFOil,Adv+λGAS×EFGas,Adv                      Equation(17)

Where:

EFCoal,adv, EFoIL,adv, EFGas,adv are the emission factors of coal, oil and gas-fired power generation with efficiency levels of the best commercially available technology in China in the previous three years.

(3) Calculate the Building Margin emission factor

EFBM,y=CAPThermalCAPTotal×EFThermal                                                                          Equation(18)

Where:

| CAPThermal | = | Total capacity additions while |
| --- | --- | --- |
| CAPTotal | = | Capacity additions of thermal power |

Following the four steps above,the build margin emission factor,EFgrid,BM,y of the ECPG is calculated to be 0.4923 tCO2/MWh.The detailed calculations and data are listed in the Annex 1.

Step 6. Calculate the combined margin emission  factor

Combined Margin emission factor(EFgrid,CM,y) is calculated as the weighted average of the operating margin emission factor(EFgrid,OM,y) and the build margin emission factor (EFgrid,BM,y),where the weights ωOM and ωBM ,by default,are 0.5 and 0.5 in the first crediting period,and EFgrid,OM,y and EFgrid,BM,yare calculated as described above and are expressed in tCO2/MWh.

EFgrid,CM,y=ωOM×EFgrid,OM,y+ωBM×EFgrid,BM,y                                                Equation(19)

EFgrid,CM,y=0.5×0.8046+0.5×0.4923=0.67145(tCO2/MWh)

The EFOM,y, EFBM,y and EFy are ex-ante calculation and are fixed during the credit period.

Project Emissions

Considering when electricity generators will be under regular maintenance and sometimes shut down, the electricity will be purchased from the ECPG during this period of time. Therefore, there will be project emissions, which is  calculated following the latest version of “Tool to calculate baseline, project and/or leakage emissions from electricity consumption”.

Project emissions are calculated as follows:

PEy=PEEC,y+PEFC,y+PEDT,y+PESP,y                                                             Equation(20)

Where:

| PEy | = | Project emission in year y(t CO2/yr) |
| --- | --- | --- |
| PEEC,y | = | Emissions from consumption of electricity due to the project activity in year y(t CO2/yr) |
| PEFC,y | = | Emissions from consumption of fossil fuels due to the project activity,for purpose other than electricity generation,in year y(t CO2/yr) |
| PEDT,y | = | Emissions from the distribution of compressed/liquefied LFG using trucks,in year y (t CO2/yr) |
| PESP,y | = | Emissions from the supply of LFG to consumers through a dedicated pipeline,in year y(tCO2/yr) |

The project does not include any activities consuming fossil fuels except electricity generation, so PEFC,y will be 0. The project does not involve the distribution of compressed/liquefied LFG using trucks and supply of LFG to consumers through a dedicated pipeline,Therefore, PEFC,y,PEDT,y,PESP,y will be 0.Hence, PEy is the same with PEEC,y.

PEEC,y is determined by “Tool to calculate baseline, project and/or leakage emissions from electricity consumption” as follow:

PEEC,y=jECPJ,j,y*EFEL,j,y*(1+TDLj,y)                                                       Equation(21)

Where:

| PEEC,y | = | Project emissions from electricity consumption in year y (tCO2/yr) |
| --- | --- | --- |
| ECPJ,j,y | = | Quantity of electricity consumed by the project electricity consumption source j in year y(MWh/yr) |
| EFEL,j,y | = | Emission factor for electricity generation for source j in year y(tCO2/MWh) |
| TDLj,y | = | Average technical transmission and distribution losses for providing electricity to source j in year y |

The “Tool to calculate baseline, project and/or leakage emissions from electricity consumption” provides 3 scenarios for different sources of electricity consumption. Since the electricity generated through the project is consumed solely from grid (ECPG), scenario A is applicable. In the case of applying scenario A, two options are available for determining emission factor. For this project, Option A1(EFEL,j,y=EFgrid,CM,y) is chosen.

For simplification,PEEC,y will be 0 in the ex-ante calculation.

Leakage

No leakage effects are accounted for under the ACM001 methodology.

Net GHG Emission Reductions and Removals

Emission reductions are calculated as follows:

ERy=BEy-PEy                                                                                               Equation(22)

Where:

| ERy | = | Emission reduction in year y (t CO2e/yr) |
| --- | --- | --- |
| BEy | = | Baseline emissions in year y (t CO2e/yr) |
| PEy | = | Project emissions in year y (t CO2e/yr) |

| Year | Estimated baseline emissions or removals (tCO2e) | Estimated project emissions or removals (tCO2e) | Estimated leakage emissions (tCO2e) | Estimated net GHG emission reductions or removals (tCO2e) |
| --- | --- | --- | --- | --- |
| 2018.11.20-2018.12.31 | 12,270 | 0 | 0 | 12,270 |
| 2019 | 113,559 | 0 | 0 | 113,559 |
| 2020 | 120,045 | 0 | 0 | 120,045 |
| 2021 | 126,248 | 0 | 0 | 126,248 |
| 2022 | 132.274 | 0 | 0 | 132.274 |
| 2023 | 98,945 | 0 | 0 | 98,945 |
| 2024 | 75,670 | 0 | 0 | 75,670 |
| 2025 | 0 | 0 | 0 | 0 |
| 2026 | 0 | 0 | 0 | 0 |
| 2027 | 0 | 0 | 0 | 0 |
| 2028.1.1-2028.11.19 | 0 | 0 | 0 | 0 |
| Total | 679,011 | 0 | 0 | 679,011 |

Monitoring

Data and Parameters Available at Validation

| Data / Parameter | OXtop_layer |
| --- | --- |
| Data unit | - |
| Description | Fraction of methane that would be oxidized in the top layer of the SWDS in the baseline |
| Source of data | “Emission from the solid waste disposal sites”(version 8.0) |
| Value applied | 0.1 |
| Justification of choice of data or description of measurement methods and procedures applied | - |
| Purpose of Data | Calculation of baseline emissions |
| Comments | - |

| Data / Parameter | GWPCH4 |
| --- | --- |
| Data unit | T CO2e/t CH4 |
| Description | Global warming potential of CH4 |
| Source of data | IPCC |
| Value applied | Default value of 25 from IPCC Fourth Assessment Report (AR4).Shall be updated according to any future COP/CMP decisions |
| Justification of choice of data or description of measurement methods and procedures applied | - |
| Purpose of Data | Calculation of baseline emissions |
| Comments | - |

| Data / Parameter | ρCH4 |
| --- | --- |
| Data unit | t/m3 |
| Description | Density of methane gas at Normal Conditions |
| Source of data | - |
| Value applied | 0.0007168(Normal contidions:0℃ and101.325kPa) |
| Justification of choice of data or description of measurement methods and procedures applied | - |
| Purpose of Data | Calculation of baseline emissions |
| Comments | - |

| Data / Parameter | ηPJ |
| --- | --- |
| Data unit | - |
| Description | Efficiency of the LFG capture system that will be installed in the project activity |
| Source of data | FSR of the project |
| Value applied | 70% |
| Justification of choice of data or description of measurement methods and procedures applied | - |
| Purpose of Data | Calculation of baseline emissions |
| Comments | - |

| Data / Parameter | φy |
| --- | --- |
| Data unit | - |
| Description | The model correction factor to account for model uncertainties |
| Source of data | Default value of the tool “Emissions from solid waste disposal sites”(version 08.0) |
| Value applied | 0.75 |
| Justification of choice of data or description of measurement methods and procedures applied | Application A is used to decide the value. And the value of dry conditions for used for the project since the MAP/PET<1. |
| Purpose of Data | - |
| Comments | - |

| Data / Parameter | fy |
| --- | --- |
| Data unit | - |
| Description | Fraction of methane captured at the SWDS and flared, combusted or used in another manner that prevents the emissions of methane to the atmosphere in year y |
| Source of data | The tool “Emissions from solid waste disposal sites”(version 08.0) |
| Value applied | 0 |
| Justification of choice of data or description of measurement methods and procedures applied | It has been considered in the section 4.1,equation (3).Therefore, fy=0 |
| Purpose of Data | - |
| Comments | - |

| Data / Parameter | OX |
| --- | --- |
| Data unit | - |
| Description | Oxidation factor |
| Source of data | Based on an extensive review of published literature on this subject,including the IPCC2006 Guidelines for National Greenhouse Gas Inventories. |
| Value applied | 0.1 |
| Justification of choice of data or description of measurement methods and procedures applied | - |
| Purpose of Data | Calculation of baseline emissions |
| Comments | - |

| Data / Parameter | F |
| --- | --- |
| Data unit | - |
| Description | Fraction of methane in the SWDS gas(volume fraction) |
| Source of data | IPCC 2006 Guidelines for National Greenhouse Gas Inventories |
| Value applied | 0.5 |
| Justification of choice of data or description of measurement methods and procedures applied | - |
| Purpose of Data | Calculation of baseline emissions |
| Comments | - |

| Data / Parameter | DOCf,y |
| --- | --- |
| Data unit | Weight fraction |
| Description | Default value for the fraction of degradable organic carbon(DOC) in MSW that decomposed in the SWDS |
| Source of data | IPCC 2006 Guidelines for National Greenhouse Gas Inventories |
| Value applied | 0.5 |
| Justification of choice of data or description of measurement methods and procedures applied | - |
| Purpose of Data | Calculation of baseline emissions |
| Comments | - |

| Data / Parameter | MCFy |
| --- | --- |
| Data unit | - |
| Description | Methane correction factor |
| Source of data | IPCC 2006 Guidelines for National Greenhouse Gas Inventories |
| Value applied | 1.0 |
| Justification of choice of data or description of measurement methods and procedures applied | The project using application A to decide the value of MCFy. The Shizaiqian landfill site has controlled placement of waste,mechanical compacting and levelling of the waste,So MCFy value of 1.0 for anaerobic managed solid waste disposal sites should be applied in line with”Emissions from solid waste disposal sites”(version 8.0) |
| Purpose of Data | Calculation of baseline emissions |
| Comments | - |

| Data / Parameter | Wx |
| --- | --- |
| Data unit | t |
| Description | Quantity of MSW land filled during 2011-2022 |
| Source of data | Records from landfill operator |
| Value applied | Year; Annual landfilled MSW (tons). 2011; 91,460. 2012; 282,094. 2013; 290,819. 2014; 299,813. 2015; 309,086. 2016; 318,645. 2017; 328,500. 2018; 338,355. 2019; 348,506. 2020; 358,961. 2021; 369,730. 2022; 380,822. |
| Justification of choice of data or description of measurement methods and procedures applied | The data FSR of the project. |
| Purpose of Data | Calculation of baseline emissions |
| Comments | - |

| Data / Parameter | DOCj |
| --- | --- |
| Data unit | - |
| Description | Fraction of degradable organic carbon in the waste type j(weight fraction) |
| Source of data | “Emissions from solid waste disposal sites”(version 08.0) |
| Value applied | Waste type j; DOCj (% wet waste). Wood and wood products; 43. Puip,paper and cardboard(other than sludge); 40. Textiles; 24. Garden,yard and park waste; 20. Glass,plastic,metal,other inert waste; 0. |
| Justification of choice of data or description of measurement methods and procedures applied | The climate condition of location of Shizaiqian landfill: Temperature:20.4℃ Mean annual precipitation：1225.5mm |
| Purpose of Data | Calculation of baseline emissions |
| Comments | - |

| Data / Parameter | kj |
| --- | --- |
| Data unit | 1/yr |
| Description | Decay rate for the waste type j |
| Source of data | “Emissions from solid waste disposal sites”(version 08.0) |
| Value applied | Waste type j; kj (MAT>20℃, MAP>1000mmm). Slowly degrading; Pulp,paper,cardboard(other than sludge); 0.07. Wood,wood products and straw; 0.035. Moderately degrading; Other(non-food) organica putrescible garden and park waste; 0.17. Rapidly degrading; Food,food waste,sewage sludge,beverages ans tabacco; 0.40. |
| Justification of choice of data or description of measurement methods and procedures applied | The climate condition of location of Shizaiqian landfill: Temperature:20.4℃ Mean annual precipitation：1225.5mm |
| Purpose of Data | Calculation of baseline emissions |
| Comments | - |

| Data / Parameter | EFgrid,OM,y |
| --- | --- |
| Data unit | tCO2/MWh |
| Description | Operation margin emission factor of ECPG |
| Source of data | 2017 China baseline emission factor of regional power grid. |
| Value applied | 0.8046 |
| Justification of choice of data or description of measurement methods and procedures applied | The value is published by Ministry of Ecology and Environment of the People’s Republic of China,based on China Electric Yearbook(2014-2016)，Electricity Industry Statistics(2013-2015),China Energy Statistic Yearbook(2014-2016) and IPCC data. |
| Purpose of Data | Calculation of baseline emissions |
| Comments | - |

| Data / Parameter | EFgrid,BM,y |
| --- | --- |
| Data unit | tCO2/MWh |
| Description | Build margin emission factor of ECPG |
| Source of data | 2017 China baseline emission factor of regional power grid. |
| Value applied | 0.4923 |
| Justification of choice of data or description of measurement methods and procedures applied | The value is published by Ministry of Ecology and Environment of the People’s Republic of China,based on China Electric Yearbook(2014-2016)，Electricity Industry Statistics(2013-2015),China Energy Statistic Yearbook(2014-2016) and IPCC data. |
| Purpose of Data | Calculation of baseline emissions |
| Comments | - |

| Data / Parameter | NCVCH4 |
| --- | --- |
| Data unit | TJ/t CH4 |
| Description | Net calorific value of methane at reference conditongs |
| Source of data | Technical literature |
| Value applied | 0.0504 |
| Justification of choice of data or description of measurement methods and procedures applied | - |
| Purpose of Data | - |
| Comments | - |

| Data / Parameter | TDLj,y and TDLk,y |
| --- | --- |
| Data unit | - |
| Description | Average technical transmission and distribution losses for providing electricity to source j,k in year y |
| Source of data | Baseline,project and/or leakage emissions from electricity consumption and monitoring of electricity generation(version 03.0) |
| Value applied | Use as default values of 20% for project consumption sources; Use as default values of 3% for baseline electricity consumption sources. i.e.: TDLj,y =3%,TDLk,y=20% |
| Justification of choice of data or description of measurement methods and procedures applied | - |
| Purpose of Data | - |
| Comments | - |

Data and Parameters Monitored

| Data / Parameter | FCH4,BL,R,y |
| --- | --- |
| Data unit | t CH4/yr |
| Description | Amount of methane in the LFG which is flared due to a requirement in year y |
| Source of data | Information of the host country’s regulatory requirements relating to LFG,contractual requirements, or requirements to address safety and odour concerns. |
| Description of measurement methods and procedures to be applied | - |
| Frequency of monitoring/recording | Annually |
| Value applied | - |
| Monitoring equipment | - |
| QA/QC procedures to be applied | - |
| Purpose of data | Calculation of baseline emissions |
| Calculation method | - |
| Comments | Applicable to Case 2 of section 5.4.1.3 of ACM0001“Flaring or use of landfill gas”(version 19.0) |

| Data / Parameter | Ρreg,y |
| --- | --- |
| Data unit | Dimensionless |
| Description | Fraction of LFG that is required to be flared due to a requirement in year y |
| Source of data | Information of the host country’s regulatory requirements relating to LFG,contractual requirements, or requirements to address safety and odour concerns. |
| Description of measurement methods and procedures to be applied | - |
| Frequency of monitoring/recording | Annually |
| Value applied | - |
| Monitoring equipment | - |
| QA/QC procedures to be applied | - |
| Purpose of data | Calculation of baseline emissions |
| Calculation method | - |
| Comments | Applicable to Case 2 of section 5.4.1.3 of ACM0001“Flaring or use of landfill gas”(version 19.0) |

| Data / Parameter | EGPJ,y |
| --- | --- |
| Data unit | MWh |
| Description | Amount of electricity generated using LFG by the project activity in year y |
| Source of data | Project participant |
| Description of measurement methods and procedures to be applied | Measured continuously by electricity meter(bi-directional) installed at the project site. All data will be monitored and archived electronically.Double check by receipt of electricity sales. |
| Frequency of monitoring/recording | The recording frequency will be hourly measured and record,and monthly aggregated. |
| Value applied | The data below are calculated ex ante Year; Electricity exported to the grid (MWh). 2018.11.20-2018.12.31; 2,475. 2019; 22,908. 2020; 24,216. 2021; 25,468. 2022; 26,683. 2023; 19,960. 2024; 15,265. 2025; 0. 2026; 0. 2027; 0. 2028.11.-2028.11.19; 0. |
| Monitoring equipment | Electricity meter |
| QA/QC procedures to be applied | The calibration should be done once a year by a qualified third party. |
| Purpose of data | Calculation of baseline emissions |
| Calculation method | - |
| Comments | - |

| Data / Parameter | EGEC,y |
| --- | --- |
| Data unit | MWh |
| Description | Amount of electricity consumed by the project activity in year y |
| Source of data | Project participant |
| Description of measurement methods and procedures to be applied | Measured continuously by electricity meter(bi-directional) installed at the project site. All data will be monitored and archived electronically.Double check by receipt of electricity sales. |
| Frequency of monitoring/recording | The recording frequency will be hourly measured and recorded,and monthly aggregated. |
| Value applied | For simpilification,the data used in ex ante calculation is 0. |
| Monitoring equipment | Electricity meter |
| QA/QC procedures to be applied | The calibration should be done once a year by a qualified third party. |
| Purpose of data | Calculation of baseline emissions |
| Calculation method | - |
| Comments | - |

| Data / Parameter | Vt,db |
| --- | --- |
| Data unit | m3 dry gas/h |
| Description | Volumetric flow of the gaseous stream in time interval t on a dry basis |
| Source of data | Monitored by project participant |
| Description of measurement methods and procedures to be applied | Volume flow measurement always refer to the actual pressure and temperature. |
| Frequency of monitoring/recording | Measured continuously by a flow meter (average hourly value should be used in the calculations of baseline emission reductions). Data to be aggregated monthly. |
| Value applied | The data below are calculated ex ante. Year; Total combusted LFG(m3). 2018.11.20-2018.12.31; 1,645,854. 2019; 15,231,169. 2020; 16,101,147. 2021; 16,933,204. 2022; 17,741,467. 2023; 13,271,153. 2024; 10,149,347. 2025; 0. 2026; 0. 2027; 0. 2028.11.-2028.11.19; 0. |
| Monitoring equipment | Flow meter |
| QA/QC procedures to be applied | Flow meter should be subject to a regular maintenance and testing regime to ensure accuracy |
| Purpose of data | Calculation of project emissions |
| Calculation method | - |
| Comments | - |

| Data / Parameter | Vi,t,db |
| --- | --- |
| Data unit | m3 gas i/m3 dry gas |
| Description | Volumetric fraction of greenhouse gas I in a time interval t on s dry basis |
| Source of data | Monitored by the project participant |
| Description of measurement methods and procedures to be applied | Continuous gas analyzer operating in operating in dry-basis.Volumetric flow measurement should always refer to the actual pressure and temperature. |
| Frequency of monitoring/recording | Continuous if not specified in the underlying methodology |
| Value applied | 50%(ex ante calculation) |
| Monitoring equipment | Gas analyzer |
| QA/QC procedures to be applied | Gas analyzer should be subject to a regular maintenance and testing regime to ensure accuracy |
| Purpose of data | Calculation of baseline emissions |
| Calculation method | - |
| Comments | - |

Monitoring Plan

1. The requirement of monitoring plan

The project participants will monitor the emission reductions by methods, indicators, and frequency required by Monitoring Methodology ACM0001(Version 19.0） to ensure project ERs are measurable and real. The monitoring methodology is based on direct measurement of the amount of landfill gas captured and destroyed by the project and electricity generating units.

2．Responsibilities of operational and management structure

The project participant will implement this monitoring plan. The plan could be revised according to suggestions from DOE and the practical circumstances, in order to keep it consistent, transparent and conservative during the monitoring process.

General Manager

operators for equipment maintenance

Operators for data record

Project Manager

Figure 5.1 Operational and management structure

(1)Principal of the monitoring procedure

The general manager of the project is the leader of the monitoring tasks: he sets out the responsibility of everyone in the monitoring system, and establishes the related documents. The general manager ensures that staff in the monitoring system has the ability to deal with the assigned tasks.

(2)Executive person of the monitoring procedure

Project Manager: a Project Manager is appointed, specifically responsible for training, checking the daily operation, reporting forms and archiving emergency situation reports. The CDM Manager reports monthly to the General Manager (GM) about the project performance and monitored data. In the event that non-conformance in the performance to the mentioned procedures and/or functioning problems of the monitoring equipment are identified, the CDM manager will inform the GM about the situation and work out relevant measures to be taken. The CDM manager will also be responsible for aggregating the monitored data monthly and yearly, archiving and keeping data during the crediting period and two years after.

(3)Operators of the monitoring procedure

Operators will take turns to work in the control center 24 hours a day. They will be in charge of data supervision, filling operation report forms and, checking and inspecting the system. If necessary, they will have the responsibility for executing the emergency plan and drafting emergency situation reports.

3．Monitoring system

Figure 5.2 Monitoring system

Main parameters:

| G | Gas analyzer to continuously measure methane fraction in LFG |
| --- | --- |
| FT | Flow meter to continuously measure flow rate of total captured LFG, with automatic measure adjustment for temperature and pressure |
| F1 F2 F3 F4 F5 F6 | Flow meters to continuously measure flow rates of combusted LFG for power generation of each generator set, with automatic measure adjustment for temperature and pressure. To be conservative, the min value of FT and F1+F2+F3+F4+F5+F6 will be used to calculate the ex-post emission reductions. |
| E | Electricity meter (main meter, bi-directional) to continuously measure the electricity supplied to the ECPG and the electricity supplied by the grid |

QA/QC

All the monitoring devices listed above (flow meter, gas analyzer and electricity meter) will be calibrated once a year by a qualified third party. The calibration report will be kept in the archives by the project owner, and checked by Project Manager. When the data is not available from the main monitoring devices, the data measured by the back-up devices will be used.

- Appendix

The project started to operate on 20/11/2018,and the project owner choose 20/11/2018 as the starting date of the crediting period.

The operation period of the project is from 1/6/2018 to 31/12/2024,so the ERs during the 1/1/2015 and 19/11/2028 are 0.

2017 China baseline emission factor of regional power grid.

http://cdm.unfccc.int/UserManagement/FileStorage/AM_CLAR_QEJWJEF3CFBP1OZAK6V5YXPQKK7WYJ

The EIA of the project.

The EIA of the project.